| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on AAI_HO-CMD message (16.2.3.12) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Whai-En Chen, Shih-Yuan Cheng, Yu-Chan Lin National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Kanchei (Ken) Loa, Chun-Yen Hsu, Youn-Tai Lee, Tsung-Yu Tsai, Chiu-Wen Chen Institute for Information Industry Yang-Han Lee, Yih Guang Jan Tamkang University | wechen@niu.edu.tw stsheu@ce.ncu.edu.tw loa@iii.org.tw |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D8) |  |
| Abstract | Comments on AAI_HO-CMD message (16.2.3.12) |  |
| Purpose | To be discussed in the IEEE 802.16m working group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on AAI-HO-CMD message (16.2.3.12)

- Introduction

In section 16.2.3.12, we suggest to clarify the Table 691-AAI-HO-CMD message format based on IEEE 802.16m DRAFT Amendment [1] as shown in proposed text.

- Proposed Text

[Modify the following text in Section 16.2.3.12]

Table 691-AAI-HO-CMD message format

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI-HO-CMD message shall not include any T-ABS.; 0b11: reserved. | N/A. |
| If (Mode = 0b00) { |  |  |  |
| HO Reentry Mode | 1 | 0b0: the AMS disconnects from the S-ABS before performing network reentry with the T-ABS. 0b1: the AMS maintains communication with S-ABS while performing network reentry with the T-ABS. | Shall not be set to 1 if AMS does not support EBB (according to AAI-REG-REQ). |
| If (HO Reentry Mode = 0b1) { |  |  |  |
| HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the T-ABS within the HO Reentry Interval and continues data transmission with the S-ABS during HO Reentry Interleaving Interval. If HO Reentry Interleaving Interval = 0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2. |  |
| If (HO Reentry Interleaving Interval > 0) { |  |  |  |
| HO Reentry Interval | 8 | AMS performs network re-entry at the T-ABS during HO Reentry Interval. |  |
| HO Reentry Iteration | 3 | The requested number of iterating HO Reentry Intervals by an AMS. |  |
| } |  |  |  |
| AMS Physical carrier index at the S-ABS | 6 | Recommended carrier index of the AMS which was used in the S-ABS, and to be used for network reentry to the T-ABS. All other AMS's carriers which was used in the S-ABS can be used for data communication with the S-ABS. | May be included when AMS's multicarrier capability = 0b10, and the target carrier is different from the serving carrier. |
| }//end of If (HO Reentry Mode = 1) |  |  |  |
| Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Shall be included when Mode = 0b00. |
| Resource_Retain_Time | 8 | The time when the serving ABS discards AMS's context. | Optional. If not included, the value included in the AAI-RNG-RSP or AAI-REG-RSP shall be used. |
| for (i = 0; i < N_Target_BS; i++) { |  | N_Target_BS is the number of T-ABSs or target legacy BSs included in this message. |  |
| targetBSID | 48 | BSID of the Target T-ABS or target legacy BS. |  |
| SA-Preamble Index | 10 | Indicates the SA-Preamble index of the carrier. |  |
| FA index | 8 | Indicates the FA index. | Shall be included for the target legacy BS. |
| Action Time | 8 | The absolute frame number at the S-ABS. When CDMA_RNG_FLAG is set to 1, it shall be set to the frame where either a normal or dynamic ranging channel is present. |  |
| CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG = 0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each T-ABS when Mode = 0b00.; It shall be included when Mode = 0b01. If CDMA_RNG_FLAG is set to 0, pre-assigned STID at the target BS shall be assigned. |  |
| If (CDMA_RNG_FLAG = 0) { |  |  |  |
| offsetData | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step. |  |
| offsetControl | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step. |  |
| }else { |  |  |  |
| Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | Optional. Shall be set to 1 if Seamless HO = 1. |
| Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity of the dynamic ranging channel used in the RA-ID. Ranging opportunity index shall be assigned by the T-ABS. The T-ABS shall assign unique ranging opportunity index which is not overlapped with other ranging channel in the allocated frame. 0b0: 0b01 0b1: 0b10 | Optional. |
| Ranging opportunity Subframe index | 2 | Indicates the subframe index of the allocated ranging opportunity. The subband of a dynamic ranging channel is same as the ranging channel allocated by SFH. | Optional. |
| } |  |  |  |
| HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI-SBC-REQ/RSP MAC. control messages during reentry processing Bit #1: Omit PKM Authentication phase. Bit #2: Omit AAI-REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines. | When Bit #2 is set to 0 and IP address refresh is needed, the AAI-REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| Seamless HO | 1 | Indicates whether seamless HO is supported. |  |
| Ranging initiation deadline | 8 | An AMS shall send the AAI-RNG-REQ message during HO until Ranging initiation deadline. |  |
| Pre-assigned STID | 12 | STID assigned to the AMS by the T-ABS. |  |
| Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0b00 =: No service possible for this AMS; 0b01 =: Some service is available for one or several service flows authorized for the AMS; 0b102 =: For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.; 0b113 =: No service level prediction available. |  |
| T-ABS Physical_Carrier_Index | 6 | Physical carrier index of the recommended T-ABS. | Shall be included when T-ABS is not included in AAI-NBR-ADV message or is multi-carrier ABS. |
| Channel bandwidth | 8 | Channel bandwidth of recommended target ABS in units of 125 KHz. | Shall be included when T-ABS is not included in AAI-NBR-ADV message. |
| CP length | 2 | CP length of the recommended target T-ABS. 0b00: 1/8; 0b01: 1/16; 0b10: 1/4; 0b11: reserved. | Shall be included when T-ABS is not included in AAI-NBR-ADV message. |
| For (j = 0; j < N_Preassigned_Carriers; j++) { |  | N_Preassigned_Carrier is the number of preassigned carriers by the T-ABS. |  |
| Carrier Status Indication | 1 | Indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done. |  |
| Physical carrier index of the secondary carrier index | 6 | Physical carrier index of the pre-assigned secondary carrier. |  |
| } |  |  |  |
| Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target legacy BS. | Optionally presented only when HO to a legacy BS. |
| SFH delta information | Variable | Delta encoding with reference to the S-ABS. | May be included when SFH delta information is not included in the AAI-NBR-ADV message for the T-ABS. |
| S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information.. | Shall be included when SFH delta information is included. |
| S-SFH application time | 1 | Indicate if the SFH delta information at the target ABS is applied at the time of AMS network reentry. | Shall be included when SFH delta information is included. |
| }//end of for (i = 0; i < N_Target_BS; i++) |  |  |  |
| }else if (Mode = 0b01) { |  |  |  |
| HO Reentry Mode | 1 | 0b0: the AMS disconnects from the MZone before performing network reentry with the LZone; 0b1: the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier. |  |
| Resource_Retain_Time | 8 | The time when the serving ABS discards AMS's context. | Optional. If not included, the value included in the AAI-RNG-RSP or AAI-REG-RSP shall be used. |
| FA index | 8 | Indicates the FA index. | Shall be included for target LZONE located in a different carrier. |
| Action Time | 8 | This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number. |  |
| LZone Preamble Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone. |  |
| Pre-allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target LZONE. | Optional. |
| }else if (Mode = 0b10) { |  |  |  |
| } |  |  |  |

- References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”.