| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Femto ABS reselection by AMS (16.4.7.5) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Yung-Lan Tseng, Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Sponsor ballot IEEE P802.16m/D8 : 16.4.7.5 |  |
| Abstract | This contribution proposes the text modification for Femto ABS reselection |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Femto ABS reselection by AMS (16.4.7.5)

Yung-Lan Tseng, Yi-Ting Lin, Chun-Yen Hsu, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

# Introduction

There are three types of Femto ABS in the IEEE 802.16m D8, which are OSG/CSG-Open/CSG-Closed Femto ABS. Only the CSG-Closed Femto ABS can reject the non-member AMSs. The CSG-Open Femto ABS shall not reject the AAI-RNG-REQ from the non-member AMSs. The corresponding text is modified to clarify the reselection procedure.

# Reference

[1] IEEE P802.16m/D8, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Aug. 2010.

- Text proposal for inclusion in the 802.16m Draft 8

----------------------------------------------------------------------------------------------------------------------------------------------------------------

[Modify line 11, page 844 in Section 16.4.7.5]

Suggested Remedy

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.4.7.5 Femto ABS reselection by AMS

If the Femto ABS is an OSG Femto ABS, then the Femto ABS ignores the CSGID(s) (if sent by the AMS) in the AAI-RNG-REQ and goes ahead with the next steps. If the Femto ABS is a CSG Femto ABS, the Femto ABS receives the AAI-RNG-REQ and if needed it looks at the received CSGID(s) and checks if it matches with at least one of its CSGID(s). If there is match of the CSGID, then the Femto ABS knows that the AMS is a member of the Femto ABS and goes ahead with the next steps.

If the received CSGID(s) from the AMS does not match any of the CSGID(s) of the Femto ABS itself, and the Femto ABS is a CSG-Closed Femto ABS, the Femto ABS shall send a AAI-RNG-RSP and set the Ranging Abort bit=1 and the Ranging Abort Timer=0 to indicate in the AAI-RNG-RSP it indicates the rejection of access for this AMS.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------