| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Relay Message Table format |  |
| Date Submitted | 2010-9-8 |  |
| Source(s) | Guanghui Fan, Ke Zeng Huawei Technologies | fanguanghui@huawei.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clarification on the table format for Relay messages |  |
| Purpose | To be discussed and adopted in D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Relay Message Tables

Guanghui Fan, Ke Zeng

Huawei Technologies

Introduction

This contribution proposes the substituted control messages table format for Relay part of 16m D8. The related control messages include AAI-ARS-CONFIG-CMD.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 247, line 3, replace Table 754 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 754—AAI_ARS-CONFIG-CMD message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| AAI_DL_Relay_zone_duration | 3 | The duration of the AAI DL Relay zone in units of AAI subframes. It has 3 bits to represent the values {1, 2, 3, 4, 5. 6, 7, 8} |  |
| AAI _UL_Relay_zone_duration | 3 | The duration of the AAI UL Relay zone in units of AAI subframes. It has 3 bits to represent the values {1, 2, 3, 4, 5. 6, 7, 8} |  |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | The indicator which signalizes if the ABS may allocate resources to the AMS in the AAI DL Relay zone and AAI UL Relay zone. It has 1 bit to represent the values {0, 1}. If it is set to 1 then the AMSs are allocated in Relay zones |  |
| MIMO Midamble indication in AAI DL Relay zone | 1 | 0b0: MIMO midamble is not transmitted in AAI DL Relay zone 0b1: MIMO midamble is transmitted in AAI DL Relay zone |  |
| If(AAI _Relay_zone_AMS_allocation_indicator == 0){ |  |  |  |
| R_DCASSB0 | 5/4/3 | AAI _DL_Relay_zone_Cell_Specific_Parameter See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_DCASMB0 | 5/4/3 | AAI _DL_Relay_zone_Cell_Specific_Parameter See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_DCASi | 3/2/1 | AAI _DL_Relay_zone_Cell_Specific_Parameter See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  |
| R_UCASSB0 | 5/4/3 | AAI _UL_Relay_zone_Cell_Specific_Parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_UCASMB0 | 5/4/3 | AAI _UL_Relay_zone_Cell_Specific_Parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_UCASi | 3/2/1 | AAI _UL_Relay_zone_Cell_Specific_Parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  |
| } |  |  |  |