| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the FFR Message Table format |  |
| Date Submitted | 2010-9-16 |  |
| Source(s) | Guanghui Fan, Qiwen Song Huawei Technologies | fanguanghui@huawei.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clarification on the table format for FFR messages |  |
| Purpose | To be discussed and adopted in D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the FFR Message Tables

Guanghui Fan, Qiwen Song

Huawei Technologies

Introduction

This contribution proposes the substituted control messages table format for FFR part of 16m D8. The related control messages include AAI-FFR-CMD, AAI-FFR-REP

Proposed Text

Remedy 1:

In the P802.16m/D8, page 149, line4, replace Table 699 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 699—AAI-FFR-CMD message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. |  |
| reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance. |  |
| frameOffset | 8 | The offset (in units of frames) from the current frame in which AAI-FFR-REP message shall be transmitted on the unsolicited UL resource. |  |
| PhysicalCarrierIndex | 6 | The relevant active carrier that this control message is associated with | Present if the Carrier-specific PHY control mode is disabled. |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 149, line 50, replace Table 700 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 700—AAI-FFR-REP message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. |  |
| reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is being sent, otherwise it indicates the specific report type is not being sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| If(0th LSB of frequencyPartitionBitMap == 0b1){ |  | fp0report |  |
| FFR-FeedbackIE | Variable | See the definition below |  |
| } |  |  |  |
| If(1st LSB of frequencyPartitionBitMap == 0b1){ |  | fp1report |  |
| FFR-FeedbackIE | Variable | See the definition below |  |
| } |  |  |  |
| If(2nd LSB of frequencyPartitionBitMap == 0b1){ |  | fp2report |  |
| FFR-FeedbackIE | Variable | See the definition below |  |
| } |  |  |  |
| If(3rd LSB of frequencyPartitionBitMap == 0b1){ |  | fp3report |  |
| FFR-FeedbackIE | Variable | See the definition below |  |
| } |  |  |  |
| FFR-FeedbackIE{ | Variable | FFR-FeedbackIE definition |  |
| If(0th LSB of reportType == 0b1{ |  |  |  |
| interferenceMean | 8 | Interference mean. This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. |  |
| } |  |  |  |
| If(1st LSB of reportType == 0b1{ |  |  |  |
| interferenceVariance | 4 | Interference variance 0dB to 15dB in units of 1dB |  |
| } |  |  |  |
| If(2nd LSB of reportType == 0b1{ |  |  |  |
| sinrMean | 4 | SINR mean -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved |  |
| } |  |  |  |
| If(3rd LSB of reportType == 0b1{ |  |  |  |
| sinrVariance | 8 | SINR variance 0dB to 15dB in units of 1dB |  |
| } |  |  |  |
| } |  |  |  |
| PhysicalCarrierIndex | 6 | The relevant active carrier that this control message is associated with | Present if the Carrier-specific PHY control mode is disabled. |

--------------------------------------------------  Text End  -----------------------------------------------------------------