| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of MAC Control Messages for Multicarrier (16.2.8.2.8) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Jinyoung Chun, Youngsoo Yuk, Kiseon Ryu, Wookbong Lee, Jin Sam Kwak LG Electronics Xiangying Yang Intel Corporation Yuqin Chen, Yang Liu ZTE | jiny.chun@lge.com Xiangying.yang@intel.com Chen.yuqin@zte.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot Recirc #2 |  |
| Abstract | This contribution provides the clarification of MAC control messages for supporting Multicarrier mode. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of MAC Control Messages for Multicarrier (16.2.8.2.8)

Jinyoung Chun, Youngsoo Yuk, Kiseon Ryu, Wookbong Lee, Jin Sam Kwak

LG Electronics

Xiangying Yang

Intel Corporation

Yuqin Chen, Yang Liu

ZTE

- Introduction

In IEEE 802.16m/D8, MAC control messages and MAC headers shall be sent to the AMS through its primary carrier. But carrier index shall be known implicitly or explicitly if the value included the message depends on the carrier. For example, there are FFR-related messages (AAI_FFR-CMD, AAI_FFR-REP) and Power status-related messages (AAI_UL_PSR_CFG, UL_PSR header) and MIMO feedback messages (CMFH, MFH).

If ABS commands the report and AMS transmit the control messages, then ABS can know which carrier the message is for implicitly. But event-driven or autonomous control messages by AMS such as UL_PSR header shall include the carrier index because both of bandwidth requests and the control messages are transmitted on the AMS’s primary carrier.

Therefore we propose to insert carrier index in some MAC control messages.

- Text proposal in the P802.16m/D6

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

[Remedy 1]

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify line 61 of page 332 as following:]

16.2.8 Multicarrier operation

16.2.8.1 Multicarrier Types and Operational Modes

When supporting multicarrier operations, two types of carriers are defined from an AMS point of view:

•Primary Carrier: A primary carrier is a standalone carrier where AMS complete initial network entry or network reentry procedure. When supporting multicarrier operations, an AMS shall only have one primary carrier and may be assigned with multiple secondary carriers. All the unicast MAC control messages relative to multicarrier operations (except MIMO and Multi-BS MIMO related control messages mentioned in 16.2.8.2.8) shall be sent to the AMS through its primary carrier.

•Secondary Carrier: Secondary carriers are additional carriers which may be assigned to the AMS by the ABS.

…

------------------------------------------------   Text end  -----------------------------------------------------------

[Remedy 2]

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 54 ~ 55 of page 333:]

A secondary carrier for an AMS, if fully configured, may serve as primary carrier for other AMSs. The multiple AMSs with different primary carriers may also share the same physical carrier as their secondary carrier. The following multicarrier operation modes are identified, which may all or independently be supported:

•Multicarrier Aggregation: The multicarrier mode in which the AMS maintains its physical layer connection and monitors the control signaling on the primary carrier while proceeding data transmission on the secondary carrier. The resource allocation to an AMS may span across a primary and multiple secondary carriers. Link adaptation feedback mechanisms are independently handled by each carrier. Link adaptation feedback mechanisms should incorporate measurements relevant to both primary and secondary carriers. In this mode the system may assign secondary carriers to an AMS in the downlink and/or uplink asymmetrically based on AMS capability, system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

------------------------------------------------   Text end  -----------------------------------------------------------

[Remedy 3]

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the section 16.2.8.2.8 on page 338 as following:]

16.2.8.2.8 DL CINR report operation PHY level control and UL Feedback Channel allocation

An ABS may assign UL feedback channels to each active carrier of an AMS as defined in 16.3.5.5.2.4.

In multicarrier aggregation with fully configured carriers, an ABS may assign FastFeedback fast feedback channels to each active carrier of an AMS. When FastFeedback fast feedback channel is assigned, the AMS reports CINR for an active carrier over the assigned FastFeedback fast feedback channel of the corresponding carrier at the feedback region as defined in 16.3.7.3.3.

When only DL of fully configured carrier has been activated, UL feedback channel assigned by the ABS is located at the UL region defined in the SFH on the primary carrier. In this case, the ABS may allocate for an AMS one UL fast feedback channel per a secondary carrier over a primary carrier. If the AMS receives Feedback Allocation A-MAP IE through the DL only activated carrier, the AMS transmits the UL feedback information for the secondary carrier over the assigned UL Fast Feedback channel of the primary carrier.

In multicarrier aggregation with DL-only secondary partially configured carrier, the ABS directs the AMS to report CINRs of active DL-only secondary carriers through FastFeedback fast feedback channel(s) on the primary carrier at the feedback region The feedback region is indicated using the physical carrier index for the primary carrier, the start DLRU index for feedback channel, the number of DLRUs for feedback channel and the number of HARQ feedback channel per HARQ feedback region in the AAI-SCD message that is transmitted on the active DL-only secondary partially configured carrier as defined in 16.3.8.3.3. The feedback region of the active DL-only secondary partially configured carrier follows the feedback region of the primary carrier. When measurement/report MAC control messages (i.e., AAI-SingleBS_MIMO_FBK, AAI-MultiBS_MIMO_FBK, MIMO Feedback header, Correlation Matrix Feed-back header) are used for DL CINR report operation, the messages are transmitted on the AMS's primary carrier. Feedback Polling A-MAP IE to inform a UL burst region is transmitted on each active DL carrier and the corresponding UL burst is assigned on the AMS's primary carrier. The UL burst carries the messages for DL CINR report for the corresponding active DL carrier.

When MIMO related measurement/report MAC control messages and headers, i.e., AAI-SingleBS_MIMO_FBK, AAI-MultiBS_MIMO_FBK, MIMO Feedback header, Correlation Matrix Feed-back header, are used for DL CINR report operation, they are transmitted in the uplink grant indicated by Feedback Polling A-MAP IE. For fully configured carriers, the grant is always on the same active carrier on which the Feedback Polling IE is received. For partially configured carrier, the grant is always on the primary carrier.

Other PHY level control not related to MIMO feedback should also be handled per carrier. But MAC signaling header or MAC control message not for MIMO feedback of each carrier can be transmitted on any available grant on any carrier. A 6-bit carrier index may be included to indicate the relevant carrier that this control message or signaling header is associated with. The following MAC signaling header and MAC control message are handled in this way:

- MAC signaling header: Uplink Power Status Report

- MAC control message: AAI-FFR-CMD, AAI-FFR-REP, AAI-UL_PSR_CFG, AAI-UL_POWER_ADJ, AAI-MULTI_BS_MIMO-REQ, AAI-MULTI_BS_MIMO-RSP

In multicarrier operation mode, the pathloss reference (i.e. preamble) for uplink power control shall be the same carrier in TDD mode and the paired downlink carrier in FDD mode, respectively.

16.2.8.2.8.1 Restricted non-MIMO feedback

In order to save overhead, the BS may choose to broadcast certain restrictions for the non-MIMO feedback. This mode of restricted non-MIMO feedback shall be signaled in the AAI_MC-ADV message.

In the restricted non-MIMO feedback mode, the MS shall not include the 6 bit carrier index in the non-MIMO feedback message. This shall be understood by the BS that the feedback is only pertinent for the carrier on which the message was transmitted.

Furthermore, in the restricted mode, the carrier index shall be set to the carrier index of the transmitting carrier to indicate feedback for the transmitting carrier in the MAC signaling header.

------------------------------------------------   Text end  -----------------------------------------------------------

[Remedy 4]

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify line 7 of page 338 as following:]

16.2.8.2.6 Bandwidth Request and Resource Allocation

16.2.8.2.6.1 Bandwidth Request

All bandwidth requests that are transmitted on the AMS’s primary carrier follow following the same procedures as defined in 16.2.11. When a bandwidth request is sent on AMS’s secondary carrier using contention-based procedure, this indicates that AMS requests uplink grant specifically for this secondary carrier. Bandwidth request using piggyback may be transmitted in MAC PDUs over the secondary carrier(s) as well as the primary carrier, and does not indicate bandwidth request on any particular carrier.

-----------------------------------------------   Text End  -----------------------------------------------------------

[Remedy 5]

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 60 of page 148 as below:]

16.2.3.19 AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message may be transmitted by the ABS to instruct the AMS to perform measurement over specific frequency partition(s). Table 698699 lists the parameters to be included into the AAI_FFR-CMD message:

Table 699- AAI_FFR-CMD message format

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_FFR-CMD | N.A. |
| M | frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. | N.A. |
| M | reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| M | frameOffset | 8 | The offset (in units of frames) from the current frame in which AAI_FFR-REP message shall be transmitted on the unsolicited UL resource. |  |
| O | PhysicalCarrierIndex | 6 | The physical carrier index configured by the message |  |

16.2.3.20 AAI_FFR-REP (FFR Report) Message

Once an AMS receives AAI_FFR-CMD message, an AAI_FFR-REP message is sent by the AMS to report the downlink interference and/or SINR statistics of frequency partition(s) specified in AAI_FFR-CMD message. Table 697700 shows the parameters for AAI_FFR-REP message:

Table 700- AAI_FFR-REP message format

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_FFR-REP | N.A. |
| M | frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. | N.A. |
| M | reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| O | FFR-REPORT | Variable |  |  |
| O | A) fp0Report | Variable | FFR-FeedbackIE | Present when 0-th LSB of frequencyPartitionBitmap is 0b1 |
| O | B) fp1Report | Variable | FFR-FeedbackIE | Present when 1-th LSB of frequencyPartitionBitmap is 0b1 |
| O | C) fp2Report | Variable | FFR-FeedbackIE | Present when 2-th LSB of frequencyPartitionBitmap is 0b1 |
| O | D) fp3Report | Variable | FFR-FeedbackIE | Present when 3-th LSB of frequencyPartitionBitmap is 0b1 |
| O | FFR-FeedbackIE | Variable |  |  |
| O | A) interferenceMean | 8 | Interference mean. This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. | Present when 0-th LSB of reportType is 0b1 |
| O | B) interferenceVariance | 4 | Interference variance 0dB to 15dB in units of 1dB | Present when 1-st LSB of reportType is 0b1 |
| O | C) sinrMean | 8 | SINR mean -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved | Present when 2-nd LSB of reportType is 0b1 |
| O | D) sinrVariable | 4 | SINR variance 0dB to 15dB in units of 1dB | Present when 3-rd LSB of reportType is 0b1 |
| O | PhysicalCarrierIndex | 6 | The physical carrier index configured by the message |  |

-----------------------------------------------   Text End  -----------------------------------------------------------

[Remedy 6]

------------------------------------------------   Text Start  -----------------------------------------------------------

[insert one row in Table 713 on Page-180]

16.2.3.33 AAI-UL_POWER_ADJ message

An AAI-UL_POWER_ADJ message shall be transmitted by the ABS to control the transmit power level of AMS.

Table 713—AAI-UL_POWER_ADJ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | offsetData | 6 | offsetData is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | N/A |
| M | offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | N/A |
| O | PhysicalCarrierIndex | 6 | The physical carrier index configured by the message |  |
| O | channelIndex | 8 | Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between channel index and the AMS | Present when ABS want to change FF Channel Period by using AAI-POWER-ADJ message. (More details can be found in 16.3.6.5.2.4.5.) |
|  | shortTermFeedbackPeriod | 3 | Short-term feedback is transmitted on the FBCH every 2p frames, where p is the value specified by shortTermFeedbackPeriod |  |
|  | longTermFeedbackPeriod | 2 | Long-term feedback is transmitted on the FBCH every 2q short-term feedback reports are transmitted, where q is the value specified by longTermFeedbackPeriod. If q = 0b00, long-term feedback is not used |  |
|  | frameOffsetNumber | 2 | The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame number. If the current frame is specified, the AMS starts reporting in eight frames |  |
|  | subframeIndex | 3 | Indicates the UL AAI subframe index in the UL portion of the frame |  |
|  | allocationDuration | 3 | An FBCH is transmitted on the FBCH channels indexed by Channel Index for 8×2d frames, where d is the value given by allocationDuration. If d = 0b000, the FBCH is deallocated. If d = 0b111, the AM reports until the ABS command for the AMS to stop |  |

[Modify the text in line 1 of page 181 as below:]

16.2.3.34 AAI-Uplink Power Status Reporting Configuration (AAI-UL_PSR_Config) message

An AAI-UL_PSR_Config message is used to configure the AMS uplink power status reporting by unicasting. It is defined as:

Table 714 – AAI-UL_RSP-CFG message Field Descriptions

| M/O | Attributes/Array of attributes |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | uplinkPowerStatusReport |  | 1 | disable(0), enable(1) | N.A. |
| O | psrReport-ConfigPara-meters | txPower-ReportTh-reshold | 4 | txPowerReportThreshold is a 4 bit unsigned integer value in 0.5 dB steps, the specific value "0b1111" shall be interpreted as "infinite" | Present when uplinkPow-erStatus-Report is enabled |
|  | psrReport-ConfigPara-meters | txPower-Report-Minimum-Interval | 4 | txPowerReportMinimumInterval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |  |
|  | psrReport-ConfigPara-meters | txPower-ReportPe-riodicalIn-terval | 4 | txPowerReportPeriodicalInterval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |  |
|  | psrReport-ConfigPara-meters | Physical-Carrier-Index | 6 | The physical carrier index configured by the message |  |

[Add the text in line 36 of page 64 as below:]

16.2.2.1.3.6 Uplink Power Status Report Header

The Uplink Power Status Report Header should be used to convey AMS ULPC status from AMS to ABS. The control of the reporting is described in section 16.3.9.4.7.

Table 662—Uplink Power Status Report Header Format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| Uplink Power Status Report Header(){ |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 5 | Mac Signaling header type = 0b00101 |
| Length | 3 | Indicates the length of the signaling header: 0b101: 5 bytes 0b110: 6 bytes |
| ULPC Parameters Updating Indicator | 1 | Indicates whether the AMS has updated its ULPC parameters from AAI_SCD: 0: No ULPC parameters changed comparing to previous reporting. 1: The confirmation of new ULPC parameters from AAI_SCD applied. |
| Change configuration change | 4 | Only valid if ULPC Parameters Updating Indicator == 0b1 The value is the same as 2 LSB of “Change configuration change” in the latest AAI_SCD message. |
| txPowerPsdBase | 8 | txPowerPsdBase (PSD(base)) is coded using 8 bits in 0.5dBm steps ranging from -74dBm(coded 0x00) to 53.5dBm(coded 0xFF). |
| txSirDownlink | 10 | txSirDownlink(SIRDL) is coded using 10 bits in 1/16dB steps ranging from -12dB(coded 0x00) to 51.9375dB(coded 0x3ff) |
| Physical Carrier Index | 6 | The physical carrier index configured by the message. |
| Reserved | 57 | Shall be filled by 0 for byte alignment |
| } |  |  |

-----------------------------------------------   Text End  -----------------------------------------------------------

[Remedy 7]

------------------------------------------------   Text Start  -----------------------------------------------------------

[insert one row in Table 752 on Page-241]

16.2.3.55 AAI-MC-ADV (multicarrier Advertisement) Message

The ABS which supports multiple RF carriers shall periodically broadcast AAI-MC-ADV message for the reception by all AMSs.

Table 752—AAI-MC-ADV message format

| Field | Size (bits) | Descrpition | Condition |
| --- | --- | --- | --- |
| Multi-carrier configuration change count | 4 | The value is incremented by 1 upon each update |  |
| Serving BS Carrier Number | 3 | Indicates the number of carriers supported by the S-ABS |  |
| MAC Protocol version | 8 | Indicates MAC protocol version that the current carrier supports: it is consistent with REV.2 definition with new MAC protocol version 9 defined for AAI |  |
| Multi-carrier restricted non-MIMO feedback mode | 1 | Indicates whether non-MIMO PHY feedback defined in 16.2.8.2.8 shall be restricted to be transmitted on the pertinent carrier only |  |
| … | … |  |  |

-----------------------------------------------   Text End  -----------------------------------------------------------

- References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”