| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for CA-specific trigger conditions (16.2.8.2.11.1) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Kiseon Ryu LG Electronics Ji-Yun Seol Samsung Electronics | eunjong.lee@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com, jiyun.seol@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D8 for Sponsor Ballot |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for CA-specific trigger conditions (16.2.8.2.11.1)

Eunjong Lee, Youngsoo Yuk and Kiseon Ryu

LG Electronics

Ji-Yun Seol

Samsung Electronics

- Introduction

In the last meeting, we defined the CA-specific trigger definitions for assigned secondary carriers of smaller coverage. However, it may lead to some unnecessary scanning/reporting. So, we’d like to clean up some points related to the CA trigger issues.

Details are as follows:

- The assigned secondary carrier in Table 773 seems to be unclear because the assigned secondary carrier can be one of active or inactive secondary carriers and active secondary carrier doesn’t need to perform the scanning/reporting procedure by this trigger condition. Therefore, in order to clarify the scanning procedure by the trigger condition, we propose to add ‘inactive’ before ‘assigned secondary carrier’ of Table 773.

- If there are one or more carriers having the same smaller coverage, these carriers would have the same CA-specific trigger definitions. In order to reduce the message overheads, therefore, we propose to define the CA-specific trigger definitions only for the lowest physical carrier index among the available carriers requiring the same trigger definitions and for the remaining carriers the ABS informs the AMS of this lowest physical carrier index (i.e., Reference Carrier in AAI_MC-ADV of proposal text) instead of the CA-specific trigger definitions.

- It seems to be inefficient that all of the inactive assigned carriers requiring the same CA-specific trigger definitions perform the scanning/reporting procedure. So, we propose that an AMS perform the scanning/reporting procedure only for the carrier corresponding to the lowest physical carrier index among them if all of the assigned carriers linked to Reference Carrier of AAI_MC-ADV are inactive. In addition, if one of the inactive assigned carriers linked to Reference Carrier is activated, the AMS doesn’t need to perform the scanning/reporting for the remaining inactive carriers and the ABS can refer to the channel quality of the activated carrier.

- References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- Text proposal for the 802.16m DRAFT amendment

--------------------------------------------------- Start of the Text ------------------------------------------------------

[Modify the bit size of Type/Function/Action in Table 772 on page 347 as follows:]

Table 772—CA-specific Trigger Description

| Name | Legnth (bits) | Value |
| --- | --- | --- |
| Number of conditions | 2 | The number of conditions that are included in this trigger (see loop definition below). When more than one condition is included, this trigger is referred to as a complex trigger and is the logical AND combination of all the included conditions. |
| for(i=0; i<= Number of conditions; i++){ |  |  |
| Type/Function/Action | 53 | See Table 773—for description |
| Trigger Value | 8 | Trigger value is the value used in comparing measured metric for determining a trigger condition. |
| Trigger averaging parameter | 4 | The averaging parameter used for averaging this trigger metric according to equation (4). If not present, the default trigger averaging parameter in AAI-SCD is used. 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xF: reserved |
| } |  |  |

[Modify the texts in Table 773 on page 348 as follows:]

Table 773—CA-specific Trigger; Type/Function/Action Description

| Name | Size (bits) | Value | Description |
| --- | --- | --- | --- |
| Type | 1(MSB) | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric |  |
| Function | 21 | Computation defining scanning and reporting trigger condition: 0x0: Metric of primary carrier is greater than absolute value 0x1: Metric of primary carrier is less than absolute value 0x21: Metric of inactive assigned secondary carrier is greater than absolute value 0x3: Metric of assigned secondary carrier is less than absolute value |  |
| Action | 1(LSB) | Action performed upon reaching trigger condition: 0b0: Initiate the scanning 0b1: Respond on trigger with AAI_SCN-REP | Action 0b0 applies to only Function 0x0 and 0x1, and action 0b1 applies to only Function 0x10x2 and 0x3. |

[Modify the texts in Table 752 on page 241 as follows:]

Table 752—AAI_MC-ADV message format

| Field | Size (bits) | Description | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| Reference Carrier | 6 | Indicates the physical carrier index of the reference carrier for CA-specific Trigger definitions. The corresponding carrier uses the same CA-specific Trigger definitions as this reference carrier. If this value is present, CA-specific Trigger definitions for the corresponding carrier may be omitted. | Optional Present when the corresponding carrier is required to be scanned for carrier activation and the same CA-specific Trigger definitions have been already defined for previous carrier index in AAI_MC-ADV. |
| CA-specific Trigger definitions | Variable | Carrier Activation-specific triggers with encoding defined in Table 772 – CA-specific Trigger description. | Optional Present when the corresponding carrier is required to be scanned for carrier activation and has the lowest physical carrier index among the available carriers requiring the same CA-specific Trigger definitions. |
| PCC-specific Trigger definitions | Variable | Primary Carrier Change-specific triggers with encoding defined in Table 770 –Trigger description. | Optional Present when the corresponding carrier is required to apply different trigger condition from definitions of AAI_SCD message. |
| … |  |  |  |

[Add the texts in line 18 on page 347 as follows:]

The ABS may specify trigger parameters for both scanning and reporting of each inactive assigned secondary carrier in AAI-MC-ADV message. An AMS shall perform the scanning or reporting when the conditions on the triggers are met. If one or more of the assigned secondary carriers have the same CA-specific trigger definitions and all of the carriers are inactive, the AMS may perform the scanning/reporting procedure by the trigger conditions only for the carrier corresponding to the lowest physical carrier index among them until at least one of the assigned carriers gets activated. If one or more of inactive assigned carriers linked to Reference Carrier of AAI_MC-ADV is activated, the AMS may not perform the scanning/reporting procedure for the remaining inactive assigned carrier(s) and the ABS will refer to the channel quality of the activated carrier(s).

------------------------------------------------------------------End of the Text----------------------------------------------------------------