| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification Text on Broadcast A-A-MAP IE (Section 16.3.5.5) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Hyunkyu Yu, Chiwoo Lim Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies the text about broadcast assignment A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification Text on Broadcast A-A-MAP IE (Section 16.3.5.5)

Hyunkyu Yu, Chiwoo Lim

- Introduction

- This contribution clarifies the text about dynamic NS-RCH allocation in broadcast assignment A-MAP IE and CRC mask for broadcast assignment A-MAP IE.

- Proposed Text Changes

[Remedy-1: Change the following text, at line 39 in page 650, section 16.3.5.5.2.4.13, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast or multicast burst or NS-RCH. A broadcast burst contains one or more broadcast MAC control messages. Table 862 describes the fields in a broadcast assignment A-MAP IE.

Table 862 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| … | … | … |
| } else if (Function Index == 0b01){ |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| For (i=0; i<N; i++){ |  |  |
| Subframe index | 3 |  |
| Ranging opportunity index | 1 | Indicates 2-bit Opportunity index of the ranging channel specified in 16.2.15.3 0b0: 0b01 0b1: 0b10 |
| } |  |  |
| Reserved | 10 29/25 | Reserved bits. If Number of Ranging Opportunities is set to 0b0, Size in bits is 29. Otherwise, Size in bits is 25. |
| } else if {Function Index == 0b10} { |  |  |
| … | … | … |

*A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 846. If Function index == 0b00 or 0b01, tThe CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b001. If Function index == 0b10, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b010.

Multicast Group ID is the ID of a group that receives multicast burst and is unique to an ABS. Multicast Group ID is assigned by AAI-DSA-REQ/RSP message and changed by AAI-DSC_-REQ message.

Functional Index == 0b00

The broadcast burst signaled by the Broadcast Assignment A-MAP IE is always transmitted using SFBC as the MIMO encoder format and QPSK as the modulation scheme.

…

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Change the following text, at line 48 in page 599, section 16.3.5.5.2.4, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast or multicast burst or NS-RCH. A broadcast burst contains one or more broadcast MAC control messages. Table 862 describes the fields in a broadcast assignment A-MAP IE.

Table 846—Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type indicator | Masking code |
|  | 0b000 | 12 bit STID |
|  | 0b001 | Refer to Table 847 |
|  | 0b010 | 12 bit Masking Code (0xFFF in Hex) |
| 0b1 | … |  |

Table 847—Description of the Masking Code for type indicator 001

| Decimal Value | Description |
| --- | --- |
| 0 | Used to mask Broadcast Assignment A-MAP IE when Function index is 0b00 or 0b01 |
| 1 | Used to mask BR-ACK A-MAP IE |
| 2-128 | Used to mask Group Resource Allocation A-MAP IE (group ID) |
| 129 | Used to mask Broadcast Assignment A-MAP IE when Function index is 0b10 |

-------------------------------------------------End proposed text----------------------------------------------------------------