| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification Text for Long TTI Transmission (Section 16.3.5.5) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Hyunkyu Yu, Sangheon Kim, Ji-Yun Seol, Jaeweon Cho Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies the text about long TTI transmission. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification Text for Long TTI Transmission (Section 16.3.5.5)

Hyunkyu Yu, Sangheon Kim, Ji-Yun Seol,

Jaeweon Cho

- Introduction

This contribution clarifies the text about DL and UL long TTI transmission.

- UL long TTI

  - Due to cell coverage, UL long TTI should be supported even before capability negotiation. Thus, we propose removing “long TTI for UL” in SBC-REQ/REP messages.

- DL long TTI

  - In case of DL, there’s no critical reason to support long TTI before capability negotiation. We recommend to delete “long TTI indicator” in CDMA allocation A-MAP IE (DL allocation for AAI-RNG-RSP).

- Proposed Text Changes

[Remedy-1: Change the following text, at line 6 in page 630, section 16.3.5.5.2.4.7, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 855—CDMA Allocation A-MAP IE*

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { | - | - |
| A-MAP IE Type | 4 | CDMA Allocation A-MAP IE |
| … | … | … |
| Else if (CDMA allocation indication == 0b1) { |  |  |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator=0b1 |
| ISizeOffset | 5 | Offset used to compute burst size index |
| … | … | … |
| If (Uplink/Downlink Indicator==0b0) { |  |  |
| … | … | … |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator=0b1 |
| Reserved | 1 | Reserved bit |
| } Else { |  |  |
| … |  |  |
| Reserved | 78 | Reserved bits |
| } |  |  |
| … | … | … |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Delete “Long TTI for UL” parameter, in page 100, line 23, table 683, section 16.2.3.5]

[Remedy-3: Delete “Long TTI for UL” parameter, in page 103, line 27, table 684, section 16.2.3.6]