| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification Text for Long TTI Transmission (Section 16.3.5.5) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Hyunkyu Yu, Jeongho Park, Ji-Yun Seol, Sangheon Kim, Jaeweon Cho Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D9 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies the text about long TTI transmission. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification Text for Long TTI Transmission (Section 16.3.5.5)

Hyunkyu Yu, Jeongho Park, Ji-Yun Seol,

Sangheon Kim, Jaeweon Cho

- Introduction

This contribution proposes the clarification on the text about DL and UL long TTI transmission.

- UL long TTI

  - In order to increase cell coverage through power concentration into small bandwidth, UL long TTI should be supported even before capability negotiation. That is, UL long TTI should be mandated to all AMSs and “long TTI for UL” is not needed in capability negotiation process. Thus, we propose removing “long TTI for UL” in SBC-REQ/REP messages for capability negotiation.

- DL long TTI

  - In case of DL, there’s no critical reason to support long TTI before capability negotiation. It means that DL long TTI capability support should be negotiated during the SBC process and thus the ABS cannot know whether the capability of DL long TTI is supported by the AMS or not before the SBC process. However, in D9, there is “long TTI indicator” in CDMA allocation A-MAP IE (DL allocation for AAI-RNG-RSP) which is used before capability negotiation. We recommend deleting “long TTI indicator” in CDMA allocation A-MAP IE (DL allocation for AAI-RNG-RSP).

- Proposed Text Changes

[Remedy-1: Change the following text, at line 6 in page 677, section 16.3.5.5.2.4.7, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 851—CDMA Allocation A-MAP IE*

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { | - | - |
| A-MAP IE Type | 4 | CDMA Allocation A-MAP IE |
| … | … | … |
| Else if (CDMA allocation indication == 0b1) { |  |  |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator=0b1 |
| ISizeOffset | 5 | Offset used to compute burst size index |
| … | … | … |
| If (Uplink/Downlink Indicator==0b0) { |  |  |
| … | … | … |
| AI_SN | 1 | HARQ identifier sequence number |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator=0b1 |
| } Else { |  |  |
| … |  |  |
| Reserved | 78 | Reserved bits |
| } |  |  |
| … | … | … |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Delete “Long TTI for UL” parameter, in page 107, line 18, table 682, section 16.2.3.5]

[Remedy-3: Delete “Long TTI for UL” parameter, in page 110, line 47, table 683, section 16.2.3.6]