| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal on HFA Resource Index (Section 16.3.5.3) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Hyunkyu Yu, Jeongho Park, Ji-Yun Seol Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies the text about HFA resource index. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal on HFA Resource Index (Section 16.3.5.3)

Hyunkyu Yu, Jeongho Park, Ji-Yun Seol

- Introduction

This contribution clarifies the text about both DL and UL HFA resource indexes.

The calculation of DL and UL HFA resource indexes require modulo operation using NHF-A-MAP and LHFB, respectively. But the current D8 does not give any rule for the following case.

- When NHF-A-MAP and LHFB (which are signaled via S-SFH SP1) are changed and those values are applied at n-th superframe, and if the HARQ burst is in progress across the superframe boundary between (n-1)-th and n-th.

We propose to calculate the HFA resource indexes using NHF-A-MAP and LHFB values in S-SFH SP1 which corresponds to S-SFH change count applied in the subframe where n (3-bit HFA in assignment A-MAP IE) is signaled.

- Proposed Text Changes

[Remedy-1: Change the following text, at line 59 in page 577, section 16.3.5.3.2.2, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

where n is a 3 bit HFA value in each assignment A-MAP IE, and jϵ{0, 1} is the value of the HF-A-MAP Index Parameter in the Non-user specific A-MAP IE which is transmitted in the subframe where n is signaled., and NHF-A-MAP is the maximum number of HF-A-MAP IEs configured per HF-A-MAP region., and is the value in S-SFH SP1 which corresponds to S-SFH change count applied in the subframe where n is signaled.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Change the following text, at line 41 in page 729, section 16.3.7.3.3.2, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

For resource allocation using the DL Basic Assignment A-MAP IE, DL Subband Assignnment A-MAP IE, CDMA Allocation A-MAP IE, and DL Persistent Allocation A-MAP IE, the index k is (M(j)+n) mod LHFB, where n is a 3 bit HFA value signaled in each Assignment A-MAP IEs, jϵ{0, 1} is the value of the HFBCH Index Parameter in the Non-user specific A-MAP IE which is transmitted in the subframe where n is signaled., LHFB is the total number of HFBCH configured per HARQ feedback region, and is the value in S-SFH SP1 which corresponds to S-SFH change count applied in the subframe where n is signaled.

M(j) is STID when j = 0 and M(j) is lowest LRU index of the corresponding DL transmission when j = 1. For the DL persistent allocation A-MAP IE, M(j)is always STID regardless of value j.

In order to determine a unique set of HFBCH resource indexes in a subframe, the ABS can set j to 0. In this case, M(j) is set to the STID for the DL Basic Assignment A-MAP IE and DL Subband Assignment A-MAP IE and M(j) is set to the RA-ID for CDMA Allocation A-MAP IE. Alternatively, a different set of HFBCH resource indexes can be obtained when M(j) is set to the lowest LRU index of the corresponding DL transmission when j=1. For the DL Persistent Allocation A-MAP, M(j) is always set to the STID regardless of the value of j.

-------------------------------------------------End proposed text----------------------------------------------------------------