| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Uplink Power Control (16.3.8.4) |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | Jeongho Park Samsung Electronics | jeongho.jh.park@samsung.com |
| Re: | Proposed text changes to P802.16m/D8 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D9 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Uplink Power Control (16.3.8.4)

Jeongho Park

Samsung Electronics

- References

[1] IEEE P802.16m/D8

- Introduction

The maximum Tx Power value of AMS can be reported to ABS during SBC-REQ stage [1]. This is for helping ABS’s scheduling in order to maximize uplink resource usage after SBC-REQ procedure of the AMS. If ABS recognizes the AMS’s maximum Tx Power, then ABS can calculate power headroom and assign the AMS an appropriate resource size (the number of LRUs) for the next UL transmission. However, if there is no information about it, there is almost nothing for ABS to do more for scheduling flexibility, which eventually bring waste of resource or lengthened initial network entry.

Similarly 16e has defined this function as mandatory as follows in page 1072 of IEEE802.16-2009:

“The SS shall report the maximum available power…”

For this reason, this contribution proposes to make reporting of MAX Tx Power mandatory.

- Proposed Text

Remedy #1: page 789, Modify line 8~13 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

However, the transmitted power level shall not exceed the maximum levels dictated by signal integrity considerations and regulatory requirements.

An AMS shall report the maximum available power by MAX Tx Power field when the AMS transmits AAI-SBC-REQ.

For interference level control, current interface level of each ABS may be shared among ABSs.

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #2: page 102, Modify line 52~59 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

| O M | MAX Tx Power | 24 | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. Each unsigned 8-bit integer specifies the maximum transmitted power value in dBm. The maximum transmitted power is quantized in 0.5 dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. If AMS does not support 64-QAM, the AMS shall report the value of 0x00 for Bit 15-23. | Present as needed |
| --- | --- | --- | --- | --- |

-------------------------------------------------Text Change END -----------------------------------------------------