| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the A-Preamble boosting levels (16.3.5.1) |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | Youngbo Cho, Sangheon Kim, Seonghyeoun Chae, Chiwoo Lim, Ki-chun Cho, Sung-Eun Park, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | youngbo.cho@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE 802.16m/D8 |  |
| Abstract | The contribution proposes the text changes to A-Preamble boosting levels for multi-carrier mode of the 802.16m draft amendment. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to A-Preamble boosting level (16.3.5.1)

Youngbo Cho, Sangheon Kim, Seonghyeoun Chae, Chiwoo Lim, Ki-chun Cho, Sung-Eun Park, Jaeweon Cho, Heewon Kang
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of the A-Preamble boosting levels to be included in the 802.16m amendment.

In the current draft amendment [1], the concept of the boosting level of preamble is inconsistent with that of MIMO midamble and pilot/data symbol. It should be clarified to avoid confusion.

And, there is no description of A-Preamble boosting levels for multi-carrier mode. In calculation of boosting level of regular bandwidth, it’s chosen such that the maximum peak value of PA-Preamble/SA-Preamble in time domain should not exceed the peak value of 16e preamble which has 9dB boosting. However, in the multi-carrier, the peak powers of PA-Preamble/SA-Preamble with the present boosting levels are exceeded the dynamic range of ABS’s amplifier (which is same as 98%-tile value that is CDF of the peak values of data symbols). So, new reference values in multi-carrier mode are required.

- References

[1] IEEE P802.16m/D8, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-08-25

- Rationale of modifications

In D8 [1], A-Preamble boosting levels are chosen such that the maximum peak value of PA-Preamble/ SA-Preamble in the single-carrier case. However, in the multi-carrier case, the peak powers of PA-Preamble/ SA-Preamble with the present boosting levels are exceeded the dynamic range of ABS’s amplifier (which is same as 98%-tile value that is CDF of the peak values of data symbols).

Table 1 shows the 98%-tile CDF peak values of the data symbols

Table 1. 98%-tile CDF peak power values of data symbols (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | 0.68 | 1.01 | 1.19 |
| 1k | 0.95 | 1.27 | 1.29 |
| 2k | 1.22 | 1.50 | 1.52 |

Pilot randomization is applied for the simulation.

Table 2 shows the PA-Preamble peak power with present boosting levels.

Table 2. Boosted PA-Preamble Peak Power (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.08 | 1.49 | 3.13 |
| 1k | -1.09 | 1.78 | 3.41 |
| 2k | -0.66 | 2.35 | 3.99 |

Table 3-a, 3-b, 3-c, and 3-d present the SA-Preamble peak power with present boosting levels at the 1, 2, 3, and 4 antennas, respectively.

Table 3-a. Boosted SA-Preamble Peak Power with 1 antenna (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.06 | 1.93 | 3.57 |
| 1k | -0.99 | 1.83 | 3.47 |
| 2k | -0.36 | 2.64 | 4.28 |

Table 3-b. Boosted SA-Preamble Peak Power with 2 antennas (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.08 | 1.49 | 3.13 |
| 1k | -1.09 | 1.78 | 3.47 |
| 2k | -0.66 | 2.35 | 3.99 |

Table 3-c. Boosted SA-Preamble Peak Power with 4 antennas (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.06 | 1.83 | 3.49 |
| 1k | -1.00 | 2.01 | 3.64 |
| 2k | 0.72 | 3.71 | 5.35 |

Table 3-d. Boosted SA-Preamble Peak Power with 8 antennas (dB)

| FFT\# of Carriers | 1 CA | 2 CA | 3 CA |
| --- | --- | --- | --- |
| 512 | -1.05 | 1.95 | 3.59 |
| 1k | -1.02 | 1.93 | 3.63 |
| 2k | -0.83 | 2.17 | 3.81 |

Simulation results show that the peak powers of PA-Preamble/SA-Preamble with the present boosting levels are exceeded the 98%-tile value that is CDF of the peak values of data symbols in the multi-carrier. Therefore, the modification of boosting values is required. Table 4 present the modified boosting levels for PA-Preamble which are chosen by following rules.

- The maximum peak values of PA-Preamble are set to same as 98%-tile value that is CDF of the peak values of data symbols.

- For 512 and 1024 FFT cases, the modified value is chosen at the carrier aggregation mode with 3 carriers.

- For 2048 FFT case, the modified value is chosen at the carrier aggregation mode with 2 carriers.

In this case, if different carriers in an ABS use different preamble sequences and segments, the peak power of SA-Preamble can be reduced. Therefore, only modification of the PA-preamble boosting levels is needed.

Table 4. Modified PA-Preamble boosting levels

| FFT | Present PA-Preamble boosting level | Modified PA-Preamble boosting level | Difference (dB) |
| --- | --- | --- | --- |
| 512 | 2.3999 (7.60 dB) | 1.9216 (5.67 dB) | -1.93 |
| 1k | 3.4143 (10.67 dB) | 2.6731 (8.54 dB) | -2.13 |
| 2k | 5.132 (14.21 dB) | 4.6511 (13.35 dB) | -0.85 |

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy-1: Modify the following text on page 541 line 53 as following text]

The magnitude boosting levels in single carrier mode for different FFT size are shown in Table 820.

The magnitude boosting levels for different FFT size are shown in Table XXX.

[Remedy-2: Modify the table 820 on page 541]

Table 820XXX – PA-Preamble boosting levels

| 512 | 1k | 2k |
| --- | --- | --- |
| 2.39991.9216 | 3.41432.6731 | 5.13204.6511 |

[Remedy-3: Modify the following text on page 560 line 46-48 as following text]

The magnitude boosting levels in single carrier mode for different FFT size and number of antennas are shown in Table 826.

The magnitude boosting levels for different FFT size and number of antennas are shown in Table 826.

---------------------------------------------------  Text End  ---------------------------------------------------