| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Conversion of LBS Functional Area MAC message tables to ASN.1 |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alessandro Triglia, OSS Nokalva Scott Probasco, Nokia | E-mail:sandro@oss.com scott.probasco@nokia.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to convert LBS Functional MAC control messages into ASN.1. It is part of a coordinated effort to introduce up-to-date ASN.1 coding for all MAC messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Conversion of LBS Functional Area MAC message tables to ASN.1

- Alessandro Triglia

- OSS Nokalva

- Scott Probasco

- Nokia

- Introduction

This contribution is one of several contributions which are all part of a coordinated effort to include updated ASN.1 coding of all MAC control messages in P802.16m. The work is split along the “Functional Areas” defined in Table 678 of P802.16m/D8.

In this coordinated effort, any changes to the ASN.1 coded messages as contained in Annex P of P802.16m/D7 are shown with markup. This allows the members of the workgroup to easily determine what changes are proposed from version D7 of the draft.

This contribution introduces ASN.1 coding for the MAC messages in the LBS Functional Area (AAI-LBS-ADV and AAI-LBS-IND). There were no changes to the AAI-LBS-ADV message or AAI-LBS-IND message during session #68.5, thus no changes are shown below. The purpose of this contribution is to prepare for Session #69 and any possible changes to these two messages during this session. Any changes applied to the definition of AAI-LBS-ADV or AAI-LBS-IND messages during the meeting can be also applied to the ASN.1 coded messages below, allowing the group to see the changes and approve those changes. Thus at the conclusion of Session #69 these messages will be fully up-to-date.

- Proposed text

Annex P.2 MAC Control Message Definitions

[Editor Note: update the text in Annex P.2 from P979 L6 to P980 L26 with the following marked changes ]

------------------------------------------------- Start of proposed text --------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- LBS Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Message specific types

RelativePosition ::= SEQUENCE {

longitude INTEGER (0..65535),

latitude INTEGER (0..65535),

altitude INTEGER (0..65535) OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- LBS Advertizement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-LBS-ADV ::= SEQUENCE {

absLocation CHOICE {

absoluteLongFormat SEQUENCE {

longitude SEQUENCE {

resolution INTEGER (0..63),

integer INTEGER (0..511),

fraction INTEGER (0..33554431)

},

latitude SEQUENCE {

resolution INTEGER (0..63),

integer INTEGER (0..511),

fraction INTEGER (0..33554431)

},

altitude SEQUENCE {

unit ENUMERATED { meter, floor, ...},

resolution INTEGER (0..63),

integer INTEGER (0..4194303),

fraction INTEGER (0..255)

}

},

absoluteShortFormat SEQUENCE {

longitude INTEGER (0..16777215),

latitude INTEGER (0..16777215),

altitude INTEGER (0..65535) OPTIONAL

}

},

gpsTime SEQUENCE {

time INTEGER (0..4194303),

frameTransmissionTimeOffset INTEGER (0..1023),

accuracy INTEGER (0..31)

},

frequencyAccuracy INTEGER (0..255),

bsInfoArray SEQUENCE (SIZE(1..255)) OF SEQUENCE {

bsid BSID,

relativePosition RelativePosition

} OPTIONAL,

aaiNbrAdvConfigChangeCount INTEGER (0..255) OPTIONAL,

bsIndexInfoArray SEQUENCE (SIZE(1..256)) OF SEQUENCE {

nbrBSIndex INTEGER (0..255),

relativePosition RelativePosition

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- LBS Indication

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-LBS-IND ::= SEQUENCE {

frameIdentifier INTEGER (0..15),

dedicatedRangingCode INTEGER (0..31),

rangingOppIndex INTEGER (0..7),

subframeIndex INTEGER (0..3),

nbrAbsInfoList SEQUENCE (SIZE(1..255)) OF SEQUENCE {

nbrAbsIndex INTEGER (0..255),

actionTime INTEGER (0..255),

nbrDedicatedRangingCode INTEGER (0..31),

nbrRangingOppIndex INTEGER (0..7),

nbrSubframeIndex INTEGER (0..3)

} OPTIONAL,

...

}

------------------------------------------------- End of proposed text --------------------------------------------------