| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on D-LBS Zone Transmission for Frame Structures Supporting WirelessMAN-OFDMA (16.8.3) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alexey Khoryaev, Alexander Maltsev, Mikhail Shilov, Andrey Chervyakov, Gregory Morozov Intel Corporation | E-mails: alexey.khoryaev@intel.com alexander.maltsev@intel.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 Working Group Sponsor Ballot Topic: LBS (Section 16.8.3) |  |
| Abstract | This contribution provides clarification on D-LBS zone transmission for frame configurations with WirelessMAN-OFDMA support |  |
| Purpose | To be discussed and adopted by TGm for the next draft of the IEEE 802.16m amendment document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on D-LBS Zone Transmission for Frame Structures Supporting WirelessMAN-OFDMA

Alexey Khoryaev, Alexander Maltsev, 
Mikhail Shilov, Andrey Chervyakov, Gregory Morozov

Intel Corporation

- Introduction

This contribution provides clarification of D-LBS zone transmission for frame structure configurations supporting WirelessMAN-OFDMA. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [1] and the IEEE 802.16m SDD [2], and it follows the style and format guidelines in [3].

In accordance with the current D8 of the IEEE 802.16m amendment [4] the location beacons of D-LBS zone shall be transmitted “on the first symbol of the first subframe of the last frame of each D-LBS zone superframe”. This position in frame is valid for green field AAI deployments however can not be applied for frame structures supporting WirelessMAN-OFDMA air interface. This contribution provides clarification of D-LBS zone transmission for frame structures supporting WirelessMAN-OFDMA. It is proposed that the offset mechanism applied for coexistence of AAI and WirelessMAN-OFDMA subframes is reused for D-LBS zone transmission in frame configurations supporting WirelessMAN-OFDMA.

- References

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D8, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D6 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Change the following text in section 16.8.3 Enhanced LBS Support on page 893 (line 37) as follows]

16.8.3.1.1 Basic description of downlink LBS zone

To enable enhanced location determination, a dedicated Downlink LBS zone (D-LBS zone) may be defined to transmit location beacons to be received by AMSs and facilitate their measurement of location related parameters (RD, RTD, RSSI, etc.) with the higher accuracy.

Such dedicated D-LBS zone, when supported, shall be spread over four consecutive superframes. The location beacon shall be transmitted on the first symbol of the first subframe of the last frame of each D-LBS zone superframe. For frame structures supporting WirelessMAN-OFDMA the offset shall be applied as it is specified in 16.3.3.5.

========================== End of Proposed Text 1 =============================

=========================Start of Proposed Text 2 ==============================

[Note to Editor: Add the following underlined blue text into 16.8.3 Enhanced LBS Support section on page 893 line 48]

16.8.3.1.2 Allocation of D-LBS zone in frame structure

The one D-LBS zone shall span four consecutive superframes. When the D-LBS zone is activated the first symbol of the first subframe of the last frame of superframe, that belong to D-LBS zone shall be replaced by location beacon of AAI subframe of the last frame of superframe that belong to D-LBS zone shall be used for location beacon transmission. For the type-1 AAI subframe case, the remaining 5 consecutive symbols shall form a type-3 AAI subframe. For the type-2 AAI subframe case, the remaining 6 consecutive symbols shall form a type-1 AAI subframe. Figure 607 shows an example of the D-LBS zone allocation.

========================== End of Proposed Text 2 =============================

=========================Start of Proposed Text 3 ==============================

[Note to Editor: Correct the reference in section 16.8.3.2  on page 894 line 40]

16.8.3.2 D-LBS zone location beacon signals

The SA-Preamble shall be used as a reference location beacon signal for transmission inside of D-LBS zone. The physical structure of the SA-Preamble signal transmitted by each ABS or ARS in the D-LBS zone shall be the same as defined in section 16.3.6.1.2 16.3.5.1.2 for given frame.

========================== End of Proposed Text 3 =============================