| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | D-LBS Zone Scanning Indication (16.8.3) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alexey Khoryaev, Alexander Maltsev, Mikhail Shilov, Andrey Chervyakov, Gregory Morozov Intel Corporation | E-mails: alexey.khoryaev@intel.com alexander.maltsev@intel.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 Working Group Sponsor Ballot Topic: LBS (Section 16.8.3) |  |
| Abstract | This contribution provides clarification on D-LBS zone scanning indication |  |
| Purpose | To be discussed and adopted by TGm for the next draft of the IEEE 802.16m amendment document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

D-LBS Zone Scanning Indication

Alexey Khoryaev, Alexander Maltsev, 
Mikhail Shilov, Andrey Chervyakov, Gregory Morozov

Intel Corporation

- Introduction

This contribution provides clarification of D-LBS zone scanning indication. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [1] and the IEEE 802.16m SDD [2], and it follows the style and format guidelines in [3].

In accordance with the current D8 of the IEEE 802.16m [4], an ABS may trigger a location measurement on D-LBS zone by sending AAI_SCN-RSP to the AMS. The AAI_SCN-RSP response defines the optional field “Duration for measurements signal location parameters on D-LBS zone” that defines the duration of the requested scanning in units of superframes. This field shall be present in AAI_SCN-RSP MAC control message if ABS request AMS to scan D-LBS zone for signal location parameters measurements. However currently AMS can not recognize whether the measurements shall be performed over D-LBS and that purpose of the measurements is the network based positioning of the AMS.

So it is proposed to add to AAI_SCN-RSP message one optional bit that will indicate to the AMS that AAI_SCN-RSP message is used to trigger measurement over D-LBS zone. This bit shall be present only if the D-LBS zone configuration field of S-SFH-SP3 is not equal 0 (i.e. if D-LBS zone is transmitted by the network). If AMS recognizes that scanning over D-LBS zones is requested then the duration of D-LBS zone scanning can be represented as the number of consecutive D-LBS zones to be processed by AMS for signal location parameters measurements.

- References

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D8, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D8 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Change the following text in section 16.8.3 Enhanced LBS Support on page 895 (line 49) as follows]

16.8.3.4 D-LBS zone measurement and report

An ABS shall may trigger a location measurement on D-LBS zone by sending AAI_SCN-RSP to the AMS by setting Enhanced LBS scanning indication bit to 1.  The AMS shall responds to this with AAI_SCN-REP with the parameters that enable location determination. (See 16.2.3.14 and 16.2.3.15)

========================== End of Proposed Text 1 =============================

=========================Start of Proposed Text 2 ==============================

[Note to Editor: Add the following underlined blue text into 16.8.3 Enhanced LBS Support section on page 136 line 23]

Table 694—AAI-SCN-RSP parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| M | Report mode | 2 | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| M | Report metric | 4 | Bitmap indicator of trigger metrics that the S-ABS requests the AMS to report. The S-ABS shall indicate only the trigger metrics agreed during AAI-SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: ABS CINR mean Bit 1: ABS RSSI mean Bit 2: Relative delay Bit 3: ABS RTD |  |
| M | Report period | 8 | The period of AMS's report of scanning result when the AMS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the ABS sends an unsolicited AAI-SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the AMS receives the AAI-SCN-RSP message. For AMS scanning request denied by AAI-SCN-RSP with Scan Duration set to zero, Report period is the number of frames that ABS suggests to AMS before transmitting next AAI-SCN-REQ |  |
| O | D-LBS zone scanning indication | 1 | Indication that scanning is triggered for the measurements of signal location parameters over D-LBS zone location beacons (if set to 1 measure signal location parameters by processing D-LBS zone location beacons). | Present only when D-LBS zone configuration field of S-SFH-SP3 is > 0. |
| O | Number of D-LBS zones | 4 | Number of consecutive D-LBS zone to be used for signal location parameters measurements | Present only when enhanced LBS scanning indication is set to 1. |
| O | Duration for measurements signal location parameters on D-LBS zone | 4 | Duration (in units of 4 superframes) of the requested scanning. | Present if ABS request AMS to scan D-LBS zone for signal location parameters measurements |
| O | start super frame index | 6 | Represents the 6 least significant bits of the absolute super frame index in which the first Scanning interval starts | Present only when Scan Duration > 0 |

========================== End of Proposed Text 2 =============================