| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CLC Limit Parameter Clarification |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to clarify CLC Limit Parameter |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- CLC Limit Parameter Clarification

- Joey Chou

- Intel

- Introduction

This contribution is intended to clarift CLC limit parameter in AAI-REG-RSP.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.9 AAI_REG-RSP

[Editor note: The following table only includes the attributes that have been changed.]

Table 687—AAI_REG-RSP

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | CLC Limit | array | see Table 688 | present if ABS configures the CLC Limit parameters different from the default values in Table 770, and may include up to 2 CLC Limit parameter sets |
| O | For (i = 0 ; i < N ; i++) { | Variable | CLC Limit. The maximum value of N is 2 | present if ABS configures the CLC Limit parameters different from the default values in Table 770 |
| M | Type I Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type I CLC class | N/A |
| M | Type II Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type II CLC class | N/A |
| M | Active Class Limit | 3 | 0: the maximum number of active CLC classes is 8 1..7: indicates the maximum number of active CLC classes | N/A |
| M | Active Ratio Limit | 6 | The field indicates the maximum CLC active ratio. If the Active Ratio Limit is x, the maximum CLC active ratio shall be x/100. | N/A |
| M | Active Interval Limit | 5 | The field indicates the maximum CLC active interval. If the Active Interval Limit is x, and a frame consists of m AAI subframes, the maximum CLC active interval shall be x multiply m AAI subframes. | N/A |
|  | } |  |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II--------------------------------------------------

[Editor note: Remove section 16.2.3.9.1.]

------------------------------------------------- End of proposed text II --------------------------------------------------