| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Resource Index (RI) Encoding for Assignment IEs (16.3.5.5.2.4.1) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-3 for P802.16m/D9. |  |
| Abstract | The contribution proposes an alternative RI encoding scheme for Assignment IEs. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Resource Index (RI) Encoding for Assignment IEs (16.3.5.5.2.4.1)

Lei Wang

InterDigital Communications, LLC

- Introduction

This contribution proposes an alternative Resource Index (RI) encoding scheme as a follow-up effort to the comment B155 in 802.16-10/0047r3 and the comment A036 in 802.16-10/0045r2, about allocation granularity in the 20MHz system bandwidth.

As pointed out by comment the comment B155 in 802.16-10/0047r3 and A036 in 802.16-10/0045r2, there were some serious doubts about the correctness of the original analysis, e.g., conclusions based on 1/6 < 31/1422.

In addition, we think sacrificing the allocation granularity seems not a good design choice, particularly at steps as big as 8 LRUs. Even with code-matching schemes, the offset of the required size to the nearest allowed S value can be up to 4 LRUs.  This makes the ratio of the offset to the assigned size is greater than majority of the code steps based on the nominal MCS table given in Table 927, on page 825 in 16m/D9.

We would recommend reconsidering the RI field encoding issue, particularly for the 20MHz system bandwidth, instead of just sacrificing the allocation granularity,  looking for some other alternatives, e.g., change the RI field from 11 bits to 12 bits by using the 1 reserved bit, and/or consider the constraints of the allocations to remove those ones that do not need  to be signaled by the assignment A-MAP IEs, e.g., the control channel occupied resources, and/or allocations spanning over multiple frequency partitions, etc.

Note that this contribution, C80216m-10_1195r1, proposes the same technical solution as its previous version, C80216m-10_1195. The only differences of this version from its previous version are that the references are changed to 802.16m/D9 from 802.16m/D8.

- Discussions

For simplicity and also for the purpose of triggering the discussion, this contribution proposes:

- Change the RI field from 11 bits to 12 bits by using the 1 reserved bit, which gives a total of 4096 RI codes;

- For 20MHz systems, reduce the allowed assignable sizes, i.e., allow all the resource sizes except the odd resource sizes in the range of 46 to 80 LRUs, which gives a total of 4078 combinations of allocation size (S) and location (L).

With the above proposal, the allocation granularity issue is significantly released, as the maximum step size is reduced from 8 LRUs to 2 LRUs and also the number of disallowed sizes is reduced from 61 to 17.  In addition, since we follow the same RI usage design in the current spec, the required changes are minimized.

- Suggested changes in the 802.16m/D9

The following is the proposed change in the 802.16m/D9. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 59, page 650

Change line 59 to 65 on page 650 as follows:

•All resource sizes in increments of 1 LRU are assignable in the range of 1 to 46 12 LRUs.

•Only even resource sizes are assignable in increments of 2 LRUs in the range of 47 14 to 79 24 LRUs.

•Resource sizes are assignable in increments of 4 LRUs in the range of 28 to 48 LRUs .

•Resource sizes are assignable in increments of 8 LRUs in the range of 56 to 88 LRUs.

•All resource sizes in increments of 1 LRU are assignable in the range of 80 92 to 96 LRUs.

Suggested change #2: line 1, page 652, Formula (227)

Replace the formula (227) in line 1 to 15 on page 652 by the following:

Suggested change #3: line 8, page 650

Change the row of Resource Index in Table 844 as follows:

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Resource Index. | 12 11 | 5 MHz: 0 in first 3 2 MSB bits + 9 bits for resource index 10 MHz: 0 in first 1 MSB bit + 11 bits for resource index 20 MHz: 12 11 bits for resource index Resource index includes location and allocation size. |

Suggested change #4: line 37, page 650

Remove the row of the “Reserved” row.

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”