| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Conversion of ARQ Functional Area MAC message tables to ASN.1 |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alessandro Triglia, OSS Nokalva | E-mail:sandro@oss.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to convert ARQ Functional MAC control messages into ASN.1. It is part of a coordinated effort to introduce up-to-date ASN.1 coding for all MAC messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Conversion of ARQ Functional Area MAC message tables to ASN.1

- Alessandro Triglia

- OSS Nokalva

- Introduction

This contribution is one of several contributions which are all part of a coordinated effort to include updated ASN.1 coding of all MAC control messages in P802.16m. The work is split along the “Functional Areas” defined in Table 678 of P802.16m/D8.

In this coordinated effort, any changes to the ASN.1 coded messages as contained in Annex R.2 of P802.16m/D8 are shown with markup. This allows the members of the workgroup to easily determine what changes are proposed from version D8 of the draft. These changes are intended to align the ASN.1 specification in Annex R.2 of P802.16m/D8 to the MAC message tables present in the same draft.

This contribution introduces ASN.1 coding for the MAC messages in the ARQ Functional Area (AAI-ARQ-FBK, AAI-ARQ-DSC, and AAI-ARQ-RST).

There were no changes to the messages in the ARQ Functional Area during session #68.5, thus no changes are shown below. The purpose of this contribution is to prepare for Session #69 and any possible changes to these two messages during this session.

Any changes applied to the definition of AAI-ARQ-FBK, AAI-ARQ-DSC, and AAI-ARQ-RSTduring the meeting can be also applied to the ASN.1 coded messages below, allowing the group to see the changes and approve those changes. Thus at the conclusion of Session #69 these messages will be fully up-to-date.

- Proposed text

Annex R.2 MAC Control Message Definitions

[Editor Note: update the text in Annex R.2 from P950 L37 to P952 L17 with the following marked changes]

------------------------------------------------- Start of proposed text --------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- ARQ Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

--

ArqBlockSn ::= INTEGER (0..1023)

-- Maximum size of ARQFeedbackIe is the maximum number of transport

-- flow IDs

ARQFeedbackIe ::= SEQUENCE (SIZE(0..11)) OF SEQUENCE {

lastArqFeedbackIe ENUMERATED {

moreArqIe,

lastArqIe

},

fid FID,

arqFeedbackIeType CHOICE {

cumulativeAck SEQUENCE {

-- indicates ARQ blocks up to and including the sequence number

-- in the SN field have been received successfully.

sequenceNumber ArqBlockSn

},

selectiveAck SEQUENCE {

-- indicates ARQ blocks less than the sequence number in the SN

-- field have been received successfully.

sequenceNumber ArqBlockSn,

selectiveAckInfo SEQUENCE {

nackSuspendedIndicator ENUMERATED {

zeroIndicatesNACK,

zeroIndicatesNACKSuspended

},

-- Each bit in the map represents ACK or NACK or NACK Suspended

-- of corresponding ARQ block.

-- 0: NACK or NACK suspended, 1: ACK

-- Maximum number of selectiveAckMap is the ARQ_Window_size

-- (512)

selectiveAckMap BIT STRING (SIZE(1..512)),

-- The numbers of bits in Sub-block-Existence Map is equal to

-- the number of bits that indicates the NACKed ARQ blocks in

-- the Selective ACK MAPs.

-- 1: Partially NACKed ARQ block,

-- 0: Completely NACKed ARQ blocks

subBlockExistenceMap BIT STRING (SIZE(1..512)) OPTIONAL,

ssnSeriesList SEQUENCE (SIZE(1..512)) OF SEQUENCE {

-- Start of ARQ sub-block SN which was received correctly.

startSsnRecvCorrectly INTEGER (0..2047),

-- Number of consecutive ARQ sub-blocks which were received

-- correctly from START_SSN onwards

numOfSsnRecvCorrectly INTEGER (0..2047)

}

}

}

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Standalone ARQ Feedback

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-ARQ-FBK ::= SEQUENCE {

arqFeedback ARQFeedbackIe,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- ARQ Discard

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-ARQ-DSC ::= SEQUENCE {

fid FID,

sequenceNumber ArqBlockSn,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- ARQ Reset

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-ARQ-RST ::= SEQUENCE {

directionIndicator DirIndicator,

fid FID,

type ENUMERATED {

originalMessage,

acknowledgement,

confirmation

},

sequenceNumber ArqBlockSn,

...

}

------------------------------------------------- End of proposed text --------------------------------------------------