| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Conversion of FFR Functional Area MAC message tables to ASN.1 |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alessandro Triglia, OSS Nokalva | E-mail:sandro@oss.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to convert FFR Functional MAC control messages into ASN.1. It is part of a coordinated effort to introduce up-to-date ASN.1 coding for all MAC messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Conversion of FFR Functional Area MAC message tables to ASN.1

- Alessandro Triglia

- OSS Nokalva

- Introduction

This contribution is one of several contributions which are all part of a coordinated effort to include updated ASN.1 coding of all MAC control messages in P802.16m. The work is split along the “Functional Areas” defined in Table 678 of P802.16m/D8.

In this coordinated effort, any changes to the ASN.1 coded messages as contained in Annex R.2 of P802.16m/D8 are shown with markup. This allows the members of the workgroup to easily determine what changes are proposed from version D8 of the draft. These changes are intended to align the ASN.1 specification in Annex R.2 of P802.16m/D8 to the MAC message tables present in the same draft.

This contribution introduces ASN.1 coding for the MAC messages in the FFR Functional Area (AAI-FFR-CMD and AAI-FFR-REP).

There were no changes to the messages in the FFR Functional Area during session #68.5, thus no changes are shown below. The purpose of this contribution is to prepare for Session #69 and any possible changes to these two messages during this session.

Any changes applied to the definition of AAI-FFR-CMD and AAI-FFR-REP during the meeting can be also applied to the ASN.1 coded messages below, allowing the group to see the changes and approve those changes. Thus at the conclusion of Session #69 these messages will be fully up-to-date.

- Proposed text

Annex R.2 MAC Control Message Definitions

[Editor Note: update the text in Annex R.2 from P976 L6 to P977 L4 with the following marked changes]

------------------------------------------------- Start of proposed text --------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- FFR Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

InterferenceMean ::= INTEGER (0..255)

InterferenceVariance ::= INTEGER (0..15)

SINRMean ::= INTEGER (0..255)

SINRVariance ::= INTEGER (0..15)

FFR-FeedbackIE ::= SEQUENCE {

interferenceMean InterferenceMean OPTIONAL,

interferenceVariance InterferenceVariance OPTIONAL,

sinrMean SINRMean OPTIONAL,

sinrVariance SINRVariance OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- FFR Measurement Report Command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-FFR-CMD ::= SEQUENCE {

frequencyPartitionBitmap BIT STRING (SIZE(4)),

reportType BIT STRING {

interferenceMean (0),

interferenceVariance (1),

sinrMean (2),

sinrVariance (3)

} (SIZE(4)),

frameOffset INTEGER (0..255),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- FFR Measurement Report

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-FFR-REP ::= SEQUENCE {

frequencyPartitionBitmap BIT STRING (SIZE(4)),

reportType BIT STRING {

interferenceMean (0),

interferenceVariance (1),

sinrMean (2),

sinrVariance (3)

} (SIZE(4)),

ffrReports SEQUENCE (SIZE(4)) OF SEQUENCE {

report FFR-FeedbackIE OPTIONAL

}, --one for each frequency partition

...

}

------------------------------------------------- End of proposed text --------------------------------------------------