| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Conversion of Handover Functional Area MAC message tables to ASN.1 |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alessandro Triglia, OSS Nokalva | E-mail:sandro@oss.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to convert Handover Functional MAC control messages into ASN.1. It is part of a coordinated effort to introduce up-to-date ASN.1 coding for all MAC messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Conversion of Handover Functional Area MAC message tables to ASN.1

- Alessandro Triglia

- OSS Nokalva

- Introduction

This contribution is one of several contributions which are all part of a coordinated effort to include updated ASN.1 coding of all MAC control messages in P802.16m. The work is split along the “Functional Areas” defined in Table 678 of P802.16m/D8.

In this coordinated effort, any changes to the ASN.1 coded messages as contained in Annex R.2 of P802.16m/D8 are shown with markup. This allows the members of the workgroup to easily determine what changes are proposed from version D8 of the draft. These changes are intended to align the ASN.1 specification in Annex R.2 of P802.16m/D8 to the MAC message tables present in the same draft.

This contribution introduces ASN.1 coding for the MAC messages in the Handover Functional Area (AAI-HO-IND, AAI-HO-REQ, AAI-HO-CMD, AAI-NBR-ADV, AAI-NBR-REQ, AAI-SCN-REQ, AAI-SCN-RSP, and AAI-SCN-REP).

Any changes applied to the definition of AAI-HO-IND, AAI-HO-REQ, AAI-HO-CMD, AAI-NBR-ADV, AAI-NBR-REQ, AAI-SCN-REQ, AAI-SCN-RSP, and AAI-SCN-REP during the meeting can be also applied to the ASN.1 coded messages below, allowing the group to see the changes and approve those changes. Thus at the conclusion of Session #69 these messages will be fully up-to-date.

- Proposed text

Annex R.2 MAC Control Message Definitions

[Editor Note: update the text in Annex R.2 from P955 L21 to P962 L16 with the following marked changes]

------------------------------------------------- Start of proposed text --------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Handover Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

StartFrameNumber ::= INTEGER (0..3)

SuperframeNumberLSB ::= INTEGER (0..63)

ABSID ::= BSID

CarrierIndexArray ::= SEQUENCE (SIZE(0..63)) OF PhyCarrierIndex

ABSCINRMean ::= INTEGER (0..255)

ABSRSSIMean ::= INTEGER (0..255)

RelativeDelay ::= INTEGER (0..255)

ABSRTD ::= INTEGER (0..255)

HandoverEventCode ::= ENUMERATED {

targetABSSelection,

allTargetABSesUnreachable,

servingABSUnreachable,

hoCancel

}

ServiceLevelPrediction ::= ENUMERATED {

noServicePossible,

serviceAvailableFromSomeServiceFlows,

macConnectionPossibleWithSpecifiedQoS,

noPredictionAvailable

}

PreassignedCarrierInfo ::= SEQUENCE {

carrierStatus ENUMERATED {

doNotActivate,

activateImmediatelyAfterHO },

preAssignedSecondaryCarrier PhyCarrierIndex

}

CPLength ::= ENUMERATED {

oneSixteenth,

oneEighth,

oneFourth

}

SFHDeltaInfo ::= SEQUENCE { -- ??? Needs clarification

sfhSubpacket1 SFHSubpacket1 OPTIONAL,

sfhSubpacket2 SFHSubpacket2 OPTIONAL,

sfhSubpacket3 SFHSubpacket3 OPTIONAL

}

CarrierInfo ::= SEQUENCE {

saPreambleIndex PreambleIndex,

saPreambleTransmitPower INTEGER (0..255),

carrierIndex PhyCarrierIndex,

pagingCarrier BOOLEAN OPTIONAL,

pagingGroupID PGID,

ssfhChangeCount INTEGER (0..15)

}

SFHEncodingFormat ::= ENUMERATED {

fullSubpacketInfo,

deltaFromCurrentCarrier,

deltaFromPrecedingCarrier,

noSFHIncluded

}

SFHSubpacket1 ::= SEQUENCE {

-- ??? what is the definition?

}

SFHSubpacket2 ::= SEQUENCE {

-- ??? what is the definition?

}

SFHSubpacket3 ::= SEQUENCE {

-- ??? what is the definition?

}

PhyModeID ::= SEQUENCE { -- ??? Definition?

--channelBandwidth

--fftSize

--cyclicPrefix

--frameDuration

}

CarrierIndexList ::= SEQUENCE (SIZE(0..63)) OF SEQUENCE {

carrierIndex PhyCarrierIndex,

cinrMean ABSCINRMean OPTIONAL,

rssiMean ABSRSSIMean OPTIONAL,

relativeDelay RelativeDelay OPTIONAL

}

ScanPurpose ::= ENUMERATED {

listedInNbrAdv,

notListedInNbrAdv

}

ReportMode ::= ENUMERATED {

noReport,

periodicReport,

eventTriggeredReport,

oneTimeScanReport

}

ReportMode2 ::= ENUMERATED {

eventTriggeredReport,

periodicReport

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- AMS Handover Indication

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HO-IND ::= SEQUENCE {

hoEventCode CHOICE {

targetABSSelection TargetABSSelection, -- 0b00

targetABSUnreachable TargetABSSelection, -- 0b01

servingABSUnreachable TargetABSSelection, -- 0b10

handoverCancel HandoverCancel

}, -- 0b11

...

}

TargetABSSelection ::= SEQUENCE {

targetABSID ABSID,

targetABScarrier PhyCarrierIndex OPTIONAL,

amsCarrierAtServingABS PhyCarrierIndex OPTIONAL

}

HandoverCancel ::= SEQUENCE {

sfhMismatchIndication ENUMERATED {

sfhMismatch,

sfhMatch

} OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- AMS Handover Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HO-REQ ::= SEQUENCE {

nbrAdvChangeCount INTEGER (0..255) OPTIONAL,

carrierPreassignment BOOLEAN OPTIONAL,

nbrABSInfoList SEQUENCE (SIZE(0..511)) OF SEQUENCE {

nbrAbsIdInfo CHOICE {

nbrAbsIndex AbsIndex,

nbrAbsId ABSID

},

absCINRMean ABSCINRMean OPTIONAL,

absRSSIMean ABSRSSIMean OPTIONAL,

absCarrierIndex PhyCarrierIndex OPTIONAL

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- AMS Handover Command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HO-CMD ::= SEQUENCE {

mode CHOICE {

hoCommand HOCommand,

zoneSwitchCommand ZoneSwitchCommand,

amsHORequestRejected NULL

},

...

}

HOCommand ::= SEQUENCE {

hoReentryMode ENUMERATED {

amsDiscFromServingABS,

amsMaintainsCommunication

},

hoReentryInterleavingInterval INTEGER (0..255) OPTIONAL,

hoReentryInterval INTEGER (0..255) OPTIONAL,

hoReentryIteration INTEGER (0..7) OPTIONAL,

amsCarrierAtServingABS PhyCarrierIndex OPTIONAL,

disconnectTimeOffset INTEGER (0..255) OPTIONAL,

resourceRetainTime INTEGER (0..255) OPTIONAL,

targetBSInfo SEQUENCE (SIZE(0..255)) OF SEQUENCE {

targetBSID BSID OPTIONAL,

saPreambleIndex PreambleIndex OPTIONAL,

faIndex FAIndex OPTIONAL,

actionTime INTEGER (0..255) OPTIONAL,

cdmaBasedRanging CHOICE {

noRanging SEQUENCE {

offsetData INTEGER(0..127),

offsetControl INTEGER(0..127)

},

ranging SEQUENCE {

dedicatedCDMARngCode INTEGER (0..31) OPTIONAL,

rngOpportunity INTEGER (0..7) OPTIONAL,

rngOpportunitySubframe INTEGER (0..3) OPTIONAL

}

} OPTIONAL,

hoProcessOptimization HoProcOptimization OPTIONAL,

seamlessHOSupported BOOLEAN OPTIONAL,

rangingIndicationDeadline INTEGER (0..255) OPTIONAL,

preassignedSTID STID OPTIONAL,

serviceLevelPrediction ServiceLevelPrediction OPTIONAL,

targetABSCarrier PhyCarrierIndex OPTIONAL,

channelBandwidth INTEGER (0..255) OPTIONAL,

cpLength CPLength OPTIONAL,

preassignedCarriers SEQUENCE (SIZE(0..8)) OF PreassignedCarrierInfo,

lzonePreambleIndex PreambleIndex3 OPTIONAL,

preallocatedBasicCID CID OPTIONAL,

sfhDeltaInfo SFHDeltaInfo OPTIONAL,

ssfhChangeCount INTEGER (0..15) OPTIONAL,

ssfhApplicationTime BOOLEAN OPTIONAL

},

...

}

ZoneSwitchCommand ::= SEQUENCE {

hoReentryMode ENUMERATED {

amsDiscFromServingMZone,

amsMaintainsCommunication

},

resourceRetainTime INTEGER (0..255) OPTIONAL,

faIndex FAIndex OPTIONAL,

actionTime INTEGER (0..255),

lzonePreambleIndex PreambleIndex3,

preallocatedBasicCID CID OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Neighbor Advertisement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-NBR-ADV ::= SEQUENCE {

changeCount INTEGER (0..7),

numberOfCellTypes INTEGER (0..7),

cellType INTEGER {

macro (0),

micro (1),

macroHotzone (2),

femto (3),

relay (4),

r1BZOrLZoneOfABS (5)

} (0..15),

numberOfNbrAdvSegments INTEGER (0..15),

nbrAdvSegmentIndex INTEGER (0..15),

startingAbsIndex AbsIndex,

nbrABSInfoList SEQUENCE (SIZE(0..255)) OF SEQUENCE {

absid ABSID,

macProtocolVersion MacProtocolVersion,

cpTime CPLength,

carrierInfoArray SEQUENCE (SIZE(1..7)) OF CarrierInfo,

sfhEncodingFormat SFHEncodingFormat,

sfhSubpacket1 SFHSubpacket1 OPTIONAL,

sfhSubpacket2 SFHSubpacket2 OPTIONAL,

sfhSubpacket3 SFHSubpacket3 OPTIONAL,

sfhDeltaInfo SFHDeltaInfo OPTIONAL,

nbrSpecificTrigger TriggerConditions OPTIONAL

},

r1NbrABSInfoList SEQUENCE (SIZE(0..255)) OF SEQUENCE {

nbrBSID BSID,

reuseFactor ENUMERATED {

reuseFactor1,

reuseFactor3

},

preambleIndex PreambleIndex3,

phyModeID PhyModeID,

carriers SEQUENCE (SIZE(0..64)) OF PhyCarrierIndex,

idCellRangeStart INTEGER (0..1023) OPTIONAL,

idCellRangeEnd INTEGER (0..1023) OPTIONAL,

nbrAdvOffset INTEGER (0..15) OPTIONAL,

nbrAdvInterval INTEGER (0..15) OPTIONAL,

servABSLZonePreambleIndex PreambleIndex3 OPTIONAL,

nbrBSID2 BSID OPTIONAL,

-- ??? duplicate fields?

reuseFactor2 ENUMERATED {

reuseFactor1,

reuseFactor3

},

preambleIndex2 PreambleIndex3,

channelBandwidth ENUMERATED {

fiveMHz,

sevenMHz,

eightDot75MHz,

tenMHz

} OPTIONAL,

r1BSCarrier PhyCarrierIndex OPTIONAL,

ldmParameterAI INTEGER (0..15) OPTIONAL,

ldmParameterUAI INTEGER (0..255) OPTIONAL,

ldmParameterSFO INTEGER (0..511) OPTIONAL

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Neighbor List Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-NBR-REQ ::= SEQUENCE {

lsbRequestedBSID INTEGER (0..4095),

requestBSType INTEGER {

csgFemtoABS (0),

osgFemtoABS (1)

} (0..3),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Scanning Interval Allocation Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SCN-REQ ::= SEQUENCE {

scanDuration INTEGER (0..255),

interleaving INTEGER (0..255),

scanIteration INTEGER (0..63),

recommendStartSuperFrame SuperframeNumberLSB OPTIONAL,

recommendedStartFrame StartFrameNumber OPTIONAL,

configChangeCount INTEGER (0..7) OPTIONAL,

recommendedNbrAbsIndexList SEQUENCE (SIZE(0..63)) OF AbsIndex,

nbrAdvSegmentIndex INTEGER(0..15) OPTIONAL,

nbrBitmapIndex BIT STRING (SIZE(8)) OPTIONAL,

recommendedFullNbrABSIDList SEQUENCE (SIZE(0..63)) OF BSID,

saPreambleIndexList SEQUENCE (SIZE(0..15)) OF PreambleIndex2,

nbrAbsList SEQUENCE (SIZE(0..63)) OF CarrierIndexArray,

servingAbscarrierIndexList SEQUENCE (SIZE(0..63)) OF PhyCarrierIndex,

csgidList SEQUENCE (SIZE(0..15)) OF SEQUENCE {

operatorID 	OperatorID OPTIONAL,

csgid		CSGID

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Scanning Interval Allocation Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SCN-RSP ::= SEQUENCE {

scanDuration INTEGER (0..255),

reportMode ReportMode,

reportMetric ReportMetric,

reportPeriod INTEGER (0..255),

durationOfSignalLocationMeas INTEGER (0..15) OPTIONAL,

startSuperFrame SuperframeNumberLSB OPTIONAL,

startFrame StartFrameNumber OPTIONAL,

interleavingInterval INTEGER (0..255) OPTIONAL,

scanIteration INTEGER (0..63) OPTIONAL,

configChangeCount INTEGER (0..7) OPTIONAL,

recommendedNbrAbsIndexList SEQUENCE (SIZE(0..63)) OF SEQUENCE {

nbrAbsIndex AbsIndex OPTIONAL,

recommendedCarrierIndexList SEQUENCE (SIZE(0..63)) OF PhyCarrierIndex OPTIONAL

},

nbrAdvSegmentIndex INTEGER (0..3) OPTIONAL,

nbrBitmapIndex BIT STRING (SIZE(8)) OPTIONAL,

-- ??? size?

requestedBitmapIndex BIT STRING (SIZE(8)) OPTIONAL,

recommendedNbrFullBsidList SEQUENCE (SIZE(0..63)) OF SEQUENCE {

fullNbrABSID ABSID,

recommendedCarrierIndexList SEQUENCE (SIZE(0..63)) OF PhyCarrierIndex,

saPreambleIndex PreambleIndex,

cpLength CPLength OPTIONAL

},

servingAbscarrierIndexList SEQUENCE (SIZE(0..63)) OF PhyCarrierIndex OPTIONAL,

scanningCarrier PhyCarrierIndex OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Scanning Result Report

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SCN-REP ::= SEQUENCE {

reportMode ReportMode2,

sABSCarrierInfoList SEQUENCE (SIZE(0..15)) OF SEQUENCE {

carrierIndex PhyCarrierIndex

},

--??? should the following fields be moved into the above sequence?

absCINRMean ABSCINRMean OPTIONAL,

absRSSIMean ABSRSSIMean OPTIONAL,

absRTD ABSRTD OPTIONAL,

configChangeCount INTEGER (0..7) OPTIONAL,

reportedNbrAbsIndexList SEQUENCE (SIZE(0..63)) OF AbsIndex,

nbrAdvSegmentIndex INTEGER (0..3) OPTIONAL,

nbrBitmapIndex BIT STRING (SIZE(8)) OPTIONAL,

-- ??? size?

reportedBitmapIndex BIT STRING (SIZE(8)) OPTIONAL,

reportedNbrFullBSIDArray SEQUENCE (SIZE(0..63)) OF BSID,

saPreambleIndexArray SEQUENCE (SIZE(0..15)) OF SEQUENCE {

saPreambleIndex PreambleIndex,

cinrMean ABSCINRMean OPTIONAL,

rssiMean ABSRSSIMean OPTIONAL,

relativeDelay RelativeDelay OPTIONAL

},

nbrAbsList SEQUENCE (SIZE(0..63)) OF CarrierIndexList OPTIONAL,

nbrRequestIndication BOOLEAN,

nbrRequestBSTypes BIT STRING {

csgClosedFemtoABS (0),

csgOpenFemtoABS (1),

osgFemtoABS (2)

} (SIZE(3)),

csgids SEQUENCE (SIZE(0..999)) OF CSGID,    --??? upper bound?

...

}

------------------------------------------------- End of proposed text --------------------------------------------------