| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal on Resource Index (Section 16.3.5.5) |  |
| Date Submitted | 2010-09-12 |  |
| Source(s) | Youngbo Cho, Hyunkyu Yu, Heewon Kang Samsung Electronics Co., Ltd. | youngbo.cho@samsung.com |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution proposes the text about resource index. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal on Resource Index (Section 16.3.5.5)

Youngbo Cho, Hyunkyu Yu, Heewon Kang

- Introduction

In D8[1], data burst is only allowed to be constructed within the same LRU type. However, when we consider the peak user throughput in some case such as DL:UL ratio is 6:2 or multicarrier mode, the number of data burst in a subframe should be minimized because an AMS can have maximum 16 HARQ channels. Thus, we recommend to allow the data construction from both NLRU and SLRU.

- References

[1] IEEE P802.16m/D8, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-08-25

- Rationale of modifications

In D8[1], HARQ channel limitation problem is occurred during operation for obtaining peak user throughput as follows:

- For 10MHz operation, the maximum number of resource units is limited to 42 using LRU and the maximum number of resource units is limited to 40 using SLRU.

- If RU types are different, separate burst allocations are required for SLRU and NLRU, respectively.

- ACID is also allocated separately.

For example, in DL:UL ratio = 6:2 case, 2 ACIDs are required in a sub-frame. Even in error free case, resource cannot be allocated in some sub-frames because there is no ACID to be allocated in some sub-frames as shown figure 1. Thus, we recommend to allow the data construction from both NLRU and SLRU to prevent this problem. In this case, the ABS requires 3 sub-frames after receiving feedback from the corresponding AMS to release ACID due to processing for decoding and releasing ACID.

In these sub-frames, resource cannot be allocated due to ACID limitation

Figure 1 ACID limitation problem in DL during operation for obtaining peak user throughput (DL:UL = 6:2)

- Proposed Text Changes

[Remedy-1: Change the following text, at line 59 in page 600, section 16.3.5.5.2.4.1, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

… The contiguous LRUs shall be constructed from the same LRU type i.e., DLRU, NLRU, or SLRU or both NLRU and SLRU.

-------------------------------------------------End proposed text---------------------------------------------------------------