| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Conversion of Network Entry/Reentry Functional Area MAC message tables to ASN.1 |  |
| Date Submitted | 2010-09-15 |  |
| Source(s) | Alessandro Triglia, OSS Nokalva | E-mail:sandro@oss.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to convert Network Entry/Reentry Functional MAC control messages into ASN.1. It is part of a coordinated effort to introduce up-to-date ASN.1 coding for all MAC messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Conversion of Network Entry/Reentry Functional Area MAC message tables to ASN.1

- Alessandro Triglia

- OSS Nokalva

- Introduction

This contribution is one of several contributions which are all part of a coordinated effort to include updated ASN.1 coding of all MAC control messages in P802.16m. The work is split along the “Functional Areas” defined in Table 678 of P802.16m/D8.

In this coordinated effort, any changes to the ASN.1 coded messages as contained in Annex R.2 of P802.16m/D8 are shown with markup. This allows the members of the workgroup to easily determine what changes are proposed from version D8 of the draft. These changes are intended to align the ASN.1 specification in Annex R.2 of P802.16m/D8 to the MAC message tables present in the same draft.

This contribution introduces ASN.1 coding for the MAC messages in the Network Entry/Reentry Functional Area (AAI-RNG-REQ, AAI-RNG-RSP, AAI-RNG-ACK, AAI-RNG-CFM, AAI-SBC-REQ, AAI-SBC-RSP, AAI-REG-REQ, and AAI-REG-RSP).

Any changes applied to the definition of AAI-RNG-REQ, AAI-RNG-RSP, AAI-RNG-ACK, AAI-RNG-CFM, AAI-SBC-REQ, AAI-SBC-RSP, AAI-REG-REQ, and AAI-REG-RSP during the meeting can be also applied to the ASN.1 coded messages below, allowing the group to see the changes and approve those changes. Thus at the conclusion of Session #69 these messages will be fully up-to-date.

- Proposed text

Annex R.2 MAC Control Message Definitions

[Editor Note: update the text in Annex R.2 from P927 L54 to P938 L41 with the following marked changes]

------------------------------------------------- Start of proposed text --------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Network entry / re-entry messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

MACAddress ::= BIT STRING (SIZE(48))

MACVersion ::= INTEGER (0..255)

CRID ::= BIT STRING (SIZE(72))

DID ::= BIT STRING (SIZE(12))

AKCount ::= INTEGER (0..65535)

OperatorID ::= OCTET STRING (SIZE(3))

CSGID ::= BIT STRING (SIZE(1..24))

MulticastGroupID ::= INTEGER (0..4095)

SMS ::= OCTET STRING (SIZE(1..140))

SFID ::= BIT STRING (SIZE(32))

CPMKSN ::= INTEGER (0..15)

CMACPN ::= BIT STRING (SIZE(24))

CMACValue ::= BIT STRING (SIZE(64))

EMBSID ::= INTEGER(0..4095)

Supported ::= BOOLEAN

IPv4Address ::= OCTET STRING (SIZE(4))

IPv6Address ::= OCTET STRING (SIZE(16))

IPPortNumber ::= INTEGER (0..65535)

MACAddrMask ::= MACAddress

SfNumberLSB ::= INTEGER (0..63)

CapabilityIndex ::= INTEGER (0..31)

DeviceClass ::= INTEGER (0..31)

FeatureSupport ::= ENUMERATED {notSupported, supported}

IPv6HomeNetworkPrefix ::= BIT STRING (SIZE(64))

RangingPurpose ::= INTEGER {

networkEntry (0),

hOReentry (1),

reentryFromIdleModeToABS (2),

idleModeLocationUpdate (3),

dcrModeExtension (4),

emergencyCallSetup (5),

locationUpdateEMBSFlows (6),

locationUpdateTransitionToDCRMode (7),

reentryFromDcrModeOrCoverLossOrBadRestart (8),

reentryFromLegacyBS (9),

zoneSwitchToMzone (10),

locationUpdatePowerDown (11),

experiencingFemtoInterference (12),

nsepCallSetup (13)

} (0..15)

-- 0 no ranging retry

-- 1..65535 in units of second

RangingAbortTimer ::= INTEGER (0..65535)

FidInfo ::= SEQUENCE {

flowIdentifier FID,

fidChangeCount FidChangeCount

}

CMAC ::= SEQUENCE {

pmkSN CPMKSN,

cmacPN CMACPN,

cmacValue CMACValue

}

LocUpdateRsp ::= INTEGER {

success (0),

failure (1),

dlTrafficPending (2),

allowDcrModeInitOrExt (3),

rejectDcrModeInitOrExt (4)

} (0..15)

HoProcOptimization ::= BIT STRING {

omitSbcMessages (0),

omitPkmAuthenticationPhase (1),

omitRegMessages (2),

omitFullStateTransfer (3)

} (SIZE(4))

EmbsZoneIdUpdate ::= SEQUENCE {

embsZoneID EMBSZoneID OPTIONAL,

newEMBSZoneID EMBSZoneID OPTIONAL,

physicalCarrierIndex PhyCarrierIndex OPTIONAL,

serviceFlowUpdateType CHOICE {

bitmapAndNewServiceFlow SEQUENCE {

serviceFlowUpdateBitmap BIT STRING (SIZE(16)),

embsIdAndFIDList SEQUENCE (SIZE(1..16)) OF SEQUENCE {

newEMBSid EMBSID,

newFID FID

}

},

currentAndNewServiceFlow SEQUENCE (SIZE(1..16)) OF SEQUENCE {

currentEMBSid EMBSID,

currentFID FID,

newEMBSid EMBSID,

newFID FID

}

}

}

ShortFrameId ::= SEQUENCE {

superframe INTEGER (0..3),

frameIndex INTEGER (0..3)

}

RangingStatus ::= ENUMERATED {

success,

abort,

continue

}

RangingRsp ::= SEQUENCE {

rngPreambleCodeIndex INTEGER (0..63) OPTIONAL,

rngStatus RangingStatus OPTIONAL,

timingOffsetAdjustment INTEGER (0..32767) OPTIONAL,

powerLevelAdustment INTEGER (0..15) OPTIONAL,

freqOffsetAdjustment INTEGER (0..511) OPTIONAL

}

ImCapabilities ::= BIT STRING {    -- 1: supported

dlPMICoordination (0),

dlCollaborativeMBSMIMO (1),

dlClosedLoopMbsMacroDiversity (2),

ulPmiCombination (3),

multiBsSoundingCalibration (4)

} (SIZE(5))

LbsCapabilities ::= SEQUENCE {

agpsMethod FeatureSupport

}

EmbsCapabilities ::= BIT STRING {

servingAbsOnly (0),

macroDiversityMultiAbs (1),

nonMacroDiversityMultiAbs (2)

} (SIZE(3))

McCapabilities ::= ENUMERATED {

noMcModes,

basicMcMode,

mcAggregation,

mcSwitching,

mcAggregationAndSwitching

}

AmsCapabilities ::= SEQUENCE {

arqParameters SEQUENCE {

arqSNModulus INTEGER (0..1023) OPTIONAL,

arqWindowSize INTEGER (0..31) OPTIONAL,

arqSubBlockSize INTEGER (0..1023) OPTIONAL,

arqBlockLifetime INTEGER (0..65535) OPTIONAL,

arqRxPurgeTimeout INTEGER (0..65535) OPTIONAL,

arqMaxBufferSize INTEGER (0..65535) OPTIONAL

} OPTIONAL,

maxNonARQBufferSize INTEGER (0..8388607) OPTIONAL,

multicarrierCapabilities McCapabilities OPTIONAL,

zoneSwitchingMode FeatureSupport OPTIONAL,

lbsCapabilities LbsCapabilities OPTIONAL,

imCapabilities ImCapabilities OPTIONAL,

embsCapabilities EmbsCapabilities OPTIONAL,

channelBwAndCyclicPrefix BIT STRING {

fiveMHz1Over16 (0),

fiveMHz1Over8 (1),

fiveMHz1Over4 (2),

tenMHz1Over16 (3),

tenMHz1Over8 (4),

tenMHz1Over4 (5),

twentyMHz1Over16 (6),

twentyMHz1Over8 (7),

twentyMHz1Over4 (8),

eightDotSevenFiveMHz1Over16 (9),

eightDotSevenFiveMHz1Over8 (10),

eightDotSevenFive5MHz1Over4 (11),

sevenMHz1Over16 (12),

sevenMHz1Over8 (13),

sevenMHz1Over4 (14)

} (SIZE(15)) OPTIONAL,

frameConfig5MHz FeatureSupport OPTIONAL,

frameConfigLegacy10MHz FeatureSupport OPTIONAL,

persistentAllocation FeatureSupport OPTIONAL,

groupResourceAllocation FeatureSupport OPTIONAL,

coLocatedCoexistence BIT STRING {

typeI (0),

typeII-1 (1),

typeII-2 (2),

typeII-3 (3),

typeIII (4)

} (SIZE(5)) OPTIONAL,

hoTriggerMetric ReportMetric OPTIONAL,

handoverEbbMode SEQUENCE {

ebb FeatureSupport OPTIONAL,

minHoRentryIntlvInterval INTEGER (0..3) OPTIONAL

} OPTIONAL,

soundingAntSwitchCapability SEQUENCE {

soundingAntSwitching FeatureSupport OPTIONAL,

antennaConfig ENUMERATED {

amongDLRx,

amongULTx

} OPTIONAL

}

}

ReqHostConfig ::= OCTET STRING (SIZE(0..999))   --??? upper bound?

CsCapabilities ::= SEQUENCE {

csType BIT STRING {

packet-ipv4 (1),

packet-ipv6 (2),

packet (14),

multiprotocolFlow (15)

} (SIZE(32)) OPTIONAL,

-- ??? the draft says "16 or 32"!

maxNoOfClassificationRules INTEGER (0..65535) OPTIONAL,

rohc FeatureSupport OPTIONAL,

phs INTEGER {

packetPhs (1)

} (0..1) OPTIONAL,

-- may only be present AAI-REG-RSP

resourceRetainTime INTEGER (0..65535) OPTIONAL,

ipServiceType INTEGER {

ipv4 (0),

ipv6 (1),

ipv4IPv6DualMode (2)

} (0..3) OPTIONAL

}

ClcLimits ::= SEQUENCE { -- Table 688

typeIIndicator BOOLEAN,

typeIIIndicator BOOLEAN,

-- ??? needs clarification

activeClassLimit INTEGER (1..8),

activeRatioLimit INTEGER (0..63),

activeIntervalLimit INTEGER (0..31)

}

maxRcvdCodes INTEGER ::= 64

maxRngOpps INTEGER ::= 4

RangingCodeIndex ::= INTEGER(0..63)

UnicastRngAck ::= RngBehavior

RangingOppStatus ::= SEQUENCE {

receivedCodesList SEQUENCE (SIZE(1..maxRcvdCodes)) OF RcvdRngCodeInfo

}

RcvdRngCodeInfo ::= SEQUENCE {

rngCodeIndex RangingCodeIndex,

dedicatedRngCodeIndicator BOOLEAN,

rngStatus RngBehavior

}

BroadcastRngAck ::= SEQUENCE {

frameIdentifier ShortFrameId,

rngAckBitmap BIT STRING {

rngOpp1(0),

rngOpp2(1),

rngOpp3(2),

rngOpp4(3)

},

rngOppsList SEQUENCE (SIZE (1..maxRngOpps))	OF RangingOppStatus OPTIONAL

}

RngBehavior ::= CHOICE {

rngSuccess AdjustmentParameters,

rngAbort RangingAbortTimer,

rngContinue AdjustmentParameters

}

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment INTEGER (0..31) OPTIONAL,

powerLevelAdjustment INTEGER (0..7) OPTIONAL,

frequencyOffsetAdjustment INTEGER (0..511) OPTIONAL

}

InterRatOpMode ::= INTEGER {

singleRadioMode (0),

multiRadioMode (1)

} (0..3)

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Ranging Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-RNG-REQ ::= SEQUENCE {

amsidHashValue MACAddress OPTIONAL,

macVersion MACVersion OPTIONAL,

macAddress MACAddress OPTIONAL,

rangingPurpose RangingPurpose,

servingBsid BSID OPTIONAL,

crid CRID OPTIONAL,

previousSTID STID OPTIONAL,

previousBasicCID CID OPTIONAL,

pagingControllerID PCID OPTIONAL,

deregistrationID DID OPTIONAL,

pagingGroupID PGID OPTIONAL,

pagingCycle PgCycle OPTIONAL,

pagingOffset PgOffset OPTIONAL,

pagingCycleChange PgCycle OPTIONAL,

pagingCarrierUpdate INTEGER(0..15) OPTIONAL,

akCount AKCount OPTIONAL,

amsMobility ENUMERATED {

slow,

medium,

fast

} OPTIONAL,

csgIdList SEQUENCE (SIZE(0..15)) OF SEQUENCE {

operatorID 	OperatorID OPTIONAL,

csgid		CSGID

} OPTIONAL,

fidList SEQUENCE (SIZE(0..15)) OF FidInfo OPTIONAL,

smsMessage SMS OPTIONAL,

cmac CMAC OPTIONAL,

-- The bit size represents power level ranging from -15dB (0x00) to

-- 26dB (0x1F)

-- The value is determined by AMS after successful initial ranging

-- process.

initialOffsetUlpc INTEGER(0..31) OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Ranging Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-RNG-RSP ::= SEQUENCE {

rangingAbort BOOLEAN OPTIONAL,

rangingAbortTimer RangingAbortTimer OPTIONAL,

temporaryStid STID OPTIONAL,

amsId MACAddress OPTIONAL,

stid STID OPTIONAL,

crid CRID OPTIONAL,

emergencyServiceFid FID OPTIONAL,

nsEpServiceFid FID OPTIONAL,

locationUpdateResp LocUpdateRsp OPTIONAL,

newPagingCycle PgCycle OPTIONAL,

newPagingGroupID PGID OPTIONAL,

newPagingOffset PgOffset OPTIONAL,

deregistrationID DID OPTIONAL,

newPagingControllerID PCID OPTIONAL,

reentryProcOptimization HoProcOptimization OPTIONAL,

activationDeadline SfNumberLSB OPTIONAL,

nbrMeasurementRptInd ENUMERATED {

doNotPerform,

perform

} OPTIONAL,

resourceRetainTime INTEGER (0..256) OPTIONAL,

fidUpdate SEQUENCE {

sfid SFID OPTIONAL,

updateOrDelete ENUMERATED {

deleteFlow,

updateQoS

} OPTIONAL,

updatedQoSInfo SEQUENCE { } OPTIONAL   -- ??? no definition

},

unsolicitedBwGrantInd ENUMERATED {

doNotAllocate,

allocate

} OPTIONAL,

clcResponse ClcResponse OPTIONAL,

csgid CSGID OPTIONAL,

redirectionInfoList SEQUENCE (SIZE(0..8)) OF RedirectionInfo OPTIONAL,

smsMessage SMS OPTIONAL,

rangingRequest ENUMERATED {

notSendAaiRngCfm,

sendAaiRngCfm

} OPTIONAL,

invalidFids SEQUENCE (SIZE(0..16)) OF FID,

saidUpdateBitmap BIT STRING (SIZE(0..16)) OPTIONAL,

embsZoneIDUpdateList SEQUENCE (SIZE(0..8)) OF EmbsZoneIdUpdate OPTIONAL,

multicastGroupIDUpdateList SEQUENCE (SIZE(0..15)) OF SEQUENCE {

currentMulticastGroupID 	MulticastGroupID,

currentFID 			FID,

newMulticastGroupID		MulticastGroupID,

newFID				FID

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Ranging Acknowledge

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-RNG-ACK ::= SEQUENCE {

rangingAcknowledge CHOICE {

broadcastRngAck BroadcastRngAck,

unicastRngAck UnicastRngAck

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Ranging Confirmation

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-RNG-CFM ::= SEQUENCE {

amsStid STID,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Basic Capability Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SBC-REQ ::= SEQUENCE {

capabilityIndex CapabilityIndex,

deviceClass DeviceClass,

clcRequest ClcRequest OPTIONAL,

ulLongTTI FeatureSupport OPTIONAL,

dlLongTTI FeatureSupport OPTIONAL,

ulSounding FeatureSupport OPTIONAL,

dlFfrResourceMetric FeatureSupport OPTIONAL,

dlMaxNumberOfSuMimoStreams INTEGER (1..8) OPTIONAL,

dlMaxNumberOfMuMimoStreams INTEGER (1..2) OPTIONAL,

ulMaxNumberOfSuMimoStreams INTEGER (1..4) OPTIONAL,

ulMaxNumberOfMuMimoStreams INTEGER (1..3) OPTIONAL,

dlMIMOModes BIT STRING { -- 1: supported

mode0 (0), -- 0: not supported

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4),

mode5 (5)

} (SIZE(6)) OPTIONAL,

ulMIMOModes BIT STRING { -- 1: supported

mode0 (0), -- 0: not supported

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4)

} (SIZE(5)) OPTIONAL,

dlFeedback BIT STRING { -- 1: supported

differentialMode (0),

mimoFeedbackMode0 (1),

mimoFeedbackMode1 (2),

mimoFeedbackMode2 (3),

mimoFeedbackMode3 (4),

mimoFeedbackMode4 (5),

mimoFeedbackMode5 (6),

mimoFeedbackMode6 (7),

mimoFeedbackMode7 (8)

} (SIZE(9)) OPTIONAL,

numberOfTxAntennas ENUMERATED {

oneAntenna,

twoAntenna,

fourAntenna

} OPTIONAL,

modulationSchemes BIT STRING {

dl64QAM (0),

ul64QAM (1)

} (SIZE(2)) OPTIONAL,

ulHARQBufferingCapability INTEGER (1..128) OPTIONAL,

dlHARQBufferingCapability INTEGER (1..128) OPTIONAL,

amsDlProcessingCapability INTEGER (1..128) OPTIONAL,

amsUlProcessingCapability INTEGER (1..128) OPTIONAL,

fftSizes BIT STRING {

fft2048 (0),

fft1024 (1),

fft512 (2)

} (SIZE(3)) OPTIONAL,

authorizationPolicy ENUMERATED {

noAuthorization,

eapBasedAuthorization

} OPTIONAL,

interRatOperationMode InterRatOpMode OPTIONAL,

interRatTypesSupport BIT STRING {

ieee802-11 (0),

geran (1),

utran (2),

e-utran (3),

cdma2000(4)

} (SIZE(5)) OPTIONAL,

mihCapabilitySupported FeatureSupport OPTIONAL,

maxTxPower SEQUENCE {

maxTxPowerForQPSK INTEGER(0..127),

maxTxPowerFor16QAM INTEGER(0..127),

maxTxPowerFor64QAM INTEGER(0..127)

} OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Basic Capability Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SBC-RSP ::= SEQUENCE {

capabilityIndex CapabilityIndex,

deviceClass DeviceClass,

clcResponse ClcResponse OPTIONAL,

ulLongTTI FeatureSupport OPTIONAL,

dlLongTTI FeatureSupport OPTIONAL,

ulSounding FeatureSupport OPTIONAL,

dlFfrResourceMetric FeatureSupport OPTIONAL,

dlMaxNumberOfSuMimoStreams INTEGER (1..8) OPTIONAL,

dlMaxNumberOfMuMimoStreams INTEGER (1..2) OPTIONAL,

ulMaxNumberOfSuMimoStreams INTEGER (1..4) OPTIONAL,

ulMaxNumberOfMuMimoStreams INTEGER (1..3) OPTIONAL,

dlMIMOModes BIT STRING { -- 1: supported

mode0 (0), -- 0: not supported

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4),

mode5 (5)

} (SIZE(6)) OPTIONAL,

ulMIMOModes BIT STRING { -- 1: supported

mode0 (0), -- 0: not supported

mode1 (1),

mode2 (2),

mode3 (3),

mode4 (4)

} (SIZE(5)) OPTIONAL,

dlFeedback BIT STRING { -- 1: supported

differentialMode (0),

mimoFeedbackMode0 (1),

mimoFeedbackMode1 (2),

mimoFeedbackMode2 (3),

mimoFeedbackMode3 (4),

mimoFeedbackMode4 (5),

mimoFeedbackMode5 (6),

mimoFeedbackMode6 (7),

mimoFeedbackMode7 (8)

} (SIZE(9)) OPTIONAL,

numberOfTxAntennas ENUMERATED {

oneAntenna,

twoAntenna,

fourAntenna

} OPTIONAL,

modulationSchemes BIT STRING {

dl64QAM (0),

ul64QAM (1)

} (SIZE(2)) OPTIONAL,

ulHARQBufferingCapability INTEGER (1..128) OPTIONAL,

dlHARQBufferingCapability INTEGER (1..128) OPTIONAL,

amsDlProcessingCapability INTEGER (1..128) OPTIONAL,

amsUlProcessingCapability INTEGER (1..128) OPTIONAL,

fftSizes BIT STRING {

fft2048 (0),

fft1024 (1),

fft512 (2)

} (SIZE(3)) OPTIONAL,

authorizationPolicy ENUMERATED {

noAuthorization,

eapBasedAuthorization

} OPTIONAL,

interRatOperationMode InterRatOpMode OPTIONAL,

interRatTypesSupport BIT STRING {

ieee802-11 (0),

geran (1),

utran (2),

e-utran (3),

cdma2000(4)

} (SIZE(5)) OPTIONAL,

mihCapabilitySupported FeatureSupport OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Registration Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-REG-REQ ::= SEQUENCE {

amsMacAddress MACAddress,

amsCapNegotiation AmsCapabilities OPTIONAL,

csCapabilities CsCapabilities OPTIONAL,

hostConfigCapability FeatureSupport OPTIONAL,

requestedHostConfig ReqHostConfig OPTIONAL,

globalCarrierConfigChangeCount INTEGER (0..7) OPTIONAL,

amsInitAgpServiceAdaptation FeatureSupport OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Registration Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-REG-RSP ::= SEQUENCE {

stid STID,

crid CRID OPTIONAL,

arqParameters SEQUENCE {

arqSNModulus INTEGER (0..1023) OPTIONAL,

arqWindowSize INTEGER (0..31) OPTIONAL,

arqSubBlockSize INTEGER (0..1023) OPTIONAL,

arqBlockLifetime INTEGER (0..65535) OPTIONAL,

arqRxPurgeTimeout INTEGER (0..65535) OPTIONAL

} OPTIONAL,

femtoAbsLdm SEQUENCE {

startSuperframeOffset INTEGER (0..511) OPTIONAL,

availableIntervalLeng INTEGER (0..15) OPTIONAL,

unavailableIntervalLeng INTEGER (0..15) OPTIONAL

},

lbsCapabilities LbsCapabilities OPTIONAL,

imCapabilities ImCapabilities OPTIONAL,

antennaConfigSounfingSw ENUMERATED {

amongDLRx,

amongULTx

} OPTIONAL,

embsCapabilities EmbsCapabilities OPTIONAL,

persistentAllocation FeatureSupport OPTIONAL,

groupResourceAllocation FeatureSupport OPTIONAL,

hoTriggerMetric ReportMetric OPTIONAL,

csCapabilities CsCapabilities OPTIONAL,

ipv4HostAddress IPv4Address OPTIONAL,

ipv6HomeNetworkPrefix IPv6HomeNetworkPrefix OPTIONAL,

additionalHostConfigIe OCTET STRING (SIZE(0..999)) OPTIONAL,

redirectionInfoArray SEQUENCE (SIZE(1..8)) OF RedirectionInfo OPTIONAL,

csgidLength INTEGER (1..24) OPTIONAL,

globalCarrierConfigChangeCount INTEGER (0..7) OPTIONAL,

multicarrierCapabilities McCapabilities OPTIONAL,

-- ??? need CS type definition

csTypeOfDefaultServiceFlow  CsSpecification OPTIONAL,

clcLimits ClcLimits OPTIONAL,

amsInitAgpServiceAdaptation FeatureSupport OPTIONAL,

...

}

------------------------------------------------- End of proposed text --------------------------------------------------