| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Conversion of Sleep Mode Functional Area MAC message tables to ASN.1 |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Alessandro Triglia, OSS Nokalva | E-mail:sandro@oss.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to convert Sleep Mode Functional MAC control messages into ASN.1. It is part of a coordinated effort to introduce up-to-date ASN.1 coding for all MAC messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Conversion of Sleep Mode Functional Area MAC message tables to ASN.1

- Alessandro Triglia

- OSS Nokalva

- Introduction

This contribution is one of several contributions which are all part of a coordinated effort to include updated ASN.1 coding of all MAC control messages in P802.16m. The work is split along the “Functional Areas” defined in Table 678 of P802.16m/D8.

In this coordinated effort, any changes to the ASN.1 coded messages as contained in Annex R.2 of P802.16m/D8 are shown with markup. This allows the members of the workgroup to easily determine what changes are proposed from version D8 of the draft. These changes are intended to align the ASN.1 specification in Annex R.2 of P802.16m/D8 to the MAC message tables present in the same draft.

This contribution introduces ASN.1 coding for the MAC messages in the Sleep Mode Functional Area (AAI-SLP-REQ, AAI-SLP-RSP, AAI-TRF-IND, AAI-TRF-IND-REQ, and AAI-TRF-IND-RSP).

Any changes applied to the definition of AAI-SLP-REQ, AAI-SLP-RSP, AAI-TRF-IND, AAI-TRF-IND-REQ, and AAI-TRF-IND-RSP during the meeting can be also applied to the ASN.1 coded messages below, allowing the group to see the changes and approve those changes. Thus at the conclusion of Session #69 these messages will be fully up-to-date.

- Proposed text

Annex R.2 MAC Control Message Definitions

[Editor Note: update the text in Annex R.2 from P952 L18 to P955 L20 with the following marked changes]

------------------------------------------------- Start of proposed text --------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Sleep Mode Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

SCID ::= INTEGER (0..15)

TrafficIndicationFlag ::= ENUMERATED {

doNotSend,

send

}

ListeningWindowExtFlag ::= ENUMERATED {

fixed,

extensible

}

NextSleepCycleIndicator ::= INTEGER {

resetToInitialSleepCycle (0),

doubleOrFinal (1),

resetToAnotherValue (2)

} (0..3)

EnterOrChangeSleepMode ::= SEQUENCE {

sleepCycleID SCID,

startFrameNumber INTEGER (0..63),

trafficIndicationFlag TrafficIndicationFlag,

listeningWindowExtFlag ListeningWindowExtFlag,

nextSleepCycleIndicator NextSleepCycleIndicator,

initialSleepCycle INTEGER (0..15),

finalSleepCycle INTEGER (0..1023),

listeningWindow INTEGER (1..64),

listeningSubframeBitmap BIT STRING (SIZE(8)),

newInitialSleepCycle INTEGER (0..31) OPTIONAL,

tAMS INTEGER (0..31) OPTIONAL

}

SleepOperation ::= ENUMERATED {

exitSleepMode,

enterSleepMode,

changeSleepCycleSetting,

switchSleepCycleSetting

}

SLPID ::= INTEGER (0..1023)

FfbchOperation ::= ENUMERATED {

ffbchKept,

ffbchDeallocatedAtFrame,

ffbchDeallocatedAtBeginningOfSleepWindow,

...

}

SleepResponseInfo ::= SEQUENCE {

trafficIndicationFlag TrafficIndicationFlag,

listeningWindowExtFlag ListeningWindowExtFlag,

nextSleepCycleIndicator NextSleepCycleIndicator,

initialSleepCycle INTEGER (0..15),

finalSleepCycle INTEGER (0..1023),

listeningWindow INTEGER (1..64),

listeningSubframeBitmap BIT STRING (SIZE(8))

}

ReqUnsolicitedOrApproved ::= SEQUENCE {

operation SleepOperation OPTIONAL,

sleepRspOperation CHOICE {

enterSleepMode SEQUENCE {

sleepCycleID SCID,

ffbchOperation FfbchOperation,

startFrameNumber INTEGER (0..63),

sleepResponseInfo SleepResponseInfo

},

changeSleepCycle SEQUENCE {

sleepCycleID SCID,

ffbchOperation FfbchOperation,

startFrameNumber INTEGER (0..63),

sleepResponseInfo SleepResponseInfo,

slpid SLPID OPTIONAL,

newInitialSleepCycle INTEGER (0..31) OPTIONAL,

tAMS INTEGER (0..31) OPTIONAL

},

switchSleepCycle SEQUENCE {

sleepCycleID SCID,

ffbchOperation FfbchOperation

}

}

}

SlpidBitmapBasedTrfIndInfo ::= SEQUENCE {

--the bitmap has one bit for each SLPID group

slpidGroupIndicationBitmap BIT STRING (SIZE(32)),

-- the array has one bitmap for each SLPID group whose bit in

-- slpidGroupIndicationBitmap is set to 1

-- each bitmap has one bit for each SLPID in the group

trafficIndicationBitmapArray SEQUENCE (SIZE(0..32)) OF BIT STRING (SIZE(32))

}

SlpidBasedTrfIndInfo ::= SEQUENCE {

slpidArray SEQUENCE (SIZE(0..63)) OF SLPID,

slpidUpdateArray SEQUENCE (SIZE(0..1024)) OF SEQUENCE {

oldSlpid SLPID,

newSlpid SLPID

}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Sleep Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SLP-REQ ::= SEQUENCE {

operation SleepOperation,

sleepReqOperationInfo CHOICE {

enterSleepMode EnterOrChangeSleepMode, -- Op = 1

changeSleepMode EnterOrChangeSleepMode, -- Op = 2

switchSleeCycle SEQUENCE { -- Op = 3

sleepCycleID SCID,

startFrameNumber INTEGER (0..63)

}

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Sleep Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SLP-RSP ::= SEQUENCE {

responseCode ENUMERATED {

unsolicitedReqByAbs,

requestApproval,

requestRejection

},

sleepResponseInfo CHOICE {

unsolicitRequest ReqUnsolicitedOrApproved,

sleepReqApproval ReqUnsolicitedOrApproved,

sleepReqreject SEQUENCE {

requestDuration INTEGER (0..255)

},

...

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Traffic Indication

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-TRF-IND ::= SEQUENCE {

trafficIndication CHOICE {

slpidBitmapBased SlpidBitmapBasedTrfIndInfo,

slpidBased SlpidBasedTrfIndInfo

} OPTIONAL,

emergencyAlert BOOLEAN,

...

-- ??? the text mentions a "SLPIDUpdate"

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Traffic Indication Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-TRF-IND-REQ ::= SEQUENCE {

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Traffic Indication Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-TRF-IND-RSP ::= SEQUENCE {

frameNumber INTEGER (0..1023),

sleepCycleLength INTEGER (0..1023),

...

}

------------------------------------------------- End of proposed text --------------------------------------------------