| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-DL-IM |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Taeyoung Kim, Hyunjeong Kang Samsung Electronics | ty33.kim@samsung.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clarification on the tables for AAI-DL-IM MAC messages |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-DL-IM

Taeyoung Kim, Hyunjeong Kang

Samsung Electronics

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for AAI-DL-IM MAC control messages.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 181, line 44, replace Table 715 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 715—Parameters for AAI-DL-IM message

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (ABS needs to change FP2 power){ |  |  |  |
| fp2Power | 5 | FP2_power[dB] = 0b00000: -Inf Otherwise: -10+(fp2Power-1)0.5 |  |
| } |  |  |  |
| If (ABS needs to change FP3 power){ |  |  |  |
| fp3Power | 5 | FP3_power[dB] = 0b00000: -Inf Otherwise: -10+(fp3Power-1) 0.5 |  |
| } |  |  |  |
| If (Nt == 2){ |  |  |  |
| twoTxBCSI | 8 | BC_SI in case of two transmit antenna, refer 16.3.6.2.5.5 If LSB#n is '1', the n-th codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. |  |
| } |  |  |  |
| If ((Nt == 4) and (Nt==8)){ |  |  |  |
| otherTxBCSI | 16 | BC_SI in case of four or eight transmit antenna, refer 16.3.6.2.5.5 If LSB#n is '1', the n-th codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. |  |
| } |  |  |  |
| If (multiBS MIMO is operated){ |  |  |  |
| nipTh1 | 4 | NIP_th_1 The value shall be interpreted as unsigned 4bits with units of 0.5 dB, 0b0000 is interpreted as -7.5dB and 0b1111 is interpreted as 0.0dB NIP threshold for Single BS precoding with Multi-BS Coordination trigger |  |
| nipTh2 | 4 | NIP_th_2 The value shall be interpreted as unsigned 4bits with units of 0.5 dB, 0b0000 is interpreted as -7.5dB and 0b1111 is interpreted as 0.0dB Sum NIP threshold for DL Multi-BS Joint MIMO Processing trigger |  |
| cinrTh | 4 | CINR_th The value shall be interpreted as signed 4bits with units of 0.5 dB, 0b1111 is interpreted as -4.0dB and 0b0111 is interpreted as 3.5dB Used together with NIP_th_1 or NIP_th_2 for Multi-BS MIMO trigger |  |
| diversitySetNum | 4 | The number of ABSs coordinating with the S-ABS for single BS precoding with multi-BS coordination |  |
| If (diversitySetNum > 0){ |  |  |  |
| changeCount | 8 | Change count in AAI-NBR-ADV for the following BS_index. |  |
| for (i=1; i<=diversitySetNum; i++){ |  |  |  |
| BS_index | 8 | Indicates diversity set members. Refers to the ABS index of the AAI-NBR_ADV. Temp_BSID is derived from this BS_index. The Temp_BSID is the order of the ABSs in respect of the BS_index. Temp_BSID = 0 represents the ABS that is referred by the first BS_index in AAI-DL_IM message. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------