| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-MultiBS_MIMO_FBK |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Taeyoung Kim, Hyunjeong Kang Samsung Electronics | ty33.kim@samsung.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clarification on the tables for AAI-MultiBS_MIMO_FBK MAC messages |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-MultiBS_MIMO_FBK

Taeyoung Kim, Hyunjeong Kang

Samsung Electronics

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for AAI-MultiBS_MIMO_FBK  MAC control messages.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 188, line 28, replace Table 718 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 718—AAI-MultiBS_MIMO_FBK message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (ICT==0b10){ |  | ICT indicated in Feedback Polling A-MAP IE |  |
| cqi | 4 | CQI for Co-MD |  |
| } |  |  |  |
| If (ICT==0b11){ |  |  |  |
| cqi | 4 | CQI for CL-MIMO |  |
| maxAmpBs | 1 | 0b0 : S-ABS has the maximum channel amplitude among collaborative ABS 0b1 : S-ABS does not have the maximum channel amplitude among collaborative ABS |  |
| If (maxAmpBsindex==0b1){ |  |  |  |
| maxAmpBsindex | 3 | ABS index in adjAbsBitmapMultiBSMIMO in AAI-MULTI_BS_MIMORSP. 0b000 represents the ABS indicated by the first bit, and 0b111 represents the ABS indicated by the last bit. This index indicates the ABS with largest channel amplitude |  |
| relativeServingBs | 3 | Relative value of maximum amplitude ABS's amplitude compared to S-ABS', which is representing 1dB(0b000) ~ 8dB(0b111) with 1dB step |  |
| } |  |  |  |
| } |  |  |  |
| for (i=1; i<= N_multiBS_reports; i++){ |  | N_multiBS_reports indicated in Feedback Polling A-MAP IE Range: 1-8 |  |
| pmi | 3-4 | PMI from the rank-1 base codebook or base codebook subset Nt=2: 3 bits Nt =4: 4 bits Nt =8: 4 bits |  |
| If (ICT==0b00 or ICT==0b01){ |  |  |  |
| tempBsid | 4 | Diversity set member ID refer 16.2.3.35 |  |
| measurementMetric | 2 | SINR gain assuming the reported PMI set is coordinated. This can be used for resolving conflict from multiple AMS. 0b00: 0.25dB 0b01: 0.50dB 0b10: 1.00dB 0b11: above 1.50dB |  |
| pmiSubsetSize | 1 | Indication whether 1 PMI or a PMI set is feedback 0b0: 1 PMI 0b1: multiple PMIs |  |
| If (pmiSubsetSize==0b1){ |  |  |  |
| pmiCoordinationSubset | 1 | Set of PMIs from the rank-1 base codebook or base codebook subset 0b0: correlation level n1 (as defined in section 16.5.1.2.1) 0b1: correlation level n2 (as defined in section 16.5.1.2.1) |  |
| } |  |  |  |
| } |  |  |  |
| If (ICT==0b10){ |  |  |  |
| cpmi | 3 | Concatenating PMI for neighboring cells. |  |
| } |  |  |  |
| If (ICT==0b11){ |  |  |  |
| cpmi | 3 | Concatenating PMI for neighboring cells. |  |
| RelativeAmp | 4 | Uniformly quantized relative channel amplitude (Normalized by the maximum channel amplitude among collaborative ABS) 0dB (0b0000) to -15dB(0b1111) with -1dB step |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------