| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on the E-MBS Operation on Multicarrier Deployment in IEEE P802.16m (16.9.2.4 & 16.2.3.47) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D8,” in response to the IEEE 802.16 Working Group Sponsor Ballot Recirc #2 |  |
| Abstract | E-MBS operation to support carrier switch mode on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comments on the E-MBS Operation on Multicarrier Deployment in IEEE P802.16m (16.9.2.4 & 16.2.3.47)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS) operation on the multicarrier deployment using unicast available interval bitmap. The Unicast Available Interval bitmap is transmitted in the AAI_DSA-REQ/RSP to allocate, reallocate and update. To update without adding new service flow, the AAI_DSC transaction may be proper way and the update that is Unicast Available Interval bitmap can be added along with the service flow update described in the AAI_DSC. In addition, to reallocate or release the Unicast Available Interval bitmap along with delete the E-MBS contents in AAI_DSD can alleviate the signaling overhead.

# References

- IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” July 2010

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Adapt the following change in 16.9.2.4 of 802.16m/D8]

16.9.2.4 E-MBS Operation on the multi-carrier deployment

When E-MBS data, including E-MBS configuration message, E-MBS MAP, and E-MBS contents, is transmitted on an alternative carrier, i.e. other than the AMS's primary carrier where service flows are configured the AMS is redirected to the carrier carrying E-MBS data through DSA as described in 16.9.2.1.

The AMS with multiple transceivers may be able to receive E-MBS data while communicating with ABS on primary carrier.

The AMS with only one transceiver may perform carrier switching when the E-MBS configuration message and the E-MBS data are transmitted on an alternative carrier to the AMS' primary carrier. The AMS performs carrier switching to the secondary carrier to receive E-MBS data. When the AMS finishes receiving E-MBS data, the AMS shall stay in the primary carrier

A Carrier Switching Mode is included in AAI-DSA-REQ/RSP and AAI_DSC-REQ message. If Carrier Switching Mode is 0b0, the AMS's availability in the primary carrier is indicated using Unicast Available Interval bitmap transmitted in the AAI-DSA-REQ, AAI_DSC-REQ message to add and change the Unicast Available Interval.

When Carrier Switching Mode is 0b0, Unicast Available Interval Bitmap is included in AAI-DSA-REQ/RSP message for carrier switching mode to indicate the duration in which the AMS is available in the primary carrier for Unicast and duration the AMS is in the secondary carrier to receive E-MBS. The unicast available interval bitmap is a string of N bits b0,b1,b2,...,bN where bi can take on a value of 0 or 1 and i indicates the position of the bit in the string. The value of N depends on the MBS scheduling interval(MSI). Each bit bi corresponds to a time interval in the MSI. An MSI is split into N intervals with each interval represented by a bit. When the ith bit bi takes on value 0, then the AMS is unavailable to the primary carrier for unicast scheduling and is in the secondary carrier receiving E-MBS; if the ith bit bi takes on value 1, then the AMS is available to the primary carrier for unicast scheduling. Carrier switching from the secondary E-MBS carrier to the primary carrier occurs at the end of interval i when bibi+1 ==01 i.e., when the bits in the string at the ith and i+1th position are 0 and 1 respectively. Similarly, the AMS switches carrier from the primary carrier to the secondary E-MBS carrier occurs at the end of interval i when bibi+1 ==10 i.e., when the bits in the string at the ith and i+1th position are 1 and 0 respectively. Whenever the AMS adds/removes E-MBS content, the AMS shall discontinue carrier switching, return to the primary carrier. and the ABS shall re-allocate the Unicast Available Interval using AAI_DSA transaction. and update the Unicast Available Interval Bitmap using a new AAI-DSA transaction. AAI_DSC transaction is also used to update the Unicast Available Interval Bitmap.

When Carrier Switching Mode is 0b1, an AMS transmits the AAI-E-MBS-REP message to the ABS to inform the ABS which E-MBS service(s) the AMS intends to receive. After receiving an AAI-E-MBS-REP message, the ABS uses the E-MBS connection bitmap to compute the interval during which the AMS is available at the primary carrier for unicast scheduling. The ABS schedules unicast services for the AMS on its primary carrier based on the available interval computed using the connection bitmap in the AAI-E-MBS-REP message. When the AMS changes the concurrent receiving E-MBS service(s), it sends the AAI-E-MBS-REP message with an updated E-MBS Connection Bitmap.

The AAI-E-MBS-REP message described in Section 16.2.3.58 shall be transmitted from AMS to ABS when one of the following conditions are satisfied.

1. to request a start time from the ABS at which AMS can switch to the E-MBS carrier after DSx transaction

2. to update the indication regarding the E-MBS stream(s) the AMS is currently receiving without releasing the E-MBS connection allocated via AAI-DSA-REQ/RSP

3. to indicate the time at which the AMS stops E-MBS carrier switching without releasing the E-MBS connection allocated via AAI-DSA-REQ/RSP

Upon receiving the AAI-E-MBS-REP message, the ABS shall transmits an AAI-E-MBS-RSP message to the AMS as described in Section 16.2.3.59.

When an AMS performs carrier switching from the primary carrier to the secondary carrier, the UL HARQ retransmission processes under way shall be terminated

[Remedy2: Adapt the following change in 16.2.3.47.4 of 802.16m/D8]

16.2.3.47.4 AAI_DSC-REQ

AAI-DSC-REQ message is sent by an AMS or ABS to dynamically change the parameters of an existing service flow. An AMS or ABS shall generate AAI-DSC-REQ message, including the following parameters:

—Control Message Type : Type of AAI-DSC-REQ message.

—Service Flow Parameters : Specifies the service flow's new traffic characteristics and scheduling requirements. The admitted and active QoS parameter sets currently in use by the service flow. If the DSC message is successful and it contains service flow parameters, but does not contain replacement sets for both admitted and active QoS parameter sets, the omitted set(s) shall be set to null. The service flow parameters shall contain a FID.

Following parameters may be included in the AAI-DSC-REQ message.

—SCID: To switch sleep cycle setting

—Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSC-REQ. They are determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different and shall assign different BR index to different service flows whose UL Scheduling Type is same but BR size is different.

When an ABS updates E-MBS service flow, the following parameters may be included in the AAI-DSC-REQ message.

—E-MBS Zone ID: Indicates E-MBS zone ID where the connection for associated service flow is valid

—E-MBS Service Flow Parameter: Mapping updates of E-MBS ID and FID .

—Physical Carrier Index: Target carrier which the AMS switches or is redirected by ABS to, only included in ABS initiated DSC-REQ.

- Carrier Switching Mode: Indicates the carrier switching mode based on between unicast available interval and available interval report.

- Unicast Available Interval Bitmap: Indicates when the AMS should be available in the primary carrier.

When an ABS updates multicast service flow, the following parameters may be included in the AAI-DSC-REQ message.

—Multicast Service Flow Parameter: Mapping update of Multicast Group ID

An AAI-DSC-REQ message shall not carry parameters for more than one service flow.

Table 740—AAI_DSC-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | …… | … | …… | … |
| O | Num of E-MBS Zone ID to update | 3 | Number of E-MBS ID to update | Present when ABS initiates AAI_DSC-REQ Present only if Num of E-MBS Zone ID to update > 0 |
| O | fullNumE-MBSIdtoUpdateArr[1..8] for(i=1;i<=Num of E-MBS Zone ID to update;i++) { | Variable |  | Present only if Num of E-MBS Zone ID to update > 0 |
| M | A) Service_flow_update_indicator | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" | N.A. |
| M | B) E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | N.A. |
| O | C) New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| O | D) Physical Carrier Index | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
|  | Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSC message 0b1: carrier switching parameters using AAI_E-MBS-REP message | In case of ABS-initiated DSC for carrier switching to update |
|  | Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | In case of ABS-initiated DSC for carrier switching to update Shall be present if Carrier Switching Mode = 0b0 |
| O | E) Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. | Presented only if Service_flow_update_indicator is set to 0b0. |
| O | F)fullServiceFlowUpdateBitmapArr[1..15] for(m=0:m<N_ServiceFlowUpdateBitmap;m++){ | Variable | N_ServiceFlowUpdateBitmap is the number of bits to be set to 1 of Service Flow Update Bitmap For each bit in the Service Flow Update Bitmap = 1, an enrey is present to indicate the corresponding service flow is updated Range: 0~15 | Presented only if Service_flow_update_indicator is set to 0b0. and the number of bits set to 1 in Service Flow Update Bitmap > 1. |
| O | F.1) New_E-MBS ID | 12 | New E-MBS identifier. |  |
| O | F.2) FID | 4 | Flow ID |  |
|  | } |  |  |  |
| O | G) fullEMBSIdFidMappingArr[0...16] for(k=0;k<N_E-MBS_FID_Mapping;k++) { | Variable | N_E-MBS_FID_Mapping is the number of mappings Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. | Presented only if Service_flow_update_indicator is set to 0b1 |
| O | G.1) Current E-MBS ID | 12) | current E-MBS identifier |  |
| O | G.2) Current FID | 4 | Current flow ID |  |
| O | G.3) New_E-MBS ID | 12 | New E-MBS identifier |  |
| O | G.4) New FID | 4 | New flow ID |  |
|  | } |  |  |  |
|  | } |  |  |  |

[Remedy3: Adapt the following change in 16.2.3.47.7 of 802.16m/D8]

16.2.3.47.7 AAI-DSD-REQ

An AAI-DSD-REQ message is sent by an AMS or ABS to delete an existing service flow. An AMS or ABS shall generate AAI-DSD-REQ message, including the following parameters:

—FID : Flow identifier to be deleted.

—Control Message Type : Type of AAI-DSD-REQ message.

The SCID may be included in AAI-DSD-REQ to switch Sleep Cycle setting.

When An AAI-DSD-REQ message is sent by an AMS or ABS to delete an E-MBS service flow, the following parameters may be included in the AAI-DSD-REQ message.

—E-MBS Zone ID: Indicates E-MBS zone ID where the connection for associated service flow is valid

—E-MBS ID and FID of the connection to be deleted.

Table 743—AAI-DSD-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | N/A |
| O | FID | 4 | Flow identifier to be deleted. | Present when an ABS initiates AAI-DSD-REQ |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | Present when ABS initiates AAI-DSD-REQ |
|  | N_E-MBS_ID_FID_Mapping | 4 | Indicates the number of mappings of E-MBS ID and FID in the E-MBS Zone to delete Range: 0 ~ 15 | Present when ABS initiates AAI_DSD-REQ |
| O | fullEMBSIdFidArr[1..15] for(i=1: i<=N_E-MBS_ID_FID_Mapping; i++) { | Variable | Mapping of E-MBS ID and FID | Present when ABS initiates AAI-DSD-REQ |
| M | A) E-MBS ID | 12 | E-MBS Iidentifier Identifier |  |
| M | B) FID | 4 | Flow identifier |  |
|  | } |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]