| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for clean-up of the error handling procedure in UL persistent allocation (16.2.7) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Jeongki Kim, Seunghyun Kang, Kiseon Ryu, Youngsoo Yuk, and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com , seunghyun.kang@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D8 |  |
| Abstract | This contribution proposes the texts related to UL persistent allocation |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the clean-up of the error handling procedure (16.2.7)

Jeongki Kim, Seunghyun Kang, Kiseon Ryu, Youngsoo Yuk, and Jin Sam Kwak

- LG Electronics

# Introduction

In 16m Draft [1] although the normal UL HARQ procedure is synchronous, an ABS may transmit the UL Basic assignment A-MAP IE for HARQ sub-packet retransmission in case of the change of resource position/size for retransmission.

When an ABS fails to decode the UL burst indentified by the UL Persistent allocation A-MAP IE, the ABS may transmit UL basic assignment A-MAP IE for the UL sub-packet retransmission. In this case, if the ABS successfully decodes UL burst identified by the UL Basic Assignment A-MAP IE, the ABS assumes that the UL Persistent Allocation A-MAP IE is correctly received although the IE is lost by an AMS. It results in severe PA error as shown in Figure 1.

Figure 1

# Proposal

After an ABS fails to decode the UL burst indentified by UL Persistent Allocation IE, if the ABS transmits the UL basic assignment A-MAP IE for the UL sub-packet retransmission and successfully decodes the UL burst indentified by the UL Basic assignment A-MAP IE, the same UL Persistent Allocation A-MAP IE may be transmitted again after a subsequent allocation period.

Figure 2

- References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested Remedy: Modify the Section 16.2.7.4 as follows, on page 332]

16.2.7.4 Error handling procedure

…

In the absence of the UL data burst in the resource assigned by the UL Persistent Allocation A-MAP IE, the UL data burst transmitted by the AMS is not successfully decoded at the ABS, but may be detected as a NULL. In such a case, the ABS assumes that the AMS has not received the UL Persistent Allocation A-MAP IE and the ABS may transmit the same UL Persistent Allocation A-MAP IE again after a subsequent allocation period. In order to ensure that resource assignment information for subsequent persistent allocations is received correctly, if the initial data burst identified by the UL Persistent Allocation A-MAP IE cannot be decoded successfully after N_MAX_ReTX HARQ retransmissions, and no subsequent persistent allocation is decoded successfully, the same UL Persistent Allocation A-MAP IE may be transmitted again after a subsequent allocation period. After an ABS fails to decode the UL data burst in the resource assigned by UL Persistent Allocation IE, if the ABS transmits the UL Basic Assignment A-MAP IE for the UL sub-packet retransmission and successfully decodes the UL data burst in the resource assigned by the UL Basic Assignment A-MAP IE, the ABS may transmit the same UL Persistent Allocation A-MAP IE again after a subsequent allocation period.

--------------Proposed Texts ---------------------------------------------------------------------------