| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for clean-up of the error handling procedure in UL persistent allocation (16.2.7) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Jeongki Kim, Seunghyun Kang, Kiseon Ryu, Youngsoo Yuk, and Jin Sam Kwak LG Electronics Hyunkyu Yu Samsung Electronics Roshni Srinivasan Intel Corporation | E-mail: jeongki.kim@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com , seunghyun.kang@lge.com hk.yu@samsung.com roshni.m.srinivasan@intel.com |
| Re: | Sponsor ballot comment on P802.16m/D8 |  |
| Abstract | This contribution proposes the texts related to UL persistent allocation |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the clean-up of the error handling procedure (16.2.7)

Jeongki Kim, Seunghyun Kang, Kiseon Ryu, Youngsoo Yuk, and Jin Sam Kwak

- LG Electronics

Hyunkyu Yu

Samsung Electronics

Roshni Srinivasan

- Intel Corporation

# Introduction

In 16m Draft [1] although the normal UL HARQ procedure is synchronous, the ABS may transmit the UL Basic assignment A-MAP IE for HARQ sub-packet retransmission in case of the change of resource position/size for retransmission.

In the case of UL persistent allocation, if the ABS fails to decode the UL burst indentified by the UL Persistent allocation A-MAP IE, the ABS may transmit the UL basic assignment A-MAP IE for the retransmission. However, the ABS has some difficulties to ensure that the AMS has received the UL Persistent allocation A-MAP IE correctly regardless of the successful decoding of the UL data burst during the associated HARQ retransmissions as follows.

If the ABS fails to decode the UL burst after N_MAX_ReTX HARQ retransmissions, the ABS may assume that the AMS fails to receive the UL Persistent allocation A-MAP IE regardless of having the UL basic assignment A-MAP IE for the HARQ retransmissions. So, the ABS may transmit the UL Persistent allocation A-MAP IE again in order to prevent severe persistent allocation error in a subsequent allocation period.

On the other hand, if the ABS successfully decodes UL burst identified by the UL Basic Assignment A-MAP IE, the ABS may assume that the UL Persistent Allocation A-MAP IE is correctly received although the UL Persistent allocation A-MAP IE is lost. Furthermore, as shown in Figure 1, it results in severe persistent allocation error in a subsequent allocation period unless the ABS transmits the UL Persistent allocation A-MAP IE again.

Figure 1. Persistent allocation error after successful decoding of UL retransmission

# Proposal

The ABS ensure that the AMS has received the UL Persistent allocation A-MAP IE correctly only if the ABS successfully decodes the UL burst without having the UL basic assignment A-MAP IE during the HARQ retransmissions since the UL burst is identified by the UL Persistent allocation A-MAP IE only. In this regard, for UL persistent allocation, it would be better to have the error handling text as follows rather than to provide all the error cases which the AMS has not received the UL Persistent allocation A-MAP IE correctly.

“If the initial data burst identified by the UL Persistent Allocation A-MAP IE is successfully decoded without any additional transmission of the UL basic assignment A-MAP IE for retransmission, the ABS may assumes that the UL Persistent Allocation A-MAP IE is correctly received.”

# References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

-----------------------------------------------Proposed Text -----------------------------------------------

[Suggested Remedy: Modify the Section 16.2.7.4 as follows, on page 332]

16.2.7.4 Error handling procedure

For transmissions with HARQ enabled, an ACK is transmitted to acknowledge the successful decoding of a data burst, or a NACK is transmitted to notify failure in decoding a burst transmitted on the DL/UL. If an ACK for the data burst identified by the DL Persistent Allocation A-MAP IE is detected in the assigned HARQ Feedback channel assigned to the associated HARQ process, the ABS may assumes that the DL Persistent Allocation A-MAP IE is correctly received by AMS. If the initial data burst identified by the UL Persistent Allocation A-MAP IE is successfully decoded without any additional transmission of the UL basic assignment A-MAP IE for retransmission, the ABS may assumes that the UL Persistent Allocation A-MAP IE is correctly received.

When NULL detection is used, in the absence of an ACK or a NACK in the HARQ feedback channel assigned in the DL Persistent Allocation A-MAP IE for the data burst, the ABS assumes that the AMS has not received the DL Persistent Allocation A-MAP IE and the same DL Persistent Allocation A-MAP IE may be transmitted again after a subsequent allocation period.

In the case of de-allocation of persistent allocations in the DL/UL, the ABS shall transmit a HARQ Feedback Allocation in the DL/UL Persistent Allocation A-MAP IE. This allocation is used to identify the HARQ channel in which the ACK for the DL/UL Persistent Allocation A-MAP IE signaling the de-allocation is transmitted. In the absence (NULL detection) of an ACK, the ABS assumes that the AMS has not received the DL/UL Persistent Allocation A-MAP IE, and the same DL/UL Persistent Allocation A-MAP IE that signaled the de-allocation may be transmitted again after a subsequent allocation period.

In the absence of the UL data burst in the resource assigned by the UL Persistent Allocation A-MAP IE, the UL data burst transmitted by the AMS is not successfully decoded at the ABS, but may be detected as a NULL. In such a case, the ABS assumes that the AMS has not received the UL Persistent Allocation A-MAP IE and the ABS may transmit the same UL Persistent Allocation A-MAP IE again after a subsequent allocation period. In order to ensure that resource assignment information for subsequent persistent allocations is received correctly, if the initial data burst identified by the UL Persistent Allocation A-MAP IE cannot be decoded successfully after N_MAX_ReTX HARQ retransmissions, and no subsequent persistent allocation is decoded successfully, the same UL Persistent Allocation A-MAP IE may be transmitted again after a subsequent allocation period.

-----------------------------------------------Proposed Text -----------------------------------------------