| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up text of the AAI_SCD message and connection management (D6 - 16.2.3.31) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | Sponsor Ballot on IEEE p802.16m D8 |  |
| Abstract | This contribution proposes clean-up text of the AAI_SCD message and connection management. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up text of the AAI_SCD message and connection management (D8 - 16.2.3.31)

Heejeong Cho, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[Remedy #1: Adopt the following modification in page 174, line 53, section 16.2.3.31]

16.2.3.31 AAI-System Configuration Descriptor (SCD) Message

Table 711—AAI-SCD Message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | periodicityOfRngChSync | 4 | tThe periodicity of the S-RCH allocation (Table 924 923) | This is not sent in Femtocell and WirelessMAN-OFDMA Advanced co-existing System with FDM-based UL PUSC zone. |
| O | cntlStartCodeOfRngChSync | 2 | tThe parameter ks controlling the start root index of the RP codes for the S-RCH. | This is not sent in Femtocell and WirelessMAN-OFDMA with FDM-based UL PUSC zone.. |
| O | rangingPreambleCodeSync | 2 | tThe number of the RP codes for periodic ranging (Table 922 921). | This is not sent in Femtocell and WirelessMAN-OFDMA with FDM-based UL PUSC zone. |

[Remedy #2: Adopt the following modification in page 358, line 1, section 16.2.10]

16.2.10 Connection Management

Connection is a mapping between MAC peers of an ABS and one or more AMSs. When the mapping applies to ABS and one AMS, the connection is a unicast connection. Otherwise it is a multicast or broadcast connection. As specified in 16.3.6.5.2.4 16.3.5.5.2.4, messages sent over unicast connections are distinguished by either:

a)The 16-bit CRC masking in the unicast assignment A-MAP IEs using the STID. or

b)The 16-bit CRC masking and user bitmap in the GRA A-MAP IE

Broadcast connections are intended for reception by all AMS that may be listening, not to any specific AMS. Messages sent over broadcast connections are distinguished by the 16-bit CRC masking in the broadcast assignment A-MAP IEs as specified in 16.3.6.5.2.4 16.3.5.5.2.4.

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------