| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table reformatting and cleanup on sleep mode control messages (16.2.3.25) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Yeongmoon Son, Hyunjeong Kang Samsung Electronics | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | Sponsor ballot recirc #2 on P802.16m/D8 |  |
| Abstract | The contribution proposes to clean up MAC control message table format for AAI-SLP-REQ/RSP, AAI-TRF-IND, AAI-TRF_IND-REQ/RSP. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table reformatting and cleanup on sleep mode control messages

Yeongmoon Son, Hyunjeong Kang

Samsung Electronics

- Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for control messages related to Sleep Mode: AAI-SLP-REQ/RSP, AAI-TRF-IND, and AAI-TRF_IND-REQ/RSP.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Replace the Table 705 with the followings:]

| Table 705—Parameters for AAI-SLP-REQ |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-SLP-REQ message_format() { | — | — |  |
| Operation | 2 | This indicates operation request type of AAI-SLP-REQ message 0b00 : Exit from Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting |  |
| if(Operation != 00) { | — | — |  |
| SCID | 4 | Sleep Cycle ID 0~15 |  |
| Start Frame Number | 6 | Least Significant 6 bits of Frame Number 0~63 |  |
| if(Operation == 0b01 ¦¦ Operation == 0b10) { | — | — |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI-TRF-IND message is not sent for the AMS 1 : AAI-TRF-IND message is sent to the AMS during every Listening Window 0~1 |  |
| LWEF | 1 | Listening Window Extension Flag 0: the Listening window is of fixed duration 1: the Listening window is extensible |  |
| NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |  |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 |  |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 |  |
| Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 |  |
| Listening subframe bitmap | 8 | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake. The size of the bitmap equals to the number of AAI subframes of a frame. Bit #0 is mapped to the first AAI subframe. Each bit in the bitmap indicates: 0: AMS does not wake up at the specific AAI subframe 1: AMS wakes up at the specific AAI subframe |  |
| if(NSCF == 0b10) { | — | — |  |
| New Initial Sleep Cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). Value: 0~31 New Initial Sleep Cycle = Value + 1 |  |
| } | — | — |  |
| if(LWEF == 1) { | — | — |  |
| T_AMS | 6 | This timer is for Listening Window Extension of AMS. 0~31 |  |
| } | — | — |  |
| } | — | // ENDIF (Operation == 0b01 ¦¦ Operation == 0b10) |  |
| } | — | //ENDIF (Operation != 00) |  |
| } | — | — |  |

[Remedy 2: Replace the Table 706 with the followings:]

| Table 706—Parameters for AAI-SLP-RSP |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-SLP-RSP message_format() { | — | — |  |
| Response_Code | 2 | This indicates response type of AAI-SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI-SLP-REQ 0b10 : Rejection of AAI-SLP-REQ 0b11 : Reserved |  |
| if(Response_Code != 0b10) { | — | — |  |
| Operation | 2 | This indicates operation request type of AAI-SLP-RSP message 0b00 : Exit from Sleep Mode 0b01 : Enter Sleep Mode 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting |  |
| if(Operation != 00) { | — | — |  |
| SCID | 4 | Sleep Cycle ID 0~15 |  |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept 1: the fast feedback channel is de-allocated at the frame specified by Start_Frame_Number 2: the fast feedback channel is automatically de-allocated at the beginning of the Sleep Window whenever the fast feedback channel is newly assigned to the AMS during the Listening Window 3: Reserved |  |
| Start Frame Number | 6 | Least Significant 6 bits of Frame Number 0~63 |  |
| if(Operation == 0b01 ¦¦ Operation == 0b10) { | — | — |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI-TRF-IND message is not sent for the AMS 1 : AAI-TRF-IND message is sent to the AMS during every Listening Window 0~1 |  |
| LWEF | 1 | Listening Window Extension Flag 0: the Listening window is of fixed duration 1: the Listening window is extensible |  |
| NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |  |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 |  |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 |  |
| Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 |  |
| Listening subframe bitmap | 8 | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake. The size of the bitmap equals to the number of AAI subframes of a frame. Bit #0 is mapped to the first AAI subframe. Each bit in the bitmap indicates: 0: AMS does not wake up at the specific AAI subframe 1: AMS wakes up at the specific AAI subframe |  |
| if(TIMF == 1) { | — | — |  |
| SLPID | 10 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode 0~1023 |  |
| } | — | — |  |
| if(NSCF == 0b10) { | — | — |  |
| New Initial Sleep Cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). Value: 0~31 New Initial Sleep Cycle = Value + 1 |  |
| } | — | — |  |
| if(LWEF == 1) { | — | — |  |
| T_AMS | 6 | This timer is for Listening Window Extension of AMS. 0~31 |  |
| } | — | — |  |
| } | — | // ENDIF (Operation == 0b01 ¦¦ Operation == 0b10) |  |
| } |  | // ENDIF (Operation != 0b00) |  |
| } else { | — | // equivalent to if(Response_Code == 0b10) |  |
| REQ_duration |  | Waiting value for the AAI-SLP-REQ message retransmission , which is the Least Significant 8 bits of Frame Number. If REQ_duration is missing in AAI-SLP-RSP when Response_Code == 0b10, it shall be regarded as REQ_duration = 0 0~255 | Optional: This parameter may be omitted when (Response_Code == 0b10) |
| } | — | — |  |
| } | — | — |  |

[Remedy 3: Replace the Table 707 with the followings:]

| Table 707—Parameters for AAI-TRF-IND |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-TRF-IND message_format() { | — | — |  |
| FRMT | 1 | This indicates type of Traffic Indication in AAI-TRF-IND message. 0 : It indicates the SLPID bitmap-based traffic indication 1: It indicates the SLPID-based traffic indication |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information for supporting the emergency alert service 0b0: there is no emergency information 0b1: there is emergency information |  |
| if(FMT == 0) { | — | — |  |
| SLPID Group Indication Bitmap | 32 | It indicates the existence of each SLPID group N-th bit of SLPID-Group Indication Bitmap [MSB corresponds to N = 0] is allocated to SLPID Group that includes AMS with SLPID values from N*32 to N*32+31 0: There is no traffic for any of the 32 AMSs that belong to the SLPID-Group 1: There is traffic for at least one AMS in SLPID-Group. |  |
| Traffic Indication Bitmap | N*32 | It indicates the traffic indication for 32 AMSs in each SLPID group Each Traffic Indication bitmap comprises multiples of 32-bit long Traffic Indication units. A Traffic Indication unit for 32 SLPIDs is added to AAI-TRF-IND message whenever its SLPID Group is set to 1 32 bits of Traffic Indication Unit (starting from MSB) are allocated to AMS in the ascending order of their SLPID values: 0: Negative indication 1: Positive indication N= The number of ‘1’ in SLPID Group Indication Bitmap (i.e. the number of SLPID Group which has positive traffic indication) |  |
| } else { | — | — |  |
| Num_of_SLPIDs | 6 | It indicates the number of SLPID included in AAI-TRF-IND message 0~63 |  |
| for (i = 0; i < Num_of_SLPIDs; i++) { |  |  |  |
| SLPID | 10 | Each SLPID is used to indicate the positive traffic indication for an AMS 0~1023 |  |
| } | — | — |  |
| } | — | — |  |
| } | — | — |  |

[Remedy 4: Replace the Table 708 with the followings:]

| Table 708—Parameters for AAI-TRF_IND-REQ |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-TRF_IND-REQ message_format() { | — | — |  |
| Control Message Type | 8 | This indicates type of control message is AAI-TRF_IND-REQ message. Value = (TBD) |  |
| } | — | — |  |

[Remedy 5: Replace the Table 709 with the followings:]

| Table 709—Parameters for AAI-TRF_IND-RSP |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-TRF_IND-RSP message_format() { | — | — |  |
| Frame_Number | 10 | The least significant 10 bits of the frame number in which incoming LW will start 0~1023 |  |
| Sleep Cycle Length | 10 | The length of Sleep Cycle which contains the next scheduled Listening Window. If the AMS receives the negative traffic indication during the next scheduled Listening Window, the (current) Sleep Cycle shall be set to this value at that time. Value: 0~1023 Sleep Cycle Length = Value + 1 |  |
| } | — | — |  |

-------------------------------  Text End  ---------------------------------------------------