| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Adding IPinIP to Convergence Sublayer Classification Rule (5.2.5.2) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Muthaiah Venkatachalam, Shaocheng Wang, Shantidev Mohanty Intel Phillip Barber Huawei | Voice : +1 408-765-1866 E-mail: shantidev.mohanty@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-1 for P802.16m/D8. |  |
| Abstract | The contribution proposes new classification rule to the convergence sublayer in IEEE 802.16m (5.2.5.2). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Adding IPinIP to Convergence Sublayer Classification Rule (5.2.5.2)

Muthaiah Venkatachalam, Shaocheng Wang, Shantidev Mohanty

Intel

- Introduction

In the 802.16m/D8, the IP classification rules described in Section 5.2.5.2 are not suitable to classify IPinIP traffic such as VPN traffic into a different class. It may be noted that classifying such traffic into a different class and use a separate flow ID (FID) to provide differentiated QoS to enterprise VPN traffic.

This contribution proposes to add the parameters in the inner IP header to enable classification of  IPinIP traffic.

- Suggested changes in the 802.16m/D8

The following is the proposed change in the 802.16m/D8. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 53, page 16

Add the following sentence after line 53 as shown below.

------------------------------------------------- Start of proposed text I--------------------------------------------------

IP classification rules operate on the fields of the IP header and the transport protocol. For AAI, IP classification rules may operate on the fields of the inner IP header and/or outer IP header.

------------------------------------------------- end of proposed text I--------------------------------------------------

Suggested change #3: line 11, page 195

Insert the following rows in blue text into Table 742 after line 11 as shown below

------------------------------------------------- Start of proposed text III--------------------------------------------------

Table 742—AAI_DSA-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | N.6) Vendor-specific PHS parameters | variable | Compound |  |
| O | O.1) inner source ip | 32 | Source IP address of inner IP header |  |
| O | O.2) inner destination ip | 32 | Destination IP address of inner IP header |  |
| O | O.3) inner protocol field | 8 | Protocol field of inner IP header |  |
| O | O.4) inner TOS (Type of Service) | 8 | TOS field of inner IP header |  |
| O | O.5) inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header |  |
| O | ROHC Parameter |  |  |  |

------------------------------------------------- End of proposed text III--------------------------------------------------

Suggested change #4: line 56, page 204

Insert the following rows in blue text into Table 745 after line 56 as shown below

------------------------------------------------- Start of proposed text IV--------------------------------------------------

Table 745—AAI_DSC-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | N.6) Vendor-specific PHS parameters | variable | Compound |  |
| O | O.1) inner source ip | 32 | Source IP address of inner IP header |  |
| O | O.2) inner destination ip | 32 | Destination IP address of inner IP header |  |
| O | O.3) inner protocol field | 8 | Protocol field of inner IP header |  |
| O | O.4) inner TOS (Type of Service) | 8 | TOS field of inner IP header |  |
| O | O.5) inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header |  |
| O | ROHC Parameter |  |  |  |

------------------------------------------------- End of proposed text IV--------------------------------------------------

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”