| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Adding IPinIP to Convergence Sublayer Classification Rule (5.2.5.2) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Muthaiah Venkatachalam, Shaocheng Wang, Shantidev Mohanty Intel Phillip Barber Huawei | Voice : +1 408-765-1866 E-mail: shantidev.mohanty@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-3 for P802.16m/D9. |  |
| Abstract | The contribution proposes new classification rule to the convergence sublayer in IEEE 802.16m (5.2.5.2). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Adding IPinIP to Convergence Sublayer Classification Rule (5.2.5.2)

Muthaiah Venkatachalam, Shaocheng Wang, Shantidev Mohanty

Intel

- Introduction

In the 802.16m/D9, the IP classification rules described in Section 5.2.5.2 are not suitable to classify IPinIP traffic such as VPN traffic into a different class. It may be noted that classifying such traffic into a different class and use a separate flow ID (FID) to provide differentiated QoS to enterprise VPN traffic.

This contribution proposes to add the parameters in the inner IP header to enable classification of  IPinIP traffic.

- Suggested changes in the 802.16m/D9

The following is the proposed change in the 802.16m/D9. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 16, page 17

[note to editor] Add the following sentence after line 16 as shown below.

IP classification rules operate on the fields of the IP header and the transport protocol. For AAI, IP classification rules may operate on the fields of the inner IP header and/or outer IP header.

Suggested change #2: line 26, page 228

[note to editor] Insert the following rows in blue text into Table 734 after line 28 as shown below

Table 734—AAI_DSA-REQ Message Field Description

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| IEEE 802.1Q VLAN ID field | 2 | vlan_id1, vlan_id2 | Present if needed |
| inner source ip | 32 | Source IP address of inner IP header |  |
| inner destination ip | 32 | Destination IP address of inner IP header |  |
| inner protocol field | 8 | Protocol field of inner IP header |  |
| inner TOS (Type of Service) | 8 | TOS field of inner IP header |  |
| inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header |  |
| } |  |  |  |

Suggested change #3: line 27, page 245

[note to editor] Insert the following rows in blue text into Table 737 after line 27 as shown below

Table 737—AAI_DSC-REQ

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| IEEE 802.1Q VLAN ID field | 2 | vlan_id1, vlan_id2 | Present if needed |
| inner source ip | 32 | Source IP address of inner IP header |  |
| inner destination ip | 32 | Destination IP address of inner IP header |  |
| inner protocol field | 8 | Protocol field of inner IP header |  |
| inner TOS (Type of Service) | 8 | TOS field of inner IP header |  |
| inner IPv6 Flow Label | 8 | IPv6 Flow Label of inner IP header |  |
| } |  |  |  |

Suggested change #4: line 32, page 1010

[note to editor] Add the following sentences after line 32 as shown below.

EthernetAttributes ::= SEQUENCE {

ethMaskedDestMacAddrINTEGER (0..4095) OPTIONAL,

ethMaskedSrcMacAddrINTEGER (0..4095) OPTIONAL,

ethType INTEGER (0..7) OPTIONAL, -- ?? Not clear

priority INTEGER (0..3) OPTIONAL, -- ?? Not clear

vlanId BIT STRING (SIZE (0..3)) OPTIONAL -- ?? Not clear

}

IPinIPAttributes ::= SEQUENCE {

innerSouceIpAddr                     BIT STRING (SIZE (32)) OPTIONAL,

innerDestinationIpAddr              BIT STRING (SIZE (32)) OPTIONAL,

innerProtocolField                     BIT STRING (SIZE (8)) OPTIONAL,

innerTOSField             		        BIT STRING (SIZE (8)) OPTIONAL

innerIPv6FlowLabelField                 BIT STRING (SIZE (8)) OPTIONAL

}

RohcAttributes ::= SEQUENCE {

Suggested change #5: line 44, page 1009

[note to editor] Add the following sentences after line 44 as shown below.

phsRule PhsRule OPTIONAL,

iPinIPAttributes	IPinIPAttributes OPTIONAL

}

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”