| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Text Additions for NSP lists in AAI_SBC-REQ/RSP messages (16.2.3.5) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Xiangying Yang, Joey Chou Intel | Voice : +1 503-264-7867 E-mail: shaocheng.wang@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-1 for P802.16m/D8. |  |
| Abstract | The contribution proposes Text Additions for NSP lists in AAI_SBC-REQ/RSP messages (16.2.3.5) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Text Additions for NSP lists in AAI_SBC-REQ/RSP messages (16.2.3.5)

Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty

Intel

- Introduction

In the 802.16e, there are two options to acquire NSP information, one in SBC procedure and the other in SII-ADV. In the 802.16m/D8, only AAI_SII-ADV option is available. The benefit for 802.16m system to have the second option of acquiring NSP lists in SBC procedure is very clear. It ensures effective and flexible communication of NSP information.

This contribution proposes text additions to add NSP information in AAI_SBC-REQ/RSP messages.

- Suggested changes in the 802.16m/D8

The following is the proposed change in the 802.16m/D8. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1:

Modify Table 683 in page 102, line 60 with the following additional rows

------------------------------------------------- Start of proposed text I--------------------------------------------------

Table 683—AAI-SBC-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
|  | If (SIQ is included) { |  |  |  |
| O | SIQ (Service Information Query) | 2 | Bit 0: indicates that the AMS requests transmittal of the NSP List for the list of NSP IDs supported by the Operator Network that includes the current ABS; Bit 1: indicates that the AMS requests transmittal of the Verbose NSP Name List in addition to the NSP List; bit 1 shall not be set to a value of ‘1’ unless |  |
|  | }else{ |  |  |  |
| M | CAPABILITY_INDEX | 5 | It refers to the "Capability Class" that the AMS can sup-port. Value: 0~31 |  |
| M | DEVICE_CLASS | 5 | It refers to the "Device Class" that the AMS can support. Value: 0~31 |  |
| … | … | …. | … |  |
| … | … | … | … |  |
|  | MAX Tx Power | 24 | Bit 0-7: Maximum transmit-ted power for QPSK. Bit 8-15: Maximum transmit-ted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. | Present as needed |
|  | } |  |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

Suggested change #2:

Modify Table 684 in page 105, line 52 with the following additional rows

------------------------------------------------- Start of proposed text II--------------------------------------------------

Table 684—AAI-SBC-REP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. | Present as needed |
|  | If (SIQ bit 0 in AAI-SBC_REQ is set and if AAI-SII-ADV Message Pointer is not included) { |  |  |  |
|  | Number of NSP IDs | 4 | Number of Network Service Providers |  |
|  | for (i=0; i<number of NSP IDs; i++) { |  | List of the Network Service Provider identifiers |  |
|  | NSP identifier | 24 | NSP identifier which ABS supports |  |
|  | } |  |  |  |
|  | If (if SIQ bit 1 in AAI-SBC_REQ is set) { |  |  |  |
|  | for (i=0; i< number of NSP IDs; i++) { |  | List of the verbose names of the NSPs. The value of Verbose NSP Name List is a compound list of verbose NSP name lengths and verbose NSP names. The order of the Verbose NSP Name Lengths and Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List. |  |
|  | verbose NSP name length | 7 | Length of verbose NSP name (in byte) |  |
|  | verboseNspName | Variable (1 ... 128 x 8) | Verbose NSP name string |  |
|  | } |  |  |  |
|  | } |  |  |  |
|  | }else if (SIQ bit 0 in AAI-SBC_REQ is set and if AAI-SII-ADV Message Pointer is included) { |  |  |  |
| O | AAI-SII-ADV Message Pointer | 14 | The 14 least significant bits of the frame number of the frame in which the AAI-SII-ADV message with requested NSP information is transmitted. |  |
|  | } |  |  |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”