| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modification of N_ACID in UL persistent A-MAP IE (16.3.5.5.2.4.9) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Youngsoo Yuk LG Electronics | E-mail: youngsoo.yuk@lge.com |
| Re: | Sponsor ballot recirc #2 on P802.16m/D8 |  |
| Abstract | This contribution proposes the texts related to UL persistent allocation |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification of N_ACID in UL persistent A-MAP IE (16.3.5.5.2.4.9)

Youngsoo Yuk

- LG Electronics

# Introduction

In 16m D8, N_ACID for UL is changed into lower number (2, 3, 4, 8  1, 2, 3, 4). In this case, if UL_N_MAX_ReTx is 8, allocation period of 2 frames cannot be supported with 4 ACIDs.

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested Remedy: Modify Table 857 in page 633 as follows:]

Table 857 - UL Persistent Allocation A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| UL Persistent Allocation A-MAP_IE () { |  |  |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| Allocation Period | 2 | Period of persistent allocation. If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| … |  |  |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. If (UL_N_MAX_ReTx==4){ 0b00: 1 0b01: 2 0b10: 3 0b11: 4 } else { 0b00: 2 0b01 :3 0b10: 4 0b11: 8 } |
| Reserved | 1 |  |
| } |  |  |
| } |  |  |

--------------Proposed Texts  ---------------------------------------------------------------------------