| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change for multicast operation in IEEE 802.16m (16.2.28) |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | Seho Kim, Sangheon Kim, Mingxia Xu, , Byungwook Jun, Jaehyuk Jang, Hyunkyu Yu, Youngbo Cho, Heewon Kang Samsung Electronics | E-mail: seho42.kim@samsung.com |
| Re: | Sponsor Ballot Recirc #1 Section 16.2.28 Support for Multicast Service |  |
| Abstract | The contribution provides the text for multicast operation in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change for Multicast operation in IEEE 802.16m (16.2.28)

Seho Kim, Sangheon Kim, Mingxia Xu, Byungwook Jun, Jaehyuk Jang, Hyunkyu Yu, Youngbo Cho, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

To support multicast service, an ABS establishes a DL multicast service by creating a multicast connection with each AMS to be associated with the service. Any available FID may be used for the multicast service (i.e., there are no dedicated FIDs for multicast transport connections). To ensure proper multicast operation, the FID used for the service shall be the same for all AMSs on the same channel that participate in the connection.

In order to provide multicast service with hundreds of users, an ABS shall assign common FID to users belonged to same Multicast group. It seems to be hard to support multicast service without using dedicated FIDs for multicast transport connections. However, dedicated FIDs for specific transport connections will be risky since four flows are already assigned for dedicated usage. To assign common FID for users belonged to multicast group, FID defined in domain of Multicast group is proposed and multicast flow is indentified as the combination of Multicast Group ID and FID where Multicast Group ID is 12 bit long and FID is 4 bit long.

In this proposal, the following terms are changed or modified.

- For initiating and updating multicast flow information, Multicast Group ID and FID should be provided in AAI_DSA-REG, AAI_DSA-RSP, AAI_DSC-REG and AAI_RNG-RSP message.

- For multicast assignment, Multicast Group ID and FID is indicated in Broadcast Assignment A-MAP IE.

- Proposed Text Changes

Removed text

Added text

[Remedy #1, Modify the following text after line 5 in page 467 as follows,]

16.2.28.3 Multicast Operation in Connected State

When an AMS performs handover to a new ABS while in Active Mode or Sleep Mode, the AMS shall send AAI_RNG-REQ message with Ranging Purpose Indication = 0b00001 at the target ABS. Upon reception of the AAI_RNG-REQ message from the AMS, the ABS shall include Multicast Group ID in the AAI_RNG-RSP parameters to provide an updated Multicast Group ID, if any.

[Remedy #2, Modify the following text in Table 680, page 94, section 16.2.3.2 as follows,]

Table 680 – AAI_RNG-RSP message Field Descriptions

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the target ABS | Presented only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to be updated. |
| O | Current_Multicast Group ID and FID and New_Multicast Group ID and FID | Variable (32xM) | Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Present only if Num of Multicast Group ID and FID to be updated > 0 |

[Remedy #3, Modify the following text after line 4, page 205, section 16.2.3.47.1 as follows,]

When an ABS commences multicast service, the following parameters shall be included in the AAI_DSA-REQ message.

- Multicast Service: Indicates whether the multicast service is being requested or provided for the connection that is being setup.

- Multicast Group ID for the connection that is associated with the service flow in DSA-REQ.

[Remedy #4, Delete the following text in line 40, page 217, section 16.2.3.47.2 as follows,]

When an AMS commences multicast service, the ABS shall include the following parameters in the AAI_DSA-RSP message.

- Multicast Service: Indicates whether the multicast service is being requested or provided for the connection that is being setup.

- Multicast Group ID for the connection that is associated with the service flow in DSA-RSP.