| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change for multicast operation in IEEE 802.16m (16.2.28) |  |
| Date Submitted | 2010-09-15 |  |
| Source(s) | Seho Kim, Sangheon Kim, Mingxia Xu, , Byungwook Jun, Jaehyuk Jang, Yeongmoon Son, Hyunkyu Yu, Youngbo Cho, Heewon Kang Samsung Electronics | E-mail: seho42.kim@samsung.com |
| Re: | Sponsor Ballot Recirc #1 Section 16.2.28 Support for Multicast Service |  |
| Abstract | The contribution provides the text for multicast operation in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change for Multicast operation in IEEE 802.16m (16.2.28)

Seho Kim, Sangheon Kim, Mingxia Xu, Byungwook Jun, Jaehyuk Jang, Hyunkyu Yu, Youngbo Cho, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

To support multicast service, an ABS establishes a DL multicast service by creating a multicast connection with each AMS to be associated with the service. The Multicast Group ID is defined by specific ABS so that the Multicast Group ID should be updated in handover process for providing seamless multicast service. If there are many AMS supposed to receive multicast burst, the updating relevant parameters of Multicast service for all corresponding AMSs in handover causes heavy control overhead. In order to decrease the overhead in updating Multicast parameters, the Multicast Group ID uniquely defined in entire network is required.

- To uniquely define Multicast Group ID in entire network, Multicast Group ID is extended into 16 bit long.

- Since Multicast Group ID is uniquely defined in entire network, updating multicast parameters in handover is unnecessary so that updating of Multicast Group ID in AAI_RNG-RSP is deleted.

To support multicast service, an ABS establishes a DL multicast service by creating a multicast connection with each AMS to be associated with the service. Any available FID may be used for the multicast service (i.e., there are no dedicated FIDs for multicast transport connections). To ensure proper multicast operation, the FID used for the service shall be the same for all AMSs on the same channel that participate in the connection.

In order to provide multicast service with hundreds of users, an ABS shall assign common FID to users belonged to same Multicast group. It seems to be hard to support multicast service without using dedicated FIDs for multicast transport connections. However, dedicated FIDs for specific transport connections will be risky since four FIDs are already assigned for dedicated usage. In order to assign same FID for users belonged to multicast group, FID defined in domain of Multicast group is proposed and multicast flow is indentified as the combination of Multicast Group ID and FID where Multicast Group ID is 16 bit long and FID is 4 bit long. AMS can have multiple multicast flows so that the function of Group parameter create/change should be supported in AAI_DSA-REQ and AAI_DSC-REQ messages.

- For initiating and updating multicast flow information, Multicast FID is introduced in AAI_DSA-REG, AAI_DSA-RSP, and AAI_DSC-REG message.

- Group parameter create/change for Multicast flow is supported in AAI_DSA-REQ and AAI_DSC-REQ messages.

- For multicast assignment, Multicast Group ID with 16bit long is indicated in Broadcast Assignment A-MAP IE.

- Proposed Text Changes

Removed text

Added text

[Remedy #1, Modify the following text in line 57, page 55, section 16.2.1.2.6 as follows,]

16.2.1.2.6 Multicast Group Identifier

A 12-bit value that is used to uniquely indentify a multicast flow in the domain of the ABS.

A 16-bit value that is used to uniquely indentify a multicast flow in entire network

[Remedy #2, Modify the following text after line 59, page 54, section 16.2.1.2.2. as follows,]

16.2.1.2.2 Flow Identifier (FID)

Each AMS connection is assigned a 4 bit FID that uniquely identifies the connection within the AMS. FIDs identify control connection and unicast transport connections. FID for E-MBS connection is used along with a 12-bit E-MBS ID to uniquely identify a specific E-MBS flow in the domain of an E-MBS zone (see 16.9.3.2). FID for Multicast connection is used along with a 16-bit Multicast Group ID to uniquely identify a specific Multicast flow in entire network. DL and UL Transport FIDs are allocated from the transport FID space as defined in Table 652. An FID that has been assigned to one DL transport connection shall not be assigned to another DL transport connection belonging to the same AMS.

[Remedy #3, Insert the following text after line 25, page 56, section 16.2.2.1.1. as follows,]

16.2.2.1.1 Advanced Generic MAC Header (AGMH)

The AGMH format is defined in Table 653. For E-MBS services, the FID shall be ignored by the receiver. For Multicast service, the FID associated with Multicast Group ID shall be used in FID field of AGMH.

[Remedy #4, Modify the following text in Table 680, page 94, section 16.2.3.2 as follows,]

Table 680 – AAI_RNG-RSP message Field Descriptions

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the target ABS | Presented only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and network re-entry for HO procedure if it needs to be updated. |
| O | Current_Multicast Group ID and FID and New_Multicast Group ID and FID | Variable (32xM) | Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Present only if Num of Multicast Group ID and FID to be updated > 0 |

[Remedy #5, Modify the following text after line 4, page 205, section 16.2.3.47.1 as follows,]

When an ABS commences multicast service, the following parameters shall be included in the AAI_DSA-REQ message.

- Multicast Service: Indicates whether the multicast service is being requested or provided for the connection that is being setup.

- Multicast Group ID for the connection that is associated with the service flow in DSA-REQ.

- Multicast Group ID: Indicates Multicast group for the connection that is associated with the service flow in DSA-REQ

[Remedy #6, Modify the following text in Table 737, page 215, section 16.2.3.47.1 as follows,]

Table 737 – AAI_DSA-REQ

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | For (i = 0 ; i < N ; i++) { |  | N: Number of Multicast Group ID to be added ( N = 0, 1, .., 15) | Present if ABS wants to add multicast group ID list |
| M | Multicast Group ID | 16 | ID of a group to which the flow is added |  |
|  | } |  |  |  |
| O | FID | 4 | FID which is associated with Multicast Group IDs | Present if N>0 |
| O | Group Parameter Create/Change | variable |  | Present if needed |
| O | A) Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| O | B) Qty SFID request | M | The number of service flows to create | Shall only be sent by an AMS |
| M | C) Group FID List | 4 x N | FIDs of service flows created with this Group Parameter Create/Change |  |
| O | D) Group FID Parameters List | variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
| O | Coupled Group Create/Change | variable |  |  |
| O | A) Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | B) Qty Coupled SFID request | M | The number of DL service flow and UL service flow to create |  |
| M | C) Coupled FID List | N x 8 | FIDs of DL and UL service flow (4 MSB for DL FID and 4 LSB for UL FID) |  |
| O | D) Coupled FID Parameter List | variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |
| O | Num of Multicast SFID | 4 | Number of Multicast service flow to add | Present when Num of Multicast SFID > 0 |
| M | A) Multicast Group ID | 12 | ID of a group to which the flow is added | Present when Num of Multicast SFID > 0 |
| M | B) FID | 4 | FID of service flow created | Present when Num of Multicast SFID > 0 |

[Remedy #7, Delete the following text in line 40, page 217, section 16.2.3.47.2 as follows,]

When an AMS commences multicast service, the ABS shall include the following parameters in the AAI_DSA-RSP message.

- Multicast Service: Indicates whether the multicast service is being requested or provided for the connection that is being setup.

- Multicast Group ID for the connection that is associated with the service flow in DSA-RSP.

- Multicast Group ID: Indicates Multicast group for the connection that is associated with the service flow in DSA-RSP

[Remedy #8, Modify the following text in Table 738, page 219, section 16.2.3.47.2 as follows,]

Table 738 – AAI_DSA-RSP

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | Num of Multicast Group ID | 4 | Number of Multicast group to add | Present when Num of Multicast Group ID > 0 |
| O | For (i = 0; i < N ; i++) { |  | N: Number of Multicast Group ID to be added ( N = 0, 1, .., 15) | Present if ABS wants to add multicast group ID list |
| M | Multicast Group ID | 1216 | ID of a group to which the flow is added | Present when Num of Multicast Group ID > 0 |
|  | } |  |  |  |
| O | FID | 4 | FID which is associated with Multicast Group IDs | Present if N>0 |

[Remedy #9, Modify the following text in Table 740, page 227, section 16.2.3.47.4 as follows,]

Table 740 – AAI_DSC-REQ

| M/O | Attributes/Array of attributes | Size(bit) | Value / Note | Condition |
| --- | --- | --- | --- | --- |
|  | ….. |  |  |  |
| O | For (i = 0; i < N ; i++) { |  | N: Number of Multicast Group ID to be added (N = 0, 1, .., 15) | Present if ABS wants to add multicast group ID list |
| M | Multicast Group ID to be added | 16 | Multicast Group ID to be added |  |
|  | } |  |  |  |
| O | FID | 4 | FID which is associated with newly added Multicast Group IDs | Present if N>0 |
| O | For (i = 0; i < M ; i++) { |  | M: Number of Multicast Group ID to be deleted (M = 0, 1, .., 15) | Present if ABS wants to delete multicast group ID list |
| M | Multicast Group ID to be deleted | 16 | Multicast Group ID to be deleted |  |
|  | } |  |  |  |
| O | Group Parameter Create/Change | variable |  | Present if needed |
| O | A) Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| O | B) Qty SFID request | M | The number of service flows to create | Shall only be sent by an AMS |
| M | C) Group FID List | 4 x N | FIDs of service flows created with this Group Parameter Create/Change |  |
| O | D) Group FID Parameters List | variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
| O | Coupled Group Create/Change | variable |  |  |
| O | A) Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | B) Qty Coupled SFID request | M | The number of DL service flow and UL service flow to create |  |
| M | C) Coupled FID List | N x 8 | FIDs of DL and UL service flow (4 MSB for DL FID and 4 LSB for UL FID) |  |
| O | D) Coupled FID Parameter List | variable |  |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |
| O | Initial Backoff Window Size | 4 | Window size expressed as a power of 2 | For Uplink only |
| O | Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 | For Uplink only |
| O | Backoff Scaling Factor | 4 | "0010" for Binary Exponential Backoff | For Uplink only |
| O | Num of Multicast Group ID to add | 4 | Number of Multicast Group IDs to add | Present when ABS initiates AAI_DSC-REQ Present only if Num of Multicast Group ID to add > 0 |
| M | Multicast Group ID to be added | 12 | Multicast Group ID to be added | Present only if Num of Multicast Group ID to add > 0 |
| O | Num of Multicast Group ID to delete | 4 | Number of Multicast Group IDs to delete | Present when ABS initiates AAI_DSC-REQ Present only if Num of Multicast Group ID to delete > 0 |
| M | Multicast Group ID to be deleted | 12 | Multicast Group ID to be deleted | Present only if Num of Multicast Group ID to delete > 0 |

[Remedy #10, Modify the following text after line 3 in page 467 as follows,]

16.2.28.3 Multicast Operation in Connected State

When an AMS performs handover to a new ABS while in Active Mode or Sleep Mode, the AMS shall send AAI_RNG-REQ message with Ranging Purpose Indication = 0b0000 at the target ABS. Upon reception of the AAI_RNG-REQ message from the AMS, the ABS shall include Multicast Group ID in the AAI_RNG-RSP parameters to provide an updated Multicast Group ID, if any.

[Remedy #11, Modify the following text in Table 862, page 650, section 16.3.5.5.2.4.13 as follows,]

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast or multicast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 865 describes the fields in a broadcast assignment A-MAP IE.

Table 862 – Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carriers ranging channel allocation information 0b10: This IE carriers multicast assignment information 0b11: reserved |
| If(Function Index == 0b00){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 13 | Reserved bits |
| }else{ |  |  |
| Reserved | 15 | Reserved bits |
| } |  |  |
| }else if{Function Index == 10b01}{ |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 2 | Opportunity index of the ranging channel |
| } |  |  |
| Reserved | 10 | Reserved bits |
| } else if{Function Index == 0b10}{ |  |  |
| Multicast Group ID | 1216 | ID of a group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 4 | Reserved bits |
| } |  |  |
| } |  |  |