| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Zone Switch Capability (section 16.2.6.4.1.2.1) |  |
| Date Submitted | 2010-09-15 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7188 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D8 for Sponsor Ballot |  |
| Abstract | This contribution is to propose text changes for HO (section 16.2.6.4.1.2.1) |  |
| Purpose | Update P802.16m/D8 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DL capability in MZone for expediting Zone Switch (sections 16.2.6.4.1.2.1)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes proposed text for AMS capability allowing DL operation in MZone while operating in LZone for actual zone switch benefit .

- Rationale/Motivation of modifications

Zone switch mode 0 incurs a latency over at lease 80ms, which is just for acquisition of SFH. Considering synchronization, the latency may increase over 100ms or more. Hence, the most time consuming action required is the acquisition of MZone’s SFH.

As in HO, pre-providing MZone’s SFH may decrease such latency to under 30ms.

- References

[1] IEEE P802.16m/D8, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy #1

On the 80216m_D8, page 322, line 60, add proposed text in section 16.2.6.4.1.2.1 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.6.4.1.2.1 Zone Switch based Handover Procedure

…

The Zone Switch TLV may also include the following:

- Temporary STID for being used in MZone

— Ranging initiation deadline: Valid time for Temporary STID. Shall be included if Temporary STID is included.

- System information of MZone

— SFH1: System information subpacket of SFH 1

— SFH2: System information subpacket of SFH 2

— SFH3: System information subpacket of SFH 3

- Superframe start offset

— •	Superframe start offset: The time offset indicating the start of the next superframe header in MZone in units of frames.

The AMS shall include its Zone Switch capability TLV information, defined in section 16.10.2.2, in the RNG-REQ message if the AMS supports Zone Switch Mode 1. If the AMS does not support Zone Switch Mode 1, it shall not send Zone Switch capability TLV information in the RNG-REQ message. As a response, the ABS shall send the RNG-RSP message including the Zone Switch TLV information. The Zone Switch TLV may convey system information of the MZone and the start frame offset that indicates the start of the superframe header in the MZone.

In case of Zone Switch Mode 0, the AMS performs network reentry in MZone by performing DL synchronization with the MZone and listening to the ABS's SFH to acquire the system information of the MZone, if ABS’s SFH is not already provided in Zone Switch TLV, at Action Time. Upon Action Time, the data communication in LZone is stopped. In case of Zone Switch Mode 1, the AMS receives the ABS's SFH to acquire the system information, if ABS’s SFH is not already provided in Zone Switch TLV, and maintains its normal operation in LZone (e.g., exchanging user data with the ABS in LZone) while performing network reentry in MZone if data path in LZone has been established before the start of zone switch operation. The data communication in LZone stops upon completion of network entry in MZone.

…

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #2

On the 80216m_D8, page 907, line 1, add proposed table in section 16.10.2.1 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.10.2.1 RNG-RSP management message encodings

The encodings in Table 977 are specific to the RNG-REQ message sent in LZone of an AMS indicating its capability for Zone Switch Mode defined in section 16.2.6.4.1.2.1.

Table 980—OFDMA-specific RNG-RSP message encodings

| Name | Type (1 byte) | Length | Value |
| --- | --- | --- | --- |
| Ranging initiation deadline | 47 | 1 | Valid time for Temporary STID. Shall be included if Temporary STID is included. |
| ABS’s MZone SFH Subpkt 1 | 48 | Variable | Full subpacket information of Sub-packet 1 provided |
| ABS’s MZone SFH Subpkt 2 | 49 | Variable | Full subpacket information of Sub-packet 2 provided |
| ABS’s MZone SFH Subpkt 3 | 50 | Variable | Full subpacket information of Sub-packet 3 provided |
| MZone Superframe start offset | 51 | Variable | Offset indicating the start of the next superframe header in the MZone in units of frames |

--------------------------------------------------------Text End -----------------------------------------------------