| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of Ethernet CS (5.2.4, 16.2.3.8, 16.2.3.9) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Jungshin Park, Youngkyo Baek, Rakesh Taori Samsung Electronics | shin02.park@samsung.com |
| Re: | Sponsor ballot recirc #2 on P802.16m/D8 |  |
| Abstract | The contribution proposes to support Ethernet CS in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support of Ethernet CS

Jungshin Park, Youngkyo Baek, Rakesh Taori

Samsung Electronics

- Introduction

The Ethernet service is already deployed and provided to the subscribers today by WiMAX operators in some regions and countries.

However, the current 802.16m draft specification excludes the capabilities to support transmission of Ethernet frames over the advanced air interface, except by additional manipulation of multiprotocol flow specific extension. This requires unnecessary processing overhead at AMS and network for the advanced network to interwork with the legacy network, and makes handovers between the two networks difficult with much implementation restrictions. For example, if AMS handovers from ABS to BS, leftover MAC packets at old ABS and AMS should be reconstructed to prevent any packet losses by the handover, which is not easy at all to implement in reality.

This contribution proposes the support of 16e-defined Ethernet CS in the advanced air interface (16m), to enforce the backward compatibility. Some IP CS related attributes cleanups are also included.

- Text change

-------------------------------  Change #1 Start  ---------------------------------------------------

[Remedy 1:]

[Add the following texts in subclause 5.2.4 (page 16, line 30):]

5.2.4 IEEE 802.3/Ethernet-specific part

5.2.4.1 IEEE 802.3/Ethernet CS PDU format

The IEEE 802.3/Ethernet PDUs are mapped to MAC SDUs according to Figure 14 (when header suppression is enabled at the connection, but not applied to the CS PDU) or Figure 15 (with header suppression). In the case where PHS is not enabled, PHSI field shall be omitted.

The IEEE 802.3/Ethernet PDU shall not include the Ethernet FCS when transmitted over this CS.

Figure 14—IEEE 802.3/Ethernet CS PDU format without header suppression

Figure 15—IEEE 802.3/Ethernet CS PDU format with header suppression

ROHC (refer to RFC 3095) may be used in addition to PHS to compress the IP header portion of an IP packet over Ethernet frame. The AMS and the ABS shall set bit 7 of Request/Transmission Policy (see 11.13.11) to 0 to enable ROHC. When ROHC is enabled for a service flow, the service flow constitutes what in RFC 3095 is referred to as a ROHC channel. Two service flows cannot share a ROHC channel, and two ROHC channels cannot share the same service flow. On a service flow for which ROHC has been enabled, all of the IP packet parts of IP over Ethernet frames shall pass through the ROHC compressor on the sender side and the decompressor on the receiver side. ROHC compression and decompression operation shall be performed in accordance with RFC 3095, RFC 3759, RFC 3243, RFC 4995, RFC 3843, RFC 4996. To enable ROHC, the following two steps are required:

1) Capability negotiation during AAI-REG-REQ/RSP message exchange to determine whether ROHC is supported.

2) Indication in AAI-DSA-REQ/RSP messages to enable ROHC for the service flow.

5.2.4.2 IEEE 802.3/Ethernet CS classification rules

The following parameters are relevant for IEEE 802.3/Ethernet CS classification rules:

— IEEE 802.3/Ethernet header classification parameters—zero or more of the IEEE 802.3/Ethernet, VLAN and IP headers may be included in the classification. In this case, only the IEEE 802.3/IEEE 802.1Q/IP (11.13.18.3.3.2 through 11.13.18.3.3.12 and 11.13.18.3.3.16) classification parameters are allowed.

— For IP over IEEE 802.3/Ethernet, Ethernet, and VLAN, IP headers may be included in classification. In this case, only the IP, IEEE 802.3 and IEEE 802.1Q (11.13.18.3.3.2 through 11.13.18.3.3.12 and 11.13.18.3.3.16) classification parameters are allowed.

— For IP-header compressed IP over IEEE 802.3/Ethernet, Ethernet and VLAN headers may be included in the classification. In this case, only the IEEE 802.3/IEEE 802.1Q (11.13.18.3.3.8 through 11.13.18.3.3.12) classification parameters are allowed.

------------------------------- Change #1  End  ---------------------------------------------------

------------------------------- Change #2  Start  ---------------------------------------------------

[Remedy 2:]

[change table 686 (page 111, line 6) as the following:]

Table 686—AAI-REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | A) CS type | 16 or 32 | A bit set to "1" Indicates which CS Type the AMS supports Bit #0: Reserved Bit #1: Packet, IPv4 Bit #2: Packet, IPv6 Bit #3: ReservedPacket, IEEE 802.3/Etherneta Bit #4: Reserved Bit #5: ReservedPacket, IPv4 over IEEE 802.3/Etherneta Bit #6: ReservedPacket, IPv6 over IEEE 802.3/Etherneta Bit #7: Reserved Bit #8: Reserved Bit #9: Reserved Bit #10: Reserved Bit #11: Reserved Bit #12: Reserved Bit #13: Reserved Bit #14: Packet, noneIPb Bit #15: Multiprotocol flow, IPv4 or IPv6 Traffic Bit #16 - Bit #31 Reserved (a: Classifiers for IEEE 802.1Q VLAN tags may be applied to service flows of this CS type) (b: SDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload) | Present if needed |
| … | … | … | … | … |
| O | IP-Service-Type IE | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | Present if needed |

------------------------------- Change #2  End  ---------------------------------------------------

------------------------------- Change #3  Start  ---------------------------------------------------

[Remedy 3:]

[change table 687 (page 114, line 35) as the following:]

Table 687—AAI-REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | A) CS type | 16 or 32 | A bit set to "1" Indicates which CS Type the AMS supports Bit #0: Reserved Bit #1: Packet, IPv4 Bit #2: Packet, IPv6 Bit #3: ReservedPacket, IEEE 802.3/Etherneta Bit #4: Reserved Bit #5: ReservedPacket, IPv4 over IEEE 802.3/Etherneta Bit #6: ReservedPacket, IPv6 over IEEE 802.3/Etherneta Bit #7: Reserved Bit #8: Reserved Bit #9: Reserved Bit #10: Reserved Bit #11: Reserved Bit #12: Reserved Bit #13: Reserved Bit #14: Packet, noneIPb Bit #15: Multiprotocol flow, IPv4 or IPv6 Traffic Bit #16 - Bit #31 Reserved (a: Classifiers for IEEE 802.1Q VLAN tags may be applied to service flows of this CS type) (b: SDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload) | Present if needed |
| … | … | … | … | … |
| O | F) IP-Service-Type IE | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | Present if needed |

------------------------------- Change #3  End  ---------------------------------------------------

------------------------------- Change #4  Start  ---------------------------------------------------

[Remedy 4:]

[change table 737 (page 211, line 19) as the following:]

Table 737—AAI-DSA-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | A) CS specification parameter | 8 | A bit set to "1" Indicates which CS Type the AMS supports 0: Reserved 1: Packet, IPv4 2: Packet, IPv6 3: ReservedPacket, IEEE 802.3/Etherneta 4: Reserved 5: ReservedPacket, IPv4 over IEEE 802.3/Etherneta 6: ReservedPacket, IPv6 over IEEE 802.3/Etherneta 7: Reserved 8: Reserved 9: Reserved 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 16 – 255: Reserved (a: Classifiers for IEEE 802.1Q VLAN tags may be applied to service flows of this CS type) (b: SDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload) | Present if needed |
| O | CS parameter encoding rules | 4 | 99 Reserved 100 Packet, IPv4 101 Packet, IPv6 102 Reserved 103 Reserved 104 Reserved 105 Reserved 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 Reserved 113 Packet IP 114 Packet, Multiprotocol flow |  |
| O | Packet Classification Rule parameter | variable |  | Present if needed |
| … | … | … | … | … |
| O | O) IEEE 802.3/Ethernet Destination MAC Address parameter | 12 | dst.msk | Present if needed |
| O | P) IEEE 802.3/Ethernet Source MAC Address parameter | 12 | src, msk | Present if needed |
| O | Q) Ethertype/IEEE 802.2 SAP | 3 | type, eprot1, eprot2 | Present if needed |
| O | R) IEEE 802.1D User Priority field | 2 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high | Present if needed |
| O | S) IEEE 802.1Q VLAN ID field | 2 | vlan_id1, vlan_id2 | Present if needed |

------------------------------- Change #4  End  ---------------------------------------------------

------------------------------- Change #5  Start  ---------------------------------------------------

[Remedy 5:]

[change table 740 (page 223, line 46) as the following:]

Table 737—AAI-DSC-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | CS parameter encoding rules | 4 | 99 Reserved 100 Packet, IPv4 101 Packet, IPv6 102 Reserved 103 Reserved 104 Reserved 105 Reserved 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 Reserved 113 Packet IP 114 Packet, Multiprotocol flow |  |
| O | Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier | Present for AAI-DSC-REQ |
| O | Packet Classification Rule parameter | variable |  | Present if needed |
| … | … | … | … | … |
| O | O) IEEE 802.3/Ethernet Destination MAC Address parameter | 12 | dst.msk | Present if needed |
| O | P) IEEE 802.3/Ethernet Source MAC Address parameter | 12 | src, msk | Present if needed |
| O | Q) Ethertype/IEEE 802.2 SAP | 3 | type, eprot1, eprot2 | Present if needed |
| O | R) IEEE 802.1D User Priority field | 2 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high | Present if needed |
| O | S) IEEE 802.1Q VLAN ID field | 2 | vlan_id1, vlan_id2 | Present if needed |

------------------------------- Change #5  End  ---------------------------------------------------