| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Multi-carrier HO case (16.2.8.2.9.2.2) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7188 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D8 for Sponsor Ballot |  |
| Abstract | This contribution is to propose text changes for Section 16.2.8.2.9.2.2 |  |
| Purpose | Update P802.16m/D8 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing Multi-carrier HO case (16.2.8.2.9.2.2)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution clarifies a missing Multi-carrier HO with AMS having only one RF.

- Rationale/Motivation of modifications

Currently, the Multi-carrier HO assumes AMS have more than one RF to allow maintain connection with serving ABS while performing network re-entry at target ABS.

However, AMS may be Multi-carrier capable with a single RF also. It may perform carrier aggregation. Hence, in such case, the AMS will not be able to maintain connection with serving ABS while performing network re-entry at target ABS during MC-HO. Since the current specification does not cover the case where HO_Reentry_Interleaving_Interval > 0 for Multi-carrier capable AMS with single RF it should be added.

As aforementioned, the missing MC-HO case for Multi-carrier capable AMS with single RF performing EBB HO should be described.

- References

[1] IEEE P802.16m/D8, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D8, page 341, line 41, add proposed text in section 16.2.8.2.9.2 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.8.2.9.2 Multicarrier handover (MCHO) procedure

The multicarrier handover (MCHO) is defined as the handover procedure which involves multiple radio carriers, which includes multi-carrier EBB HO for AMS with and without Multi-carrier EBB HO capability. and HO with secondary carrier pre-assignment as described in this section. For an AMS supporting basic MC mode handover from one carrier to another carrier of the same ABS, the handover procedure follows the primary carrier change procedure per 16.2.8.2.11.2.

--------------------------------------------------------Text End -----------------------------------------------------

Remedy

On the 80216m_D8, page 342, line 5, add proposed text in section 16.2.8.2.9.2.2 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.8.2.9.2.2 MCHO execution and network reentry with HO_Reentry_Mode=1

The AMS with multicarrier capability follows the network reentry procedure per section 16.2.6.3.5.

When HO_Reentry_Mode is set to 1 and HO_Reentry_Interleaving_Interval is set to 0, For AMS with Multi-carrier EBB HO capability indicated in AAI_REG-REQ, the AMS performs network reentry to the target ABS on one carrier and maintains normal communication with the serving ABS on another carrier not performing network reentry procedure. The AMS may use the original primary carrier for network reentry to the target ABS, as illustrated in Figure 419. It may also use another carrier different from its original primary carrier for network reentry procedures, as illustrated in Figure 420. In the other hand, for AMS, without Multi-carrier EBB HO capability indicated in AAI_REG-REQ, the HO_Reentry_Interleaving_Interval is set to greater than zero. In this case, the AMS follows the same HO procedure for single carrier AMS with HO_Reentry_Mode 1 described in section 16.2.6.3.5.2.

In this case, Disconnect_time should be long enough that network reentry procedure to target ABS can be completed prior to the expiration of Disconnect_time.

In case of AAI_HO-CMD message with multiple target ABS and carriers, the physical carrier index of each candidate carrier provided by each target ABS should also be indicated in the AAI_HO-CMD message. The AMS should follow the order of the candidate ABSs listed in the AAI_HO-CMD message to make decision on HO target if the CINR/RSSI values of the ABSs are good enough to support the current services of the AMS. The AMS may inform the serving ABS through AAI_HO-IND the carrier to be used for network reentry operations to avoid resource allocation by the serving ABS on that carrier.

--------------------------------------------------------Text End -----------------------------------------------------

Remedy #2

On the 80216m_D8, page 106, line 56, add proposed table in section 16.2.3.8 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.8 AAI-REG-REQ

An AAI-REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

The following parameters may be included in AMS capability negotiation parameters of AAI-REG-REQ.

Table 686—AAI-REG-REQ message Field Descriptions

| M/O | Attributes/Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | C) Multicarrier capabilities | 3 | 0b000: No MC modes 0b001: Basic MC mode 0b010: Multicarrier Aggregation 0b011: Multicarrier Switching 0b100: Both Multicarrier Aggregation and Multicarrier Switching | Present if needed |
|  | C.1) Multi-carrier EBB HO capability | 1 | 0: Indicates that multi-carrier capable AMS is not able to perform Multi-carrier EBB HO but follows Single-carrier HO (i.e. Combination capabilities of EBB support 0b1 & HO_Reentry_Interleaving_Interval > 0) 1: Indicates that multi-carrier capable AMS is able to perform Multi-carrier EBB HO (i.e. Combination capabilities of EBB support 0b1 & HO_Reentry_Interleaving_Interval=0) | Present if Multi-carrier capable |
| … | … | … | … | … |
| O | P) Capabilities for Handover EBB mode |  |  | Present if needed |
| O | P.1) Minimal HO Reentry Interleaving Interval | 2 | The minimal HO_Reentry_Interleaving_Interval measured in frames. For AMS capable of Carrier Aggregation (Multicarrier Capability = 0b010 or 0b100), this value shall be 0. Shall be included only if EBB support is set to 1 | Present if needed |

--------------------------------------------------------Text End -----------------------------------------------------