| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ Parameters modification in REG-REQ/RSP (Section 16.2.3.8) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Youngbin Chang, Anil Agiwal Samsung Electronics | yb.chang@samsung.com, anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | Sponsor Ballot Recirc # 2, P802.16m |  |
| Abstract | This contribution proposes text on control message on ARQ |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ Parameters modification in REG-REQ/RSP

Youngbin Chang, Anil Agiwal

Samsung Electronics

# Introduction

In the REG-REQ/RSP, only capability parameters on ARQ are necessary. Others should be negotiated during DSA-REQ/RSP.

- ARQ_SN_MODULUS is determined value = 2^10

- ARQ_WINDOW_SIZE, ARQ_SUB_BLOCK_SIZE, ARQ_BLOCK_LIFETIME, ARQ_RX_PURGE_TIMEOUT are dependant values on connection (flow ID).

- MAXIMUM_ARQ_BUFFER_SIZE shall be defined as MAXIMUM_NON_ARQ_BUFFER_SIZE.

# Proposed text

[Change1: Modify the following parameters in the table 686 on page 107:]

Table 686—AAI-REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | AMS MAC address | 48 | AMS’s real MAC address |  |
| O | AMS capability negotiation parameters | Variable |  | Present if needed |
| O | A) ARQ parameters | Variable |  | Present if needed |
| O | A.1) ARQ_SN_MODULUS | 10 | A.1) ARQ_SN_MODULUS | Present if needed |
| O | A.2) ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks | Present if needed |
| O | A.3) ARQ_SUB_BLOCK_SIZE | 10 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.4) ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.5) ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | Present if needed |
| O | A.61) MAXIMUM_ARQ_BUFFER_SIZE | 1623 | The maximum ARQ buffer size allowed by the system. The actual maximum ARQ buffer size depend on AMS's memory, and shall be below 2048 bytes. ARQ buffer size depending on AMS's memory. Buffer size is defined as 2P (0<=P<=23). Unit is byte. | Present if needed |
| O | B) Non-ARQ parameters |  |  |  |
| … | … | … | … | … |

[Change2: Delete the following parameters in the table 687 on page 112:]

Table 687—AAI-REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode |  |
| O | A) AMS capability negotiation parameters |  |  |  |
| O | A.1) ARQ parameters |  |  |  |
| O | A.1.1) ARQ_SN_MODULUS | 10 | the number of unique SN values |  |
| O | A.1.2) ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O | A.1.3) ARQ_SUB_BLOCK_SIZE | 10 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O | A.1.4) ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O | A.1.5) ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
|  | A.2) Femto ABS LDM parameters |  |  |  |
| … | … | … | … | … |