| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI_PKM-REQ and AAI_PKM-RSP messages |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Youngkyo Baek, Hyunjeong Kang Samsung Electronics | youngkyo.baek@samsung.com |
| Re: | P802.16m/D8 |  |
| Abstract | Suggested Table Format for security MAC Control Message (AAI_PKM-REQ and AAI_PKM-RSP) |  |
| Purpose | For TGm discussion and adoption into P802.16m/D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI_PKM-REQ and AAI_PKM-RSP messages

Youngkyo Baek, Hyunjeong Kang

Samsung Electronics

# Introduction

This contribution provides updated table format for the AAI_PKM-REQ and AAI_PKM-RSP messages.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

# Proposed Remedy

Adopt the following two remedies.

{Remedy #1: Replace Table 723 in pp. 192 with the following Table}

Table 723—AAI_PKM-REQ message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 Reauth-Request; PKM v3 message code =1 - PKMv3 EAP-Transfer; PKM v3 message code =2 -PKMv3 Key_Agreement-MSG#2; PKM v3 message code =4 - PKMv3 TEK-Request; PKM v3 message code =6 - PKMv3 TEK-Invalid; PKM v3 message code =8 9-16 : reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests |  |
| If( PKM v3 message code = =2) { |  |  |  |
| EAP Payload | Variable (1..1400 X8) | Contains the EAP authentication data, not interpreted in the MAC |  |
| } |  |  |  |
| If( PKM v3 message code ==4) { |  |  |  |
| NONCE_ABS | 64 | A random number of 64 bits used for freshness |  |
| NONCE_AMS | 64 | A random number of 64 bits used for freshness |  |
| AK ID | 64 | AK ID = Dot16KDF(AK, 0b0000¦PMK SN¦AMSID* or MS MAC address¦ABSID ¦ "AKID", 64), where either AMSID* or MS MAC address is used depending on the used AK derivation formula . This is used to verify sync of PMK SN and the corresponding AK |  |
| size of ICV | 1 | 0 : size of ICV = 32bits (default; Max Invalid value is 4096) 1 : size of ICV = 64bits (Max Invalid value is not used) | Present when it is used during network entry |
| PN window Size | 16 | The receiver shall track PNs within this window to prevent replay attacks | Present when it is used during network entry |
| } |  |  |  |
| If( PKM v3 message code ==6 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| TEK refresh flag | 1 | This flag is set to "1" in the signal this request is for the first TEK after reauthentication completion when both TEKs need to be updated one after another and set to "0" when this request is for the second TEK update after the first TEK update is done. | Present when TEK update after reauthentication |
| } |  |  |  |
| If( PKM v3 message code == 8) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| } |  |  |  |
| If( PKM v3 message code ==1,4,6 or 8 ) { |  |  |  |
| PMK SN | 4 | New PMK sequence number for PKM v3 message code = 4 Current PMK sequence number for PKM v3 message codes except 4 |  |
| CMAC_PN_U | 24 | PN used to avoid UL replay attack on the control connection |  |
| CMAC value | 64 | CMAC value calculated using CMAC_KEY_U based on new PMK SN for PKM v3 message code = 4 CMAC value calculated using CMAC_KEY_U based on new PMK SN for PKM v3 message code except 4 |  |
| } |  |  |  |

{Remedy #2: Replace Table 724 in pp. 194 with the following Table}

Table 724—AAI_PKM-RSP message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| PKM v3 message type code | 4 | - PKMv3 EAP-Transfer; PKM v3 message code =2 - PKMv3 Key_Agreement-MSG#1; PKM v3 message code =3 - PKMv3 Key_Agreement-MSG#3; PKM v3 message code =5 - PKMv3 TEK-Reply; PKM v3 message code =7 - PKMv3 TEK-Invalid; PKM v3 message code =8 9-16 : reserved |  |
| PKM identifier | 8 | A value used to match an ABS response to the AMS requests or an AMS response to the ABS requests |  |
| If( PKM v3 message code ==2 ) { |  |  |  |
| EAP Payload | Variable (1..1400 X8) | Contains the EAP authentication data, not interpreted in the MAC |  |
| } |  |  |  |
| If( PKM v3 message code ==3 ) { |  |  |  |
| NONCE_ABS | 64 | A random number of 64 bits used for freshness |  |
| AK ID | 64 | AK ID = Dot16KDF(AK, 0b0000¦PMK SN¦AMSID* or MS MAC address¦ABSID ¦ "AKID", 64), where either AMSID* or MS MAC address is used depending on the used AK derivation formula . This is used to verify sync of PMK SN and the corresponding AK |  |
| Key lifetime | 32 | MSK lifetime, this attribute is included only in case of key agreement following EAP-based authorization or EAP-based reauthorization procedures. |  |
| } |  |  |  |
| If( PKM v3 message code ==5 ) { |  |  |  |
| NONCE_ABS | 64 | A random number of 64 bits used for freshness |  |
| NONCE_AMS | 64 | A random number of 64 bits used for freshness |  |
| size of ICV | 1 | 0 : size of ICV = 32bits (default; Max Invalid value is 4096) 1 : size of ICV = 64bits (Max Invalid value is not used) | Present when it is used during network entry |
| PN window Size | 16 | The receiver shall track PNs within this window to prevent replay attacks | Present when it is used during network entry |
| Supporting SAs | 3 | SAs are supported for transport connections as following bitmap; Bit#0 : Null SA (SAID=0x00) is supported if it is set to 1 Bit#1 : SAID 0x01 is supported if it is set to 1 Bit#2 : SAID 0x02 is supported if it is set to 1 | Present when it is used during network entry |
| } |  |  |  |
| If( PKM v3 message code ==7 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| COUNTER_TEK | 16 | COUNTER_TEK used for deriving current uplink TEK |  |
| EKS | 2 | Encryption key sequence number for current uplink TEK |  |
| } |  |  |  |
| If( PKM v3 message code ==8 ) { |  |  |  |
| SAID | 8 | Security association identifier |  |
| } |  |  |  |
| If( PKM v3 message code ==3,5,7 or 8 ) { |  |  |  |
| PMK SN | 4 | New PMK sequence number for PKM v3 message code = 3 or 5 Current PMK sequence number for PKM v3 message codes except 3 and 5 |  |
| CMAC_PN_D | 24 | PN used to avoid DL replay attack on the control connection |  |
| CMAC value | 64 | CMAC value calculated using CMAC_KEY_D based on new PMK SN for PKM v3 message code = 3 or 5 CMAC value calculated using CMAC_KEY_D based on new PMK SN for PKM v3 message code except 3 and 5 |  |
| } |  |  |  |

---------------------------------------------------------Start of the Text-----------------------------------------------------------