| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods to Identify Idle Mode MSs in IEEE 802.16m (16.2.18) |  |
| Date Submitted | 2010-09-15 |  |
| Source(s) | Shantidev Mohanty, Shaocheng Wang, Muthaiah Venkatachalam Intel Corporation Ming-Hung Tao, Yung-Han Chen, Ying-Chuan Hsiao ITRI Yang Liu, Lei Zhang ZTE Corporation Phillip Barber Huawei Kiseon Ryu LG Electronics Yi-Ting Lin III Jaesun Cha ETRI Hyunjeong Kang Samsung Electronics | E-mail: shantidev.mohanty@intel.com |
| Re: | Category: D8 / Area: Chapter 16.2.18 (DID) |  |
| Abstract | This contribution proposes methods using which AMS in idle mode can determine paging parameters in the Legacy operation mode of IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods to Identify Idle Mode MSs in IEEE 802.16m (16.2.18)

Shantidev Mohanty, Shaocheng Wang, Muthaiah Venkatachalam

Intel Corporation

Ming-Hung Tao, Yung-Han Chen, Ying-Chuan Hsiao

ITRI

Yang Liu, Lei Zhang

ZTE Corporation

Phillip Barber

Huawei

Kiseon Ryu

LG Electronics

Yi-Ting Lin

III

Jaesun Cha

ETRI

Hyunjeong Kang

Samsung Electronics

- Introduction

In IEEE 802.16m based networks, idle mode MSs are identified using the Deregistration Identifier (DID), their paging cycle and paging offsets. Thus, idle mode MSs that belong to same paging group and have same paging cycle and paging offset have unique DID so that they can be identified uniquely.

The DIDs are assigned to idle mode MS by the paging controllers (PCs). One or more PCs manage each paging group. Thus, when two different PCs assign the DIDs to different idle mode MSs of the same paging group, there is a possibility that they assign the same DID to two different MSs. This is because the DID assignment of each PC is independent of the other ones. If both the MS that have the same DID also have the same paging cycle and paging offset, then these MSs have the same identification. This leads to false paging message indication as the paging message for one of these MSs also results in unwanted paging indication. This results in unwanted paging operation and unnecessary signaling overhead.

- Proposed Text for IEEE P802.16m/D8

============================ Start of Proposed Text ==================================

[Editor’s Note: Add the following text after line 36 in page 55 in “16.2.1.2.3 Deregistration Identifier (DID)” as shown below]

The network shall assign a 12-bit DID to each AMS during Idle Mode initiation. The network may assign a new DID to an AMS during location update procedure. The DID shall uniquely identify the Idle Mode AMS within the set of paging group ID, paging cycle and paging offset. The AMS shall be assigned a randomly selected DID from the available DID space.

[Editor’s Note: Add the following text after line 31 in page 431 in “16.2.18 Idle mode” as shown below]

An AMS is assigned during deregistration or location update, to one or more paging groups and, per paging group, a specific paging cycle and paging offset. The values of paging cycle and paging offset can be different among AMSs assigned to same paging group. The assignment of paging offset shall be performed in such a way that the paging offset of different idle mode AMSs is pseudo-randomized.

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Apr 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.