| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Clarification on Uplink Data and Pilot Subcarrier Power (Table 962) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Rongzhen Yang, Alexei Davydov Intel | rongzhen.yang@intel.com |
| Re: | Comments on P802.16m/D8 for SB |  |
| Abstract | The contribution provides the proposed modification of table 962 to clarify the relationship between it and uplink power control section. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clarification on Uplink Data and Pilot Subcarrier Power (Table 962)

Rongzhen Yang, Alexei Davydov

Intel Corp

# Introduction

During the uplink power control processing defined in 16.3.8.4, the uplink power per stream will be controlled for normalization by parameter “beta”, however, in the original Table 962 – Uplink data and pilot subcarriers power, the value of “Bd” is defined as 1/Mt for normalization, i.e, if “beta” is defined as 1, there is misunderstanding for duplicated normalization of power.

Therefore, it is suggested to change “Bd” as 1 (unitary power) in table 962, to avoid the potential misunderstanding of this table.

# References

[1] Draft Amendment, P802.16m/D8

# Text proposal for the 802.16m amendment

Instruction to editor;

Black text: existing D2 text

Blue and strike out text: for deletion

Red and underline text: for addition

============================== Start of Proposed Text =================================

Table 962 – Uplink data and pilot subcarriers power

| number of streams Mt | per stream power on data subcarriers Bd | per stream pilot power boosting value Bp | per stream pilot boosting level, dB |
| --- | --- | --- | --- |
| 1 | 1/ Mt 1 | 1.5849 * Mt | 2+10*log10(Mt) |
| 2-4 |  | 1 Mt | 10*log10(Mt) |
| 3-4 |  | 1 | 10*log10(Mt) |

============================== End of Proposed Text =================================