| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover with CLC classes |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | Hanan Ahmed Huawei Technologies [Address Line 1 (optional)] [Address Line 2 (optional)] | Voice: [Telephone Number (optional)]] E-mail: ahanan@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | After handover, the CLC active cycle and interval parameters shall remain the same if the CLC class meets the CLC limits of the new S-ABS |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Handover with CLC Classes

Hanan Ahmed

Huawei

# Introduction

The current standard text mandates that the new serving ABS supports CLC with the same parameters without verifying if the CLC classes are within the CLC limits of that new ABS. The text also does not address the case when the new S-ABS does not obtain information on active CLC classes from the S-ABS during handover.

# Proposed Changes

Change 1:

On page 447, starting on line 61, modify the existing text by including the red text:

The AMS shall locally terminate the active Type III CLC class at the Disconnect Time or upon transmission of AAI-HO-IND with code 0b10. The AMS shall also locally suspend all active Type I and II CLC classes at the Disconnect Time or upon transmission of AAI-HO-IND with code 0b10, and reactivate them with the new S-ABS. During HO preparation, the new S-ABS may obtain information on active CLC classes from the S-ABS via backbone network. In case the new S-ABS does not get the information then the AMS may request the CLC class with the new S-ABS by sending AAI_CLC_REQ message. The CLC active cycle and interval parameters shall remain the same after handover if the CLC class meets the CLC limits of the new S-ABS. The new S-ABS may set the Super Frame Number of the start time to the suggested value in CLC Request from the AMS in case of an uncoordinated handover, or to the beginning of the next CLC active cycle based on the information obtained from the previous S-ABS via the backbone network.