| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-SCN-REQ, AAI-SCN-RSP and AAI-SCN-REP (16.2.3.14) |  |
| Date Submitted | 2010-9-13 |  |
| Source(s) | Jaehyuk Jang Samsung Electronics | jack.jang@samsung.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clarification on the tables for MAC messages related to scanning operations |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-SCN-REQ, AAI-SCN-RSP and AAI-SCN-REP (16.2.3.14)

Jaehyuk Jang

Samsung Electronics

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Updated from C802.16m-10/1118 (comment #38)

- To use Nbr_Bitmap_Index bitmap, Cell type also needs to be added. (copied from NBR-ADV)

- The size of Nbr_Bitmap_Index is changed to 256 (2^8  8 is from the size of ABS Index in AAI-NBR-ADV).

- The size of Req_Bitmap_Index is changed to 64 (2^6  6 is from the size of N_Recommended_ABS_Full).

Proposed Text

Remedy 1:

In the P802.16m/D8, page 132, line 50, replace Table 693 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 693—AAI-SCN-REQ parameters

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Interleaving interval | 8 | The period of AMS's normal operation (in units of AAI subframes), which is interleaved between Scan Durations. |  |
| Scan Iteration | 6 | The requested number of iterating scanning interval by an AMS. |  |
| Recommended start super frame number | 6 | Represents the 6 least significant bits of the absolute super frame index for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value. | Present if AMS recommends a start super frame |
| Recommended start frame index | 2 | Recommended start frame number within a super frame. 0b00: the 1st frame in a super frame 0b01: the 2nd frame in a super frame 0b10: the 3rd frame in a super frame 0b11: the 4th frame in a super frame |  |
| N_Recommended_ABS_Index | 6 | This is the number of neighboring ABS the AMS plans to scan, which are included in AAI-NBR-ADV message [0…63]. When an AMS receives AAI-SCN-RSP message from ABS in response to AAI-SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI-SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI-SCN-REQ message to the ABS. | Present if AMS uses index instead of full 48-bits BSID to identify an ABS it plans to scan |
| If(N_Recommended_ABS_Index > 0) { | - |  |  |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101: R1 BS or LZone of ABS 0b110-0b111: reserved |  |
| AAI-NBR-ADV Segment Index | 4 | Indicates current segment index of AAI-NBR-ADV in the specific cell type used by the AMS |  |
| Nbr_Bitmap_Index | 256 | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested |  |
| For(i=0; i< N_Recommended_ABS_Index; i++) { | - |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers the AMS plans to scan at each neighbor ABS [0…63]. | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { | - |  |  |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS |  |
| } |  |  |  |
| } |  |  |  |
| }//end of If(N_Recommended_ABS_Index > 0) |  |  |  |
| N_Recommended_ABS_Full | 6 | Number of neighbor ABS the AMS plans to scan which are using full 48-bit ABSID [0…63]. | Present if AMS decides to use full 48 bits BSID to identify an ABS |
| For(i=0; i< N_Recommended_ABS_Full; i++) { | - |  |  |
| Recommended ABS ID | 48 | ABS ID of the ABSs the AMS plans to scan |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers the AMS plans to scan at each neighbor ABS [0…63]. | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { | - |  |  |
| Recommended Carrier Index | 6 | Requested physical carrier index for scan at each neighbor ABS |  |
| } |  |  |  |
| } |  |  |  |
| N_Recommended_SA_Preamble_Index | 4 | Number of SA preambles the AMS plans to scan [0..15] | Present if AMS requests to scan SA preambles |
| For(i=0; i< N_Recommended_SA_Preamble_Index; i++) { | - |  |  |
| SA Preamble Index | 6 | SA Preamble indices the AMS plans to scan |  |
| } |  |  |  |
| N_Recommended_Carrier_Index_at_Serving_ABS | 6 | Number of carriers the AMS plans to scan at each neighbor ABS. | Present if AMS decides to scan other carriers of the serving ABS |
| For(i=0; i< N_Recommended_Carrier_Index_at_Serving_ABS; i++) { | - |  |  |
| Recommended Carrier index at S-ABS | 6 | Recommended physical carrier index of the S-ABS the AMS plans to scan |  |
| } |  |  |  |
| for (i=0; i<N_CSG_ID_Infos; i++) { |  | N_CSG_ID_Infos is the number of CSG ID Information blocks [0…15]. | Optional for loop. May be included for quick CSG membership detection or ABS reselection assistance |
| Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG Femtocell | Present if the Operator ID is different from the one of the ABS |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs belongs to this Operator ID. |  |
| CSGID | var | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 135, line 32, replace Table 694 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 694—AAI-SCN-RSP parameters

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Scan duration | 8 | Duration (in units of AAI subframes) of the requested scanning period. |  |
| Report mode | 2 | 0b00:No report 0b01:Periodic report 0b10:Event-triggered report 0b11:One-time scan report |  |
| Report metric | 4 | Bitmap indicator of trigger metrics that the S-ABS requests the AMS to report. The S-ABS shall indicate only the trigger metrics agreed during AAI-SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric: Bit 0: ABS CINR mean Bit 1: ABS RSSI mean Bit 2: Relative delay Bit 3: ABS RTD |  |
| Report period | 8 | The period of AMS's report of scanning result when the AMS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the ABS sends an unsolicited AAI-SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the AMS receives the AAI-SCN-RSP message. For AMS scanning request denied by AAI-SCN-RSP with Scan Duration set to zero, Report period is the number of frames that ABS suggests to AMS before transmitting next AAI-SCN-REQ |  |
| Duration for measurements signal location parameters on D-LBS zone | 4 | Duration (in units of 4 superframes) of the requested scanning. | Present if ABS request AMS to scan D-LBS zone for signal location parameters measurements |
| If(Scan Duration > 0) { |  |  |  |
| Start superframe index | 6 | Represents the 6 least significant bits of the absolute superframe index in which the first Scanning interval starts |  |
| Start frame index | 2 | Represents start frame number in the Start super frame number. 0b00: 1st frame in the super frame 0b01: 2nd frame in the super frame 0b10: 3rd frame in the super frame 0b11: 4th frame in the super frame |  |
| Interleaving interval | 8 | The period of AMS's normal operation (in units of AAI subframes), which is interleaved between Scan Durations. |  |
| Scan Iteration | 6 | The requested number of iterating scanning interval by an AMS. |  |
| N_Recommended_ABS_Full | 6 | Number of neighbor ABS [0…63] to be scanned, which are using full 48-bit BSID. | Present only this message is sent in response to AAI-SCN-REQ with N_Recommended_ABS_ Full_indication > 0 |
| If(N_Recommended_ABS_Full > 0) { | - |  |  |
| Req_Bitmap_Index | 64 | Each bit position in this bitmap corresponds to a ABS Index of the corresponding AAI-SCN-REQ message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index, the most significant bit corresponds to the ABS Index of the last recommended ABS, and ABSs with ABS Index greater than the last recommended BS are not recommended and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not recommended. 1: the corresponding ABS is recommended. |  |
| For(i=0; i< N_Recommended_ABS_Full; i++) { | - |  |  |
| Recommended ABS ID | 48 | ABS ID of the ABSs to be scanned |  |
| N_Recommended_Carrier_Index | 6 | Number of carriers to be scanned at each neighbor ABS [0..63] |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { | - |  |  |
| Recommended_Carrier_Index | 6 | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-Config message |  |
| SA Preamble Index | 10 | SA Preamble Index to be scanned |  |
| CP length | 2 | CP length of the recommended T-ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when neighbor ABS is not included in AAI-NBR-ADV message and AAI-SCN-RSP is not sent as response to AAI-SCN-REQ which included this recommended carrier index |
| } | - |  |  |
| } | - |  |  |
| }//end of if(N_Recommended_ABS_Full > 0) | - |  |  |
| }//end of if(Scan Duration > 0) | - |  |  |
| N_Recommended_ABS_Index | 6 | This is the number of neighbor ABS [0…63] to be scanned, which are included in AAI-NBR-ADV message. When an AMS receives AAI-SCN-RSP message from ABS in response to AAI-SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI-SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI-SCN-REQ message to the ABS. | Present if ABS uses index to identify an ABSs to be scanned |
| If(N_Recommended_ABS_Index > 0) { | - |  |  |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101: R1 BS or LZone of ABS 0b110-0b111: reserved |  |
| AAI-NBR-ADV Segment Index | 4 | Indicates current segment index of AAI-NBR-ADV in the specific cell type used by the AMS |  |
| Nbr_Bitmap_Index | 256 | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested |  |
| For(i=0; i< N_Recommended_ABS_Index; i++) { | - |  |  |
| Neighbor ABS index | - |  |  |
| N_Recommended_Carrier_Index | 8 | ABS index corresponds to the position of ABS to be scanned in AAI-NBR-ADV message |  |
| For(j=0; j< N_Recommended_Carrier_Index; j++) { | 6 | Number of carriers to be scanned at each neighbor ABS [0..63] |  |
| Recommended Carrier Index | - |  |  |
| } | 6 | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-Config message |  |
| } | - |  |  |
| }//end of if(N_Recommended_ABS_Index > 0) | - |  |  |
| N_Recommended_Carrier_Index_at_Serving_ABS | 6 | Number of carriers to be scanned at the S-ABS. | Present if AMS decides to scan other carriers of the serving ABS |
| For(i=0; i< N_Recommended_Carrier_Index_at_Serving_ABS; i++) { | - |  |  |
| Recommended Carrier index at S-ABS | 6 | Recommended physical carrier index of the S-ABS the AMS plans to scan |  |
| } |  |  |  |
| Scanning_Carrier_Index | 6 | Recommended physical carrier index of the AMS to be used for scanning | Present if the AMS supports multi-carrier scanning. |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D8, page 139, line 44, replace Table 695 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 695—AAI-SCN-REP parameters

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Report mode | 2 | Action code for an AMS's scan report of its measurement 00: Event-triggered report 01: Periodic report according to Scan report period of AAI-SCN-RSP 10: One-time scan report |  |
| For(i=0; i<N-Carriers-SABS; i++) { | - | N-Carriers-SABS is the number of carriers at S-ABS to be reported in this message | Present if AMS reports measurement results measured from carriers at S-ABS |
| Carrier Index | 6 | Carrier Index of carrier at S-ABS |  |
| S-ABS CINR mean | 8 | The S-ABS CINR Mean parameter indicates the CINR measured by the AMS from the S-ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| S-ABS RSSI mean | 8 | The S-ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the S-ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An AMS shall be able to report values in the -103.75 dBm to -40 dBm. An AMS with multiple receive antennas will report the linear sum of the RSSI values calculated over all the receive antennas. | Present if AMS is asked to report RSSI |
| S-ABS RTD | 8 | The S-ABS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the S-ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs. | Present if AMS is asked to report RTD |
| } |  |  |  |
| N_Reported_ABS_Index | 6 | Number of neighbor ABS reported in this message and which are included in AAI-NBR-ADV message [0...63]. | Present only when the scanning report includes ABSs referred by ABS index |
| If(N_Reported_ABS_Index > 0) { | - |  |  |
| Configuration Change Count of AAI-NBR-ADV | 3 | The value of Configuration Change Count in AAI-NBR-ADV message used for neighbor ABS index references |  |
| Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101: R1 BS or LZone of ABS 0b110-0b111: reserved |  |
| AAI-NBR-ADV Segment Index | 4 | Indicates current segment index of AAI-NBR-ADV in the specific cell type used by the AMS |  |
| Nbr_Bitmap_Index | 8 | Each bit position corresponds to a ABS Index of the corresponding AAI-NBR-ADV message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index in sequential order, the most significant bit corresponds to the ABS Index of the last requested ABS, and ABSs with ABS Index greater than the last requested ABS are not requested and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not requested. 1: the corresponding ABS is requested |  |
| For(i=0; i< N_Reported_ABS_Index; i++) { | - |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to the position of ABS in AAI-NBR-ADV message. |  |
| N_Reported_Carrier_Index | 6 | Number of carriers the AMS reported at each neighbor ABS [0…63]. | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Reported_Carrier_Index; j++) { | - |  |  |
| Recommended Carrier Index | 6 | Reported physical carrier index at each neighbor ABS |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| Relative delay | 8 | This parameter indicates the delay of neighbor DL signals relative to the S-ABS, as measured by the MS for this carrier. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } | - |  |  |
| } | - |  |  |
| }//end of if(N_Reported_ABS_Index > 0) | - |  |  |
| N_Reported_ABS_Full | 6 | Number of neighbor ABS [0..63] reported in this message, which are using full 48-bit BSID. | Present only when the scanning report includes ABSs referred by ABS ID |
| If (N_Reported_ABS_Full > 0) { |  |  |  |
| Rsp_Bitmap_Index | 64 | Index of the corresponding AAI-SCN-RSP message, where the least significant bit corresponds to the first ABS Index, each next significant bit corresponds to the next ABS Index, the most significant bit corresponds to the ABS Index of the last reported ABS, and ABSs with ABS Index greater than the last reported ABS are not reported and do not have a corresponding bit position in the bitmap. Bitmap position bit value: 0: the corresponding ABS is not reported. 1: the corresponding ABS is reported. |  |
| For(i=0; i< N_Reported_ABS_Full; i++) { | - |  |  |
| Reported ABS ID | 48 | ABS-IDs of the ABSs reported in this message |  |
| N_Reported_Carrier_Index | 6 | Number of carriers the AMS reported at each neighbor ABS [0…63]. | Present when a neighbor ABS is a multicarrier ABS |
| For(j=0; j< N_Reported_Carrier_Index; j++) { | - |  |  |
| Recommended Carrier Index | 6 | Reported physical carrier index at each neighbor ABS |  |
| CINR mean | 8 | The ABS CINR Mean parameter indicates the CINR measured by the AMS from this carrier. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | The ABS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from this carrier. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| Relative delay | 8 | This parameter indicates the delay of neighbor DL signals relative to the S-ABS, as measured by the MS for this carrier. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } | - |  |  |
| } |  |  |  |
| }//end of If (N_Reported_ABS_Full > 0) |  |  |  |
| N_Reported_SA_Preamble_Index | 4 | Number of SA preambles reported in this message [0..15] | Present only when the scanning report includes SA preamble reports |
| For(i=0; i< N_Reported_SA_Preamble_Index; i++) { | - |  |  |
| SA Preamble Index | 10 | SA preamble indices reported in this message |  |
| CINR mean | 8 | Indicates the CINR measured by the AMS from this SA preamble. The value shall be interpreted as a signed byte with units of 0.5 dB. | Present if AMS is asked to report CINR |
| RSSI mean | 8 | Indicates the Received Signal Strength measured by the AMS from this SA preamble. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. | Present if AMS is asked to report RSSI |
| Relative delay | 8 | Indicates the delay of SA preamble DL signals relative to the S-ABS, as measured by the MS for this SA preamble. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| } |  |  |  |
| Neighbor_Request_Indication | 1 | Request indication for system information and the list of the neighbor Femto ABSs | Optional |
| If(Neighbor_Request_Indication = 1) { |  |  |  |
| Neighbor_Request_BS_Type | 3 | Indicates type of neighbor Femto ABSs for which system information is requested. Bit #0: CSG-Closed Femto ABS Bit #1: CSG-Open Femto ABS Bit #2: OSG Femto ABS | Optional |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs | Optional. Sent by AMS to aid in generation of optimized neighbor list by the ABS |
| CSGID | var | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------