| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clean up the AAI_E-MBS-CFG message (16.9.3.1) |  |
| Date Submitted | 2010-09-13 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com |
| Re: | Sponsor Ballot comment on IEEE p802.16m D8 |  |
| Abstract | This contribution proposes the texts related to AAI_E-MBS-CFG. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clean up the AAI_E-MBS-CFG message (16.9.3.1)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This is a reply comment for comment #10220 to move the AAI_E-MBS-CFG message to MAC control message section 16.2.3.

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Suggested Remedy #1: Create the Section 16.2.3.62 AAI_E-MBS-CFG message at the end of Section 16.2.3 and move all texts, figures, tables from the second paragraph in 16.9.3.1 to the section 16.2.3.62 as follows :]

16.2.3.62 AAI_E-MBS-CFG message

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and E-MBS ID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI-E-MBS-CFG shall be advertised at the superframe whose number (Nsuperframe) from SFH meets the following condition.

Nsuperframe modulo 16 == 15

In the case of AAI-SCD update that includes any updated E-MBS configuration parameter, the updated E-MBS resource configuration parameters shall be applied at the beginning of the first MSI in next AAI-E-MBS_CFG life time cycle.

…

[Suggested Remedy #2: Modify the first paragraph in the section 16.2.9.3.1 as follows and delete all texts, figures, tables from the second paragraph in 16.9.3.1:]

16.9.3.1 E-MBS Configuration Indicators

The information regarding E-MBS configuration is transmitted periodically to the AMSs interested in E-MBS using a MAC control message called the AAI-E-MBS_CFG message. The E-MBS configuration indicators specify the resources reserved for E-MBS traffic in the downlink physical resources and additional information necessary for E-MBS operation. The details of E-MBS Configuration Indicators are described in Section 16.2.3.62 AAI_E-MBS-CFG message.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and E-MBS ID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI-E-MBS-CFG shall be advertised at the superframe whose number (Nsuperframe) from SFH meets the following condition.

Nsuperframe modulo 16 == 15

In the case of AAI-SCD update that includes any updated E-MBS configuration parameter, the updated E-MBS resource configuration parameters shall be applied at the beginning of the first MSI in next AAI-E-MBS_CFG life time cycle.

…

16.9.3.2 E-MBS Zone Configuration

Different E-MBS IDs and FIDs may be used in different E-MBS Zones for the same multicast and broadcast service flow. A multicast and broadcast zone identifier (E-MBS_ZONE_ID) is used to indicate a service area in which a E-MBS ID and FID for a broadcast and multicast service flow are valid. An ABS that supports E-MBS shall include the E-MBS Zone identifier(s) to which it belongs in the AAI-E-MBS_CFG message. The E-MBS Zone identifier shall not be '0'.

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------