| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Description for AMS Operational States in IEEE 802.16m |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Erik Colban Huawei Jaesun Cha ETRI Hyunjeong Kang Samsung Yang Liu ZTE Ming-Hung Tao ITRI Shantidev Mohanty Intel Corporation | E-mail:ecolban@huawei.com shantidev.mohanty@intel.com |
| Re: | Category: D8 / Area: AMS operational states |  |
| Abstract | This contribution proposes cleanup text for sleep mode section of IEEE 802.16m specification. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Description for AMS Operational States in IEEE 802.16m

Erik Colban

Huawei

Jaesun Cha

ETRI

Hyunjeong Kang

Samsung

Yang Liu

ZTE

Ming-Hung Tao

ITRI

Shantidev Mohanty

Intel Corporation

- Introduction

This contribution proposes text related to AMS operational states in IEEE 802.16m.

- Proposed Text for IEEE P802.16m/D8

============================ Start of Proposed Text ==================================

[Editor’s Note: The following is the proposed change in the 802.16m/D8. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.]

Remedy 1: [Insert the following text before Section 16.2.1 ]

16.2.0 AMS Operational States

Figure xxx: IEEE 802.16m Mobile Station State Transition Diagram

Initialization State

In the Initialization State, the AMS performs cell selection by scanning, synchronizing and acquiring the system configuration information before entering Access State.

During this state, if the AMS cannot properly perform the system configuration information decoding and cell selection, it falls back to perform scanning and DL synchronization. If the AMS successfully decodes the system configuration information and selects a target ABS, it transitions to the Access State.

Access State

The AMS performs network entry with the target ABS while in the Access State. Network entry is a multi step process consisting of ranging, basic capability negotiation, authentication, authorization, key registration with the ABS and service flow establishment. The AMS receives its Station ID and establishes at least one connection using and transitions to the Connected State. Upon failing to complete any one of the steps of network entry the AMS transitions to the Initialization State.

Connected State

When in the Connected State, an AMS operates in one of three modes; Sleep Mode, Active Mode and Scanning Mode. During Connected State, the AMS maintains two connections established during Access State. Additionally, the AMS and the ABS may establish additional transport connections. The AMS may remain in Connected State during a hand over. The AMS transitions from the Connected State to the Idle State based on a command from the ABS. Failure to maintain the connections prompts the AMS to transition to the Initialization State.

Active Mode

When the AMS is in Active Mode, the serving ABS may schedule the AMS to transmit and receive at the earliest available opportunity provided by the protocol, i.e. the AMS is assumed to be 'available' to the ABS at all times. The AMS may request a transition to either Sleep or Scanning Mode from Active Mode. Transition to Sleep or Scanning Mode happens upon command from the serving ABS. The AMS may transition to Idle State from Active Mode of Connected State.

Sleep Mode

When in Sleep Mode, the AMS and ABS agree on the division of the radio frame in time into Sleep Windows and Listening Windows. The AMS is only expected to be capable of receiving transmissions from the ABS during the Listening Windows and any protocol exchange has to be initiated during that time. The AMS transition to Active Mode is prompted by control messages received from the ABS. The AMS may transition to Idle State from Sleep Mode of Connected State during Listening Intervals.

Scanning Mode

When in Scanning Mode, the AMS performs measurements as instructed by the serving ABS. The AMS is unavailable to the serving ABS while in scanning Mode. The AMS returns to Active Mode once the duration negotiated with the ABS for scanning expires.

Idle State

The Idle state consists of two separated modes, Paging Available Mode and Paging Unavailable Mode based on its operation and MAC message generation. During Idle State, the AMS may perform power saving by switching between Paging Available Mode and Paging Unavailable Mode.

Remedy 2: [Modify the text in D8 as follows]

Page 84, line 57: replace "normal operation" with "Connected State"

Page 91, line 38: replace "normal operation" with "Connected State"

Page 132, line 57: replace "normal operation" with "Active Mode"

Page 154, line 27: replace "Normal Operation" with "Connected State"

Page 318, line 65: replace "the AMS and S-ABS resume their normal operation" with "the ABS may resumes, communication with scheduling traffic for the AMS."

[Page 345, line 13: replace: “normal operation” with “normal sleep mode operation”

[Page 354, line 31: replace: “normal operation” with “normal GRA operation”

Page 430, line 17: replace: "can interupt the normal operation of Sleep Cycles" with "can interrupt the normal Sleep Cycle operation"

Page 465, line 55: replace: "normal operation" with "Connected State"

Page 466, line 32: replace: "normal operation" with "Connected State"

Page 896, line 15: replace: "normal operation" with "Connected State"

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Apr 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.