| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-DSA-REQ/RSP message tables and ASN.1 code |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-DSA-REQ/RSP message tables and ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-DSA-REQ/RSP message tables and ASN.1 code

Joeyn Chou

Intel

Introduction

This contribution proposes changes to errors in AAI-DSA-REQ/RSP message tables and ASN.1 code.

Proposed Text

Remedy 1:

In the P802.16m/D9, page 220, line 22, modify Table 734 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 734—AAI-DSA-REQ Message Field Descriptions

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall always be present |
| SFID | 32 | Service flow identifier | Present when an ABS initiates AAI-DSA-REQ |
| FID | 4 | Flow identifier | Present when an ABS initiates AAI-DSA-REQ |
| Uplink/Downlink Indicator | 1 | 0: uplink; 1:downlink | N/A |
| Service Class Name | 16 to 1024 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes | Present when a predefined BS service configuration is used for this service flow. |
| If (Global Service Class Name is included) { | 84 | Name of global service classes (see Table 783 in section 16.2.12.8) | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| I: Unlink/Downlink indicator | 1 | 0: UL 1: DL |  |
| S: Maximum sustained traffic rule | 6 | Extensible look-up table 187 of 6.3.14.4.1 |  |
| B: Maximum traffic burst | 6 | Extensible look-up table 187 of 6.3.14.4.1 |  |
| R: Maximum received traffic rate | 6 | Extensible look-up table 187 of 6.3.14.4.1 |  |
| L: Maximum latency | 6 | Extensible look-up table 188 of 6.3.14.4.1 |  |
| P: Paging preference | 1 | 0 = No paging generation 1 = Paging generation |  |
| S1: Request/Transmission policy | 7 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be pre-fixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. |  |
| S2: Uplink grant scheduling type | 3 | 0: Reserved 1: Undefined (BS implementation-dependent) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |  |
| If (Uplink grant scheduling type = BE ¦ nrtPS) { |  |  |  |
| S3: Traffic priority | 3 |  |  |
| }else if (Uplink grant scheduling type = rtPS) { |  |  |  |
| S3: Traffic priority | 3 |  |  |
| S5: Unsolicited polling interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. |  |
| } else if (Uplink grant scheduling type = ertPS) { |  |  |  |
| L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1. |  |
| S3: Traffic priority | 3 |  |  |
| S4: Unsolicited grant interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. |  |
| } else if (Uplink grant scheduling type = UGS) { |  |  |  |
| L1: Tolerated Jitter | 6 | Extensible look-up table 188 of 6.3.14.4.1. |  |
| S4: Unsolicited grant interval | 6 | Extensible look-up table 189 of 6.3.14.4.1. |  |
| } |  |  |  |
| } // End If (Globa Service … |  |  |  |
| QoS parameter set type | 3 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |
| If (QoS parameters are needed) { |  |  |  |
| Traffic Priority parameter | 3 | 0 to 7-Higher numbers indicate higher priority Default 0 | Shall be presented for BE flows on both UL and DL; Optionally presented for other service flows (aGPS ertPS rtPS, nrtPS and UGS) on uplink) on UL. |
| Maximum Sustained Traffic Rate parameter | 32 | Rate (in bits per second) | Present if needed |
| Maximum Traffic Burst parameter | 32 | Burst size (bytes) | Present if needed |
| Minimum Reserved Traffic Rate parameter | 32 | Rate (in bits per second) | Present if needed |
| Maximum Latency parameter | 32 | Milliseconds | Present if needed |
| Paging Preference parameter | 1 | 0: No paging generation 1: Paging generation | Present if needed |
| Request/Transmission Policy parameter | 7 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be pre-fixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. | Present if needed |
| Tolerated Jitter parameter | 32 | Milliseconds | Present when needed |
| If(Uplink/Downlink Indicator = uplink) { |  |  |  |
| UL Grant Scheduling Type | 3 | 0: Reserved 1: Undefined (BS implementation dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service |  |
| If (UL Grant Scheduling Type = ertPS) { |  |  |  |
| Unsolicited Grant Interval parameter | 16 | Milliseconds |  |
| } else if (UL Grant Scheduling Type = UGS) { |  |  |  |
| Unsolicited Grant Interval parameter | 16 | Milliseconds |  |
| } else if (UL Grant Scheduling Type = rtPS) { |  |  |  |
| Unsolicited Polling Interval parameter | 16 | Milliseconds |  |
| } else if (UL Grant Scheduling Type = aGPS) { |  |  |  |
| Primary Grant Polling Interval parameter | 16 | Milliseconds |  |
| Primary Grant Size parameter | 16 | Number of bytes. |  |
| Secondary Grant Polling Interval parameter | 16 | Milliseconds | Present if secondary parameter set is needed |
| Secondary Grant Size parameter | 16 | Number of bytes. | Present if secondary parameter set is needed |
| Adaptation Method | 1 | 0:ABS-initiated adaptation1:AMS-initiated adaptation |  |
| } |  |  |  |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. |  |
| Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |  |
| For(i=0; i<N-Predefined-BR-indices; i++) { |  | The mapping of predefined BR index used in quick access message to BR size and BR actions N-Predefined-BR-indices is the number of predefined BR indices [1..15] |  |
| Predefined BR index | 4 | Predefined BR index |  |
| BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved |  |
| If(BR action = 0b10) { |  |  |  |
| BR size | 11 | Number of bytes requested by the pre-defined BR index |  |
| } |  |  |  |
| } |  |  |  |
| Initial Backoff Window Size | 4 | Window size expressed as a power of 2 |  |
| Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 |  |
| Backoff Scaling Factor | 4 | "0010" for Binary Exponential Back-off |  |
| } // End If (Uplink/Downlink Indicator = uplink) |  |  |  |
| MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in the connection is preserved by the MAC. 0 : not preserved 1 : preserved For ARQ connections, it shall be always set to '1'. | Present if needed |
| If (MAC in-order delivery indicator = 1) { |  |  |  |
| NON_ARQ_REORDERING_TIMEOUT | 6 | 0> and <=32, Unit is PHY frame (5ms) |  |
| } |  |  |  |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. | Present if needed |
| }// End If (QoS parameters are needed) |  |  |  |
| Type of Data Delivery Services parameter | 83 | 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Ser-vice 3: Best Effort Service 4: Extended Real-Time Variable Rate Service 5: aGP service 6..7: reserved | Present if needed |
| SDU Inter-Arrival Interval parameter | 16 | SDU inter-arrival interval in the resolution of 0.5 ms | Present if needed |
| Time Base parameter | 16 | Time base in milliseconds | Present if needed |
| SDU Size parameter | 8 | Number of bytes. Default = 49. | Present if needed |
| Target SAID parameter | 8 | SAID onto which service flow is mapped | Present if needed |
| If (ARQ parameters are needed) { |  |  |  |
| ARQ Enable | 1 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted | Present if needed |
| ARQ_WINDOW_SIZE | 16 | >0 and <=(ARQ_BSN_MODULUS/2) | Present if needed |
| ARQ_BLOCK_LIFETIME | 16 | 0 = Infinite 1-6553500 ""µs(100""µs granularity) | Present if needed |
| ARQ_SYNC_LOSS_TIMEOUT | 16 | 0 = Infinite 1-6553500 ""µs(100 ""µs granularity) | Present if needed |
| ARQ_RX_PURGE_TIMEOUT | 16 | 0 = Infinite 1-6553500 ""µs(100 ""µs granularity) | Present if needed |
| ARQ_SUB_BLOCK_SIZE | 3 | Bit 0-2: encoding for selected block size (P), where the selected block size is equal to 2^(P+3), 0<=P<=7. ARQ sub-block size is byte unit | Present if needed |
| ARQ_ERROR_DETECTION_TIMEOUT | 16 | 0 = Infinite 1-6553500 s (100 s granularity) | Present if needed |
| ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 | 0 = Infinite 1-6553500 s (100 s granularity) | Present if needed |
| } |  |  |  |
| CS Specification parameter | 8 | 0: Reserved 1: Packet, IPv4 2: Packet, IPv6 3: Packet, IEEE 802.3/Etherneta 4: Reserved 5: Reserved 6: Reserved 7: Reserved 8: Reserved 9: Reserved 10: Reserved 11: Reserved 12: Reserved 13: Reserved 14: Packet, IPb 15: Multiprotocol flow 16-255 Reserved (a: Classifiers for IEEE 802.1Q VLAN tags may be applied to service flows of this CS type) (b: SDUs for service flows of this CS type may carry either IPv4 or IPv6 in the header-compressed payload) | Present if needed |
| If (Packet Classification Rule parameter is needed) { |  |  |  |
| Classification Rule Priority field | 8 | 0-255 | Present if needed |
| Protocol field | 8 | protocol | Present if needed |
| IP Masked Source Address parameter | 32x2 (IPv4) or 128x2 (IPv6) | src, smask represent IP source and mask addresses that are 32 bits x 2 (Ipv4) or 128 bits x 2 (Ipv6) in size. | Present if needed |
| IP Masked Destination Address parameter | 32x2 (IPv4) or 128x2 (IPv6) | dst, dmask represent IP destination and mask addresses that are 32 bits x 2 (Ipv4) or 128 bits x 2 (Ipv6) in size. | Present if needed |
| Protocol Source Port Range field | 16x2 | sportlow, sporthigh represent the lower bound and higher bound of the protocol source port range | Present if needed |
| Protocol Destination Port Range field | 16x2 | dportlow, dporthigh represent the lower bound and higher bound of the protocol destination port range | Present if needed |
| Associated PHSI field | 8 | Index value | Present if needed |
| Packet Classification Rule Index field | 16 | Packet Classification Rule Index | Present if needed |
| if(Vendor-specific classification parameters are needed) { |  |  |  |
| Vendor ID | 24 | Vvendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. |  |
| } |  |  |  |
| IPv6 Flow Label field | 24 | Flow Label | Present if needed |
| Classification Action Rule | 1 | 0 = none. 1 = Discard packet |  |
| IP Type of Service | 7 | DSCP value (0 to 127) | Present if needed |
| PHS DSC Action field | 2 | 0: Add PHS Rule |  |
| If (PHS Rule field is added) { |  |  |  |
| PHSI field | 8 | Index value |  |
| PHSF field | n | String of bytes suppressed | Present if needed |
| PHSM field | n | Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field | Present if needed |
| PHSS field | 8 | Number of bytes in the suppression string | Present if needed |
| PHSV field | 1 | 0: Verify 1: Don't verify | Present if needed |
| Vendor-specific PHS parameters | variable |  | Present if needed |
| } |  |  |  |
| IEEE 802.3/Ethernet Destination MAC Address parameter | 48x2 | dst.msk represent Ethernet destination and mask addresses that are 48 bits x 2 in size. | Present if needed |
| IEEE 802.3/Ethernet Source MAC Address parameter | 48x2 | src, msk represent Ethernet source and mask addresses that are 48 bits x 2 in size. | Present if needed |
| Ethertype/IEEE 802.2 SAP | 2/34/18 | type, eprot1, eprot2 are the Layer 3 protocol ID. type = 0, eprot1, eprot2 are ignored type = 1, eprot1and eprot2 give two 16 bit value of the Ethertype that the packet shall match in order to match the rule. type = 2, eprot1and eprot2 give two lower 8 bit value of the Ethertype that shall match the DSAP byte of the packet in order to match the rule | Present if needed |
| IEEE 802.1D User Priority field | 3x2 | pri-low, pri-high are the IEEE 802.1D user_priority bits. Valid range: 0–7 for pri-low and pri-high | Present if needed |
| IEEE 802.1Q VLAN ID field | 12 | specifies the matching value of the IEEE 802.1Q vlan_id bits | Present if needed |
| } // End If (Packet Classification Rule |  |  |  |
| If ( ROHC Parameter is needed) { |  |  |  |
| ROHC Max Context ID | 16 | Non-negative integer |  |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved |  |
| ROHC MRRU | 16 | 0: no segmentation 1..65535: MRRU Maximum reconstructed reception unit |  |
| } |  |  |  |
| Packet Error Rate (PER) | 8 | MSB (bit 7): 0 - PER measured by the application, post-ARQ and post-HARQ process 1 - PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 - Interpret bits 0-5 as an integer %, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1-Interpret bits 0-5 as 10 times a negative exponent of 10, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0-5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10-7 to 1x10-0] PER | Present if needed |
| Emergency Indication parameter | 1 | Indicates the associated flow is used for emergency purposes. | Present if needed |
| MAC Header Type | 1 | Indicates whether AGMH or SPMH is presented at the start of MAC PDUs of the service flow 0 = AGMH (Advanced Generic MAC Header) 1 = SPMH (Short-Packet MAC header) default value is 0. | Present if SPMH is used to transmit MAC PDUs from this service flow |
| E-MBS Service | 3 | Indicates whether the MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present when E-MAC service is provided |
| If (E-MBS Service ≠ 0) { |  |  |  |
| for (i=0; i< Num of E-MBS Zone ID; i++) { | 3 | Num of E-MBS Zone ID is the nNumber of E-MBS IDs to add [1..8] |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| for (i=0; i< N-E-MBS-ID; i++) { |  | . N-E-MBS-ID is the number of E-MBS identifiers [1...15] |  |
| E-MBS ID | 12 | E-MBS identifier |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| } |  |  |  |
| } // End If (E-MBS … |  |  |  |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSA message 0b1: carrier switching parameters using AAI-E-MBS-REP message | Presented when ABS-initiated DSA for carrier switching |
| If(Carrier Switching Mode=0b0) { |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI= 2 superframes: N= 4 bits NMSI= 4 superframes: N= 4 bits NMSI= 8 superframes: N= 8 bits NMSI= 16 superframes: N= 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI= 2, then 2 frames per bit If NMSI= 4, 8 and 16, then 4 frames per bit |  |
| } |  |  |  |
| If ( Group Parameter Create/Change is needed) { |  | Refer to 16.2.12.8.6 |  |
| Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| Qty SFID request | 8 | The number of service flows to create | Shall only be sent by an AMS |
| For(i=0; i<N-FIDs; i++) { |  | FIDs of service flows created with this Group Parameter Create/Change N-FIDs is the number of FIDs |  |
| FID | 4 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Noncommon; i++) { |  | N-FIDs-Noncommon is the number of non-common service flow IDs |  |
| FID | 4 |  |  |
| Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
| } |  |  |  |
| } |  |  |  |
| If ( Coupled Group Create/Change is needed) { |  |  |  |
| Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| Qty Coupled SFID request | 8 | The number of DL service flow and UL service flow to create |  |
| For(i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of DL and UL service flow(4 MSB for DL FID and 4 LSB for UL FID) N-FIDs-Coupled is the number of FIDs |  |
| FID | 8 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs |  |
| FID | 4 |  |  |
| Non-common for Coupled Group | Variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |
| } |  |  |  |
| } |  |  |  |
| For(i=0; i<Num of Multicast SFID; i++) { | 4 | Num of Multicast SFID is the number of Multicast service flows to add [1..8] |  |
| Multicast Group ID | 12 | ID of a group to which the flow is added |  |
| FID | 4 | FID of service flow created |  |
| } |  |  |  |
| if(Sleep Cycle Setting is included) { | — | — | May be present when Sleep Cycle setting needs to be changed or switched |
| Response_Code | 2 | This indicates response type of AAI-SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI-SLP-REQ 0b10 : Rejection of AAI-SLP-REQ 0b11 : Reserved | This parameter shall be included only when ABS transmit this control message. |
| Operation | 2 | This indicates operation request type 0b00~0b01 : Reserved 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(Operation == 0b10 ¦¦ Operation==0b11) { | — | — |  |
| SCID | 4 | Sleep Cycle ID. 0~15 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Start Frame Number | 6 | 0~63 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept 1: the fast feedback channel is de-allocated at the frame specified by Start_Frame_Number 2: the fast feedback channel is automatically de-allocated at the beginning of the Sleep Window whenever the fast feedback channel is newly assigned to the AMS during the Listening Window 0~3 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| if(Operation =0b10) | — | — |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI-TRF-IND message is not sent for the AMS 1 : AAI-TRF-IND message is sent to the AMS during every Listening Window | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| LWEF | 1 | Listening window Extension Flag 0: the Listening window is of fixed duration 1: the Listening window is extensible | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00~0b11 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(TIMF == 1) { | — | — |  |
| SLPID | 10 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode 0~1023 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| } |  |  |  |
| if(NSCF == 0b10) { |  |  |  |
| New Initial Sleep Cycle | 5 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode Value: 0~31 New_Initial_Sleep_Cyle = Value+1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| if(LWEF == 1) { | — | — |  |
| T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| } //ENDIF (Operation =0b10) | — |  |  |
| } //ENDIF (Operation == 0b10 ¦¦ Operation==0b11) | — |  |  |
| REQ_duration | 8 | Waiting value for the AAI-SLP-REQ message retransmission, which is the Least Significant 8 bits of Frame Number. If REQ_duration is missing when Response_Code == 0b10 in AAI-DSx-RSP message, the AMS shall consider REQ_duration = 0 0~255 | This parameter can be included only when BS transmits AAI-DSx-RSP message in response to AAI-DSx-REQ message sent by AMS. |
| } // ENDIF (Sleep Cycle Setting is included) | — |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D9, page 235, line 51, modify Table 735 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 738—AAI-DSA-RSP Message Field Descriptions

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSA-REQ message | Shall always be present |
| Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure | Shall always be present |
| If (Confirmation Code = 0 && AMS initiated AAI-DSA-REQ) { |  |  |  |
| FID | 4 | An identifier of a service flow |  |
| } |  |  |  |
| If(received an AAI-DSA-REQ including Group Parameter Create/Change parameter ) { |  |  |  |
| For(i=0; i<N-FIDs; i++) { |  | N-FIDs is the number of successfully created service flow. | Shall be included when N-FIDs > 0 |
| FID | 4 | FIDs of service flows created successfully with this Group Parameter Create/Change. |  |
| } |  |  |  |
| } |  |  |  |
| E-MBS Service | 3 | Indicates whether the MBS service is being requested or provided for the connection that is being successfully setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| for (i=0; i< Num of E-MBS Zone ID; i++) { | 3 | Num of E-MBS Zone ID is the nNumber of E-MBS IDs to add [1..8] |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| for (i=0; i< N-E-MBS-ID; i++) { |  | N-E-MBS-ID is the number of E-MBS identifiers [1...15] |  |
| E-MBS ID | 12 | E-MBS identifier |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSA message 0b1: carrier switching parameters using AAI-E-MBS-REP message | Present if ABS indicates carrier switching when receiving AMS-initiated DSA |
| If(Carrier Switching Mode = 0b0) { |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be avail-able in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI= 2 superframes: N= 4 bits NMSI= 4 superframes: N= 4 bits NMSI= 8 superframes: N= 8 bits NMSI= 16 superframes: N= 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI= 2, then 2 frames per bitIf NMSI = 4, 8 and 16, then 4 frames per bit |  |
| } |  |  |  |
| For(i=0; i<Num of Multicast Group ID; i++) { | 4 | Num of Multicast Group ID is the number of Multicast Group ID to add [1..16] | Present when Num of Multicast Group ID > 0 |
| A) Multicast Group ID | 12 | ID of a group to which the flow is added |  |
| } |  |  |  |
| if(Sleep Cycle Setting is included) { | — | — | May be present when Sleep Cycle setting needs to be changed or switched |
| Response_Code | 2 | This indicates response type of AAI-SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI-SLP-REQ 0b10 : Rejection of AAI-SLP-REQ 0b11 : Reserved | This parameter shall be included only when ABS transmit this control message. |
| Operation | 2 | This indicates operation request type 0b00~0b01 : Reserved 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(Operation == 0b10 ¦¦ Operation==0b11) { | — | — |  |
| SCID | 4 | Sleep Cycle ID. 0~15 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Start Frame Number | 6 | 0~63 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept 1: the fast feedback channel is de-allocated at the frame specified by Start_Frame_Number 2: the fast feedback channel is automatically de-allocated at the beginning of the Sleep Window whenever the fast feedback channel is newly assigned to the AMS during the Listening Window 0~3 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| if(Operation =0b10) | — | — |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI-TRF-IND message is not sent for the AMS 1 : AAI-TRF-IND message is sent to the AMS during every Listening Window | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| LWEF | 1 | Listening window Extension Flag 0: the Listening window is of fixed duration 1: the Listening window is extensible | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00~0b11 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(TIMF == 1) { | — | — |  |
| SLPID | 10 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode 0~1023 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| } |  |  |  |
| if(NSCF == 0b10) { |  |  |  |
| New Initial Sleep Cycle | 5 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode Value: 0~31 New_Initial_Sleep_Cyle = Value+1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| if(LWEF == 1) { | — | — |  |
| T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| }} //ENDIF (Operation ==0b10) | — |  |  |
| } }//ENDIF (Operation == 0b10 ¦¦ Operation==0b11) | — |  |  |
| REQ_duration | 8 | Waiting value for the AAI-SLP-REQ message retransmission, which is the Least Significant 8 bits of Frame Number. If REQ_duration is missing when Response_Code == 0b10 in AAI-DSx-RSP message, the AMS shall consider REQ_duration = 0 0~255 | This parameter can be included only when BS transmits AAI-DSx-RSP message in response to AAI-DSx-REQ message sent by AMS. |
| }// ENDIF (Sleep Cycle Setting is included) | — |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D9, page 1004, line 51, changes as shown below:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Connection management Messages                         *

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

ServiceClassName ::= IA5String (SIZE(2..128))

DataRate ::= INTEGER (0..4294967295)

Interval ::= INTEGER (0..65535)

GlobalServiceClassName ::= SEQUENCE {

directionIndicator             DirIndicator OPTIONAL,

maxSustainedRate               INTEGER (0..63) OPTIONAL,

maxTrafficBurst                INTEGER (0..63) OPTIONAL,

minReservedTrafficRate        INTEGER (0..63) OPTIONAL,

maxLatency                     INTEGER (0..63) OPTIONAL,

pagingPreference               PagingPreference OPTIONAL,

reqTransmissionPolicy          ReqTransmissionPolicy OPTIONAL,

ulGrantScheduling              UlGrantScheduling OPTIONAL,

toleratedJitter                INTEGER (0..63) OPTIONAL,

trafficPriority                INTEGER (0..7) OPTIONAL,

unsolicitedGrantInterval       INTEGER (0..63) OPTIONAL,

unsolicitedPollingInterval     INTEGER (0..63) OPTIONAL}

QosParameterSetType ::= BIT STRING {provisionedSet (0),

admittedSet    (1),

activeSet      (2)} (SIZE(3))

DirIndicator ::= ENUMERATED {uplink,

downlink}

ConfirmationCode ::= ENUMERATED {successful,

failure}

-- The mapping of predefined BR index used in quick access message to

-- BR size and BR actions

PredefinedBrIndex ::= SEQUENCE {

brIndex                        INTEGER (0..15) OPTIONAL,

brAction                       INTEGER {ertPS (0),

aGP   (1),

br    (2)

} (0..3) OPTIONAL,

brSize                         INTEGER (0..2047) OPTIONAL} -- bytes

UplinkQosInfo ::= SEQUENCE {

ulGrantScheduling            UlGrantScheduling,

ulSchedulingType               CHOICE {

ertPs                        SEQUENCE {

unsolicitedGrantInterval   Interval}, -- ms

ugs                          SEQUENCE {

unsolicitedGrantInterval   Interval}, -- ms

rtPs                         SEQUENCE {

unsolicitedPollingInterval Interval}, -- ms

agPs                         SEQUENCE {

priGrantPollingInterval    Interval, -- ms

secGrantPollingInterval    Interval OPTIONAL, -- ms

priGrantSize               GrantSize, -- bytes

secGrantSize               GrantSize OPTIONAL, -- bytes

adaptationMethod           ENUMERATED {

absInitiated,

amsInitiated}}

},

accessClass                    INTEGER (0..3),

differentiatedBrTimer          INTEGER (1..64), -- frames

predefinedBrIndexList          SEQUENCE (SIZE(0..15)) OF

PredefinedBrIndex,

initialBackoffWindowSize       INTEGER (0..15),

maxBackoffWindowSize           INTEGER (0..15),

backoffScalingFactor           INTEGER (0..15)

}

QosParameter ::= SEQUENCE {

trafficPriority                INTEGER (0..7) OPTIONAL,

maxSustainedRate               DataRate OPTIONAL, -- bps

maxTrafficBurst                DataRate OPTIONAL, -- bytes

minReservedTrafficRate         DataRate OPTIONAL, -- bps

maxLaytency                    DataRate OPTIONAL, -- ms

pagingPreference               PagingPreference OPTIONAL,

reqTransmissionPolicy          ReqTransmissionPolicy OPTIONAL,

toleratedJitter                DataRate OPTIONAL, -- ms

directionalQosInfo             CHOICE {

downlinkInfo                 NULL,

uplinkInfo                   UplinkQosInfo},

macInOrderDelivery             ENUMERATED {

notPreserved,

preserved} OPTIONAL,

orderPreservation              CHOICE {

notPreservation              NULL,

preservation                 SEQUENCE {

nonArqReorderTimeout       INTEGER (1..32)}}, -- 5ms

vendorId                     OCTET STRING (SIZE(3)) OPTIONAL

}

GrantSize ::= INTEGER (0..65535)

UlGrantScheduling ::=           ENUMERATED {

undefined,

bestEffort,

nrtPs,

rtPs,

ertPs,

ugs}

ReqTransmissionPolicy ::=       BIT STRING {

broadcastBr     (0),

multicastBr     (1),

piggyback       (2),

noFragmentation (3),

noPhs           (4),

noPcking        (5),

noRohc          (6)} (SIZE(7))

PagingPreference ::=            ENUMERATED {

noPagingGeneration,

pagingGeneration}

DataDeliveryServices ::=        INTEGER {

ugs(0),

rtVrs(1),

nrtVrs(2),

be(3),

ertVrs(4),

aGPs(5)} (0..255)

CsSpecification ::=             INTEGER {

packetIpv4 (1),

packetIpv6 (2),

packetEthernet (3),

packetIpv4OrIpv6 (14),

multiProtocol (15)} (0..255)

ClassifierDscAction ::=         ENUMERATED {

dscAddClassifier,

dscReplaceClassifier,

dscDeleteClassifier}

FsnSize ::=                     ENUMERATED {

fsn3Bits,

fsn11bits}

MbsService ::=                  ENUMERATED {

noMbs,

mbsInServingBs,

noMacroDiversity,

withMacroDiversity}

SnFeedback ::=                  ENUMERATED {

snFeedbackDisable,

snFeedbackEnable}

HarqServiceFlow ::=             ENUMERATED {

noHarq,

withHarq}

PacketErrorRate ::=             SEQUENCE {

perMeasurement                ENUMERATED {

postArqHarq,

beforeArqHarq},

perValue                    CHOICE {

percentage                INTEGER (0..63), -- 0 to 63%

negExponentialOf10        INTEGER (0..63)} -- PER = 10EXP -N/10

}

AlertClass ::=                  BIT STRING {

priority0 (0),

priority1 (1),

priority2 (2),

priority3 (3)} (SIZE(4))

ArqAttribute ::= SEQUENCE {

arqEnable                      ENUMERATED {

arqNotRequested,

arqRequested} OPTIONAL,

arqWindowSize                  INTEGER (1..65535) OPTIONAL, --

arqBlockLifeTime               INTEGER (0..65535) OPTIONAL, -- 100us

arqSyncLossTimeout             INTEGER (0..65535) OPTIONAL, -- 100us

arqPurgeTimeout                INTEGER (0..65535) OPTIONAL, -- 100us

-- sub-block size = 2^(P+3), 0<=P<=7

arqSubBlockSize                INTEGER (0..7) OPTIONAL, -- Bytes

arqErrorDetectionTimeout       INTEGER (0..65535) OPTIONAL, -- 100s

arqFeedbackPollRetryTimeout    INTEGER (0..65535) OPTIONAL  -- 100s

}

IpAddrAndMask ::= SEQUENCE {

ipv4VsIpv6                     CHOICE {

ipV4                         SEQUENCE {

ipAddr                     BIT STRING (SIZE (32)),

ipMask                     BIT STRING (SIZE (32))},

ipV6                         SEQUENCE {

ipAddr                     BIT STRING (SIZE (128)),

ipMask                     BIT STRING (SIZE (128))}

}

}

ProtocolPortRange ::= SEQUENCE {

lowPortNumber                IPPortNumber,

highPortNumber               IPPortNumber}

ClassificationRule ::= SEQUENCE {

priority                       INTEGER (0..255) OPTIONAL,

protocolField                  INTEGER (0..255) OPTIONAL,

ipMaskAndSrcAddr            IpAddrAndMask

OPTIONAL,

ipMaskAndDestAddr          IpAddrAndMask

OPTIONAL,

protocolSrcPortRange       ProtocolPortRange

OPTIONAL,

protocolDestPortRange       ProtocolPortRange

OPTIONAL,

associatedPhsiIndex            INTEGER (0..255) OPTIONAL,

classificationRuleIndex        INTEGER (0..65535) OPTIONAL,

vendorID                       OCTET STRING (SIZE(3)) OPTIONAL,

ipV6FlowLabel                  BIT STRING (SIZE(24)) OPTIONAL,

classificationAction           ENUMERATED{

none(0)

discard (1)

} OPTIONAL,

dscpValue                      INTEGER (0..63) OPTIONAL,

phsDscAction                   PHSDSCAction OPTIONAL,

phsRuleAdded                   PhsRuleActions OPTIONAL,

ethernetAttributes            EthernetAttributes OPTIONAL

}

PHSDSCAction ::= ENUMERATED {

addPHSRule,

setPHSRule,

deletePHSRule,

deleteAllPHSRules }

PhsRuleActions ::= CHOICE {

addPhsRule                      PhsRule,

setPhsRule                      PhsRule,

deletePhsRule                   INTEGER (0..255),

deleteAllPhsRules               NULL}

PhsRule ::= SEQUENCE {

phsiField                      INTEGER (0..255) OPTIONAL,

phsfField                      OCTET STRING (SIZE(0..255))OPTIONAL,

-- phsmField[x] = 0 don't surpress x's byte,

-- phsmField[x] = 1 subpress x's byte

phsmField                      BIT STRING (SIZE (0..255)) OPTIONAL,

phsvField                      ENUMERATED {

verify,

doNotVerify} OPTIONAL,

vendorSpecificPHSParameters    OCTET STRING (SIZE(0..999)) OPTIONAL

}

EthMacAddrAndMask ::= SEQUENCE {

ethMacAddress                  BIT STRING (SIZE (48)),

ethMaskAddress                 BIT STRING (SIZE (48))}

EtherType ::= ENUMERATED {

noLayer3Matching,

dixSnap,

dsap}

EtherType16Bits ::= SEQUENCE {

eprot1                         BIT STRING (SIZE (16)),

eprot2                         BIT STRING (SIZE (16))}

EtherType8Bits ::= SEQUENCE {

eprot1                         BIT STRING (SIZE (8)),

eprot2                         BIT STRING (SIZE (8))}

Ieee802Dot2Sap ::= SEQUENCE {

ethType                        EtherType,

layer3ProtocolId               CHOICE {

noL3Matching                 NULL,

dixSnapEncp                  EtherType16Bits,

dsapEncp                     EtherType8Bits}

}

UserPriority ::= SEQUENCE {

priorityLow                    INTEGER (0..7),

priorityHigh                   INTEGER (0..7)}

EthernetAttributes ::= SEQUENCE {

ethMaskAndDestMacAddr	EthMacAddrAndMask OPTIONAL,

ethMaskAndSrcMacAddr	EthMacAddrAndMask OPTIONAL,

ieee802Dot2Sap	Ieee802Dot2Sap OPTIONAL,

userPriority	          UserPriority OPTIONAL,

vlanId		        BIT STRING (SIZE (12)) OPTIONAL

}

RohcAttributes ::= SEQUENCE {

maxContextId                   INTEGER (0..65535) OPTIONAL,

largeContextId                 INTEGER{

smallCid (0),

largeCid (1)} (0..255) OPTIONAL,

-- 0: no segmentation,

-- 1..65535 Maximum reconstructed reception unit in Byte

rohcMrru                       INTEGER(0..65535) OPTIONAL}

EMBSService ::= BIT STRING {

embsInServingABSOnly (0),

embsInZoneWithMacroDiversity (1),

embsInZoneWithoutMacroDiversity (2)} (SIZE(3))

ClassifierDSCAction ::= ENUMERATED {

dscAddClassifier,

dscReplaceClassifier,

dscDeleteClassifier}

GroupID ::= INTEGER (0..4095)

GroupResourceAllocInfo ::= SEQUENCE {

groupID                        GroupID,

longTTIIndicator               ENUMERATED {

oneAAISubframe,

fourOrAllAAISubframes},

periodicity                    ENUMERATED {

oneFrame,

twoFrames,

fourFrames,

eightFrames},

mimoModeSet                    ENUMERATED {

mode0,

mode1And2,

mode2,

mode2And4},

userBitmapSize                 ENUMERATED {

size4,

size8,

size16,

size32},

userBitmapIndex                INTEGER (0..31),

initialACID                    INTEGER (0..15),

numberOfACIDs                  INTEGER (0..7),

resourceSizeInclusionBitmap    BIT STRING (SIZE(16))

}

SleepRspCode ::= ENUMERATED {unsolicitedRequest,

slpReqApproved,

slpReqRejected }

FastFbkChannelOp ::= ENUMERATED {fastFbkChKept,

fastFbkChDeallocAtFrame,

autoFastFbkChDealloc }

SleepOperationDSx ::= ENUMERATED {

null,

null2,

changeSleepCycleSetting,

switchSleepCycleSetting }

SleepCycleSetting ::= SEQUENCE {

sleepReqResponses                 CHOICE {

amsSlpReq         SleepReqAction,

absSlpRes            SleepRspAction}

}

SleepReqAction ::= SEQUENCE {

operation                     SleepOperationDSx,

sleepReqOperationInfo          CHOICE {

switchSleepCycle             SEQUENCE {

sleepCycleID               SCID,

startFrameNumber           INTEGER (0..63)},

changeSleepCycle              SEQUENCE {

sleepCycleID               SCID,

startFrameNumber           INTEGER (0..63),

trafficIndicationFlag      TrafficIndicationFlag,

listeningWindowExtFlag     ListeningWindowExtFlag,

nextSleepCycleIndicator    NextSleepCycleIndicator,

initialSleepCycle          INTEGER (0..15),

finalSleepCycle            INTEGER (0..1023),

listeningWindow            INTEGER (0..63),

newInitialSleepCycle       INTEGER (0..31) OPTIONAL,

tAMS                       INTEGER (0..31) OPTIONAL}

}

}

SleepRspAction ::= SEQUENCE {

responseCode                   SleepRspCode,

operation                      SleepOperationDSx OPTIONAL,

sleepReqOperationInfo          CHOICE {

switchSleepCycle              SEQUENCE {

sleepCycleID               SCID OPTIONAL,

startFrameNumber           INTEGER (0..63) OPTIONAL,

ffbchOperation             FastFbkChannelOp OPTIONAL },

changeSleepCycle              SEQUENCE {

sleepCycleID               SCID OPTIONAL,

startFrameNumber           INTEGER (0..63) OPTIONAL,

ffbchOperation             FastFbkChannelOp OPTIONAL,

trafficIndicationFlag      TrafficIndicationFlag OPTIONAL,

listeningWindowExtFlag     ListeningWindowExtFlag OPTIONAL,

nextSleepCycleIndicator    NextSleepCycleIndicator OPTIONAL,

initialSleepCycle          INTEGER (0..15) OPTIONAL,

finalSleepCycle            INTEGER (0..1023) OPTIONAL,

listeningWindow            INTEGER (0..63) OPTIONAL,

slpId                      SLPID OPTIONAL,

newInitialSleepCycle       INTEGER (0..31) OPTIONAL,

tAMS                       INTEGER (0..31) OPTIONAL}

} OPTIONAL,

reqDuration                    INTEGER (0..255) OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DSA Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AbsInitDsaInfo ::= SEQUENCE {

sfid                                 SFID,

fid                                  FID

}

AAI-DSA-REQ ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

absInitDsaInfo                 AbsInitDsaInfo OPTIONAL,

directionIndicator             DirIndicator,

serviceClassName               ServiceClassName OPTIONAL,

globalServiceClass             GlobalServiceClassName OPTIONAL,

qosParameterSet                   QosParameterSetType OPTIONAL,

qoSParameters                  QosParameter OPTIONAL,

dataDeliveryServices           DataDeliveryServices OPTIONAL,

sduInterArrival                Interval OPTIONAL, -- 0.5ms

timeBase                       Interval OPTIONAL, -- ms

sduSize                        INTEGER (0..255) DEFAULT 49,

targetSaid                     INTEGER (0..255) OPTIONAL,

arqAttributes                  ArqAttribute OPTIONAL,

csSpecificationType            CsSpecification OPTIONAL,

classificationRules            ClassificationRule OPTIONAL,

rohcAttributes                 RohcAttributes OPTIONAL,

packetErrorRate                PacketErrorRate OPTIONAL,

emergencyIndication            BOOLEAN OPTIONAL,

macHeaderType                  ENUMERATED {

agmh,

spmh} OPTIONAL,

embsService                    EMBSService OPTIONAL,

fullEMBSIDArray                SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

carrierIndex                 PhyCarrierIndex,

embsidFIDMappingList         SEQUENCE (SIZE(1..15)) OF SEQUENCE {

embsid                     EMBSID,

fid                        FID}

},

unicastAvailIntervalBitmap     CHOICE {

nmsi2                        BIT STRING (SIZE(4)),

nmsi4                        BIT STRING (SIZE(4)),

nmsi8                        BIT STRING (SIZE(8)),

nmsi16                       BIT STRING (SIZE(16))} OPTIONAL,

groupParameterCreateChange     SEQUENCE {

-- ??? this part of the table is obscure

commonQosParameters           QosParameter OPTIONAL,

qtySfid                          INTEGER (0..255) OPTIONAL,

fidArray                         SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                              INTEGER (0..15) OPTIONAL,

nonCommonQosParameters       QosParameter OPTIONAL}

} OPTIONAL,

coupledGroupCreateChange        SEQUENCE {

-- ??? this part of the table is obscure

commonQoSParameters           QosParameter OPTIONAL,

qtyCoupledSfid                   INTEGER (0..255) OPTIONAL,

coupledFidArray                  SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                              INTEGER (0..15) OPTIONAL},

couplednonCommonFidArray      SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                              INTEGER (0..15) OPTIONAL,

nonCommonQosParameters       QosParameter OPTIONAL}

} OPTIONAL,

multicastGroup                    SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupId                 INTEGER (0..4095),

fid                                 INTEGER (0..15)} OPTIONAL,

sleepCycleSetting                SleepCycleSetting OPTIONAL,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DSA Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DSA-RSP ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

confirmationCode               ConfirmationCode,

fid                            FID OPTIONAL,

groupParameterCreateChange     SEQUENCE {

fidArray                   SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                        INTEGER (0..15) OPTIONAL}

} OPTIONAL,

embsService                    EMBSService OPTIONAL,

fullEMBSIDArray                SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

carrierIndex                 PhyCarrierIndex,

embsidFIDMappingArray        SEQUENCE (SIZE(1..15)) OF SEQUENCE {

embsid                     EMBSID,

fid                        FID}

},

carrierSwitching               CHOICE {

unicastAvailIntervalBitmap     CHOICE {

nmsi2                        BIT STRING (SIZE(4)),

nmsi4                        BIT STRING (SIZE(4)),

nmsi8                        BIT STRING (SIZE(8)),

nmsi16                       BIT STRING (SIZE(16))},

aaiEmbsRepMsg                NULL},

multicastGroup                    SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupId                 INTEGER (0..4095)} OPTIONAL,

sleepCycleSetting             SleepCycleSetting OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------