| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-DSC-REQ/RSP messages tables and ASN.1 code |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-DSC-REQ/RSP messages tables and ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-DSC-REQ/RSP messages tables and ASN.1 code

Joey Chou

Intel

Introduction

This contribution proposes changes to AAI-DSC-REQ/RSP messages tables and ASN.1 code

Proposed Text

Remedy 1:

In the P802.16m/D9, page 241, line 23, modify Table 717 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 737—AAI-DSC-REQ Message Field Descriptions

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | Shall be always present |
| SFID | 32 | Service flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| FID | 4 | Flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| Uplink/Downlink Indicator | 1 | 0: uplink; 1:downlink | Shall be always present |
| Service Class Name | 16 to 1024 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes | Present when a predefined BS service configuration is used for this service flow. |
| Global Service Class Name field | 84 | Name of global service classes (see Table 783 in section 16.2.12.8) | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| QoS parameter set type | 3 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |
| If ( QoS parameters are needed ) { |  |  |  |
| Traffic Priority parameter | 3 | 0 to 7-Higher numbers indicate higher priority Default 0 | Present when needed |
| Maximum Sustained Traffic Rate parameter | 32 | Rate (in bits per second) | Present when needed |
| Maximum Traffic Burst parameter | 32 | Burst size (bytes) | Present when needed |
| Minimum Reserved Traffic Rate parameter | 32 | Rate (in bits per second) | Present when needed |
| Maximum Latency parameter | 32 | Milliseconds | Present when needed |
| Paging Preference parameter | 1 | 0: No paging generation 1: Paging generation |  |
| Tolerated Jitter parameter | 32 | Milliseconds | Present when needed |
| If (UL scheduling type = UGS ¦ ertPS) { |  |  |  |
| Unsolicited Grant Interval parameter | 16 | Milliseconds |  |
| }else if (UL scheduling type = rtPS) { |  |  |  |
| Unsolicited Polling Interval parameter | 16 | Milliseconds |  |
| }else if (UL scheduling type = rtPS) { |  |  |  |
| Primary Grant Polling Interval parameter | 16 | Milliseconds |  |
| Primary Grant Size parameter | 16 | Number of bytes. |  |
| Secondary Grant Polling Interval parameter | 16 | Milliseconds | Present when secondary parameter set is needed |
| Secondary Grant Size parameter | 16 | Number of bytes. | Present when if secondary parameter set is needed |
| } |  |  |  |
| If (Uplink/Downlink Indicator = 0) { |  |  |  |
| Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. |  |
| Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |  |
| For(i=0; i<N-Predefined-BR-indices; i++) { |  | The mapping of predefined BR index used in quick access message to BR size and BR actions (see table 789) N-Predefined-BR-indices is the number of predefined BR indices [1..15] |  |
| Predefined BR index | 4 | Predefined BR index |  |
| BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved |  |
| If(BR action = 0b10) { |  |  |  |
| BR size | 11 | Number of bytes requested by the pre-defined BR index |  |
| } |  |  |  |
| } |  |  |  |
| Initial Backoff Window Size | 4 | Window size expressed as a power of 2 |  |
| Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 |  |
| Backoff Scaling Factor | 4 | "0010" for Binary Exponential Back-off |  |
| }End If (Uplink/Downlink Indicator |  |  |  |
| MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in the connection is preserved by the MAC. 0 : not preserved 1 : preserved For ARQ connections, it shall be always set to '1'. |  |
| Vendor ID | 24 | Vvendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. | Present if needed |
| SDU Inter-Arrival Interval parameter | 16 | SDU inter-arrival interval in the resolution of 0.5 ms | Present when needed |
| Time Base parameter | 16 | Time base in milliseconds | Present when needed |
| Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier |  |
| If (Packet Classification Rule parameter is needed) { |  |  |  |
| Classification Rule Priority field | 8 | 0-255 |  |
| Protocol field | 8 | protocol |  |
| IP Masked Source Address parameter | 32x2 (IPv4) or 128x2 (IPv6) | src, smask represent IP source and mask addresses that are 32 bits x 2 (Ipv4) or 128 bits x 2 (Ipv6) in size. | Present when needed |
| IP Masked Destination Address parameter | 32x2 (IPv4) or 128x2 (IPv6) | dst, dmask represent IP destination and mask addresses that are 32 bits x 2 (Ipv4) or 128 bits x 2 (Ipv6) in size. | Present when needed |
| Protocol Source Port Range field | 16x2 | sportlow, sporthigh represent the lower bound and higher bound of the protocol source port range | Present when needed |
| Protocol Destination Port Range field | 16x2 | dportlow, dporthigh represent the lower bound and higher bound of the protocol destination port range | Present when needed |
| Associated PHSI field | 8 | Index value | Present when needed |
| Packet Classification Rule Index field | 16 | Packet Classification Rule Index | Present when needed |
| Vendor ID | 24 | Vvendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. | Present when needed |
| IPv6 Flow Label field | 24 | Flow Label | Present when needed |
| Classification Action Rule | 1 | 0 = none. 1 = Discard packet |  |
| IP Type of Service | 7 | DSCP value (0 to 127) | Present when needed |
| PHS DSC Action field | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules | Present when needed |
| if(PHS DSC Action field = Add PHS Rule ¦¦ Set PHS Rule ) { |  |  |  |
| PHSI field | 8 | Index value |  |
| PHSF field | n | String of bytes suppressed | Present if needed |
| PHSM field | n | Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field1 = Suppress (x + 1) byte of the suppression field | Present if needed |
| PHSS field | 8 | Number of bytes in the suppression string | Present if needed |
| PHSV field | 1 | 0: Verify 1: Don't verify | Present if needed |
| Vendor-specific PHS parameters | variable |  | Present if needed |
| } |  |  |  |
| Else if (PHS DSC Action field = Delete PHS Rule) { |  |  |  |
| PHSI field | 8 | Index value |  |
| } |  |  |  |
| IEEE 802.3/Ethernet Destination MAC Address parameter | 48x2 | Dst,msk represent Ethernet destination and mask addresses that are 48 bits x 2 in size. | Present if needed |
| IEEE 802.3/Ethernet Source MAC Address parameter | 48x2 | src, msk represent Ethernet source and mask addresses that are 48 bits x 2 in size. | Present if needed |
| Ethertype/IEEE 802.2 SAP | 2/34/18 | type, eprot1, eprot2 are the Layer 3 protocol ID. type = 0, eprot1, eprot2 are ignored type = 1, eprot1and eprot2 give two 16 bit value of the Ethertype that the packet shall match in order to match the rule. type = 2, eprot1and eprot2 give two lower 8 bit value of the Ethertype that shall match the DSAP byte of the packet in order to match the rule | Present if needed |
| IEEE 802.1D User Priority field | 3x2 | pri-low, pri-high are the IEEE 802.1D user_priority bits. Valid range: 0–7 for pri-low and pri-high | Present if needed |
| IEEE 802.1Q VLAN ID field | 12 | specifies the matching value of the IEEE 802.1Q vlan_id bits. | Present if needed |
| } |  |  |  |
| } // end if QoS |  |  |  |
| If ( ROHC Parameter is needed) { |  |  | Present if needed |
| ROHC Max Context ID | 16 | Non-negative integer |  |
| Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved |  |
| ROHC MRRU | 16 | 0: no segmentation 1..65535: MRRU Maximum reconstructed reception unit |  |
| } |  |  |  |
| Packet Error Rate (PER) | 8 | MSB (bit 7): 0 - PER measured by the application, post-ARQ and post-HARQ process 1 - PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 - Interpret bits 0-5 as an integer %, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1-Interpret bits 0-5 as 10 times a negative exponent of 10, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0-5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10-7 to 1x10-0] PER | Present if needed |
| Emergency Indication parameter | 1 | Indicates the associated flow is used for emergency purposes. | Present if needed |
| for (i=0; i< Num of E-MBS Zone ID; i++) { | 3 | Num of E-MBS Zone ID [1…8] is the nNumber of E-MBS ID to update | Present when ABS initiates AAI-DSC-REQ Present only if Num of E-MBS Zone ID > 0 |
| Service_flow_update_indicator | 1 | Indicator whether "bitmap + new ser-vice flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Map-pings" | N/A |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | N/A |
| New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| Physical Carrier Index | 6 | The physical carrier index that is sup-port for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSC message 0b1: carrier switching parameters using AAI_E-MBS-REP message | In case of ABS-initiated DSC for carrier switching to update |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | In case of ABS-initiated DSC for carrier switching to update Shall be present if Carrier Switching Mode = 0b0 |
| If (Service_flow_update_indicator = 0b0) { |  |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS.A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. |  |
| for(m=0:m<N_ServiceFlowUpdateBitmap;m++){ |  | N_ServiceFlowUpdateBitmap is the number of bits to be set to 1 of Service Flow Update Bitmap For each bit in the Service Flow Update Bitmap = 1 to indicate the corresponding service flow is updated Range: 0~15 | Presented only if the number of bits set to 1 in Service Flow Update Bitmap > 1. |
| New_E-MBS ID | 12 | New E-MBS identifier. |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| }else if (Service_flow_update_indicator = 0b1) { |  |  |  |
| for(k=0;k<N_E-MBS_FID_Mapping;k++) { |  | N_E-MBS_FID_Mapping is the number of mappings of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. N-E-MBS-IDs is the number of E-MBS IDs |  |
| Current E-MBS ID | 12) | current E-MBS identifier |  |
| Current FID | 4 | Current flow ID |  |
| New_E-MBS ID | 12 | New E-MBS identifier |  |
| New FID | 4 | New flow ID |  |
| } |  |  |  |
| }//end of for |  |  |  |
| If ( Group Parameter Create/Change is needed) { |  | Refer to 16.2.12.8.6 |  |
| Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| Qty SFID request | 8 | The number of service flows to change | Shall only be sent by an AMS |
| For(i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of service flows changed with this Group Parameter Create/Change. |  |
| FID | 4 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Noncommon; i++) { |  | N-FIDs-Noncommon is the number of non-common service flow IDs |  |
| FID | 4 |  |  |
| Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
| } |  |  |  |
| } |  |  |  |
| If ( Coupled Group Create/Change is needed) { |  |  |  |
| Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| Qty Coupled SFID request | M8 | The number of DL service flow and UL service flow to change |  |
| For(i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of DL and UL service flow4MSB for DL FID and 4LSB for UL FID N-FIDs-Coupled is the number of FIDs |  |
| FID | 8 |  |  |
| } |  | Flow identifier |  |
| For(i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs |  |
| FID | 4 |  |  |
| Non-common for Coupled Group | Variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |
| } |  |  |  |
| } |  |  |  |
| For(i=0; i< Num of Multicast Group ID; i++) { | 4 | Num of Multicast Group ID is the nNumber of Multicast Group IDs to add | Present when ABS initiates AAI-DSC-REQ Present only if Num of Multi-cast Group ID > 0 |
| Multicast Group ID to be added | 12 | Multicast Group ID to be added |  |
| } |  |  |  |
| For(i=0; i< Num of Multicast Group ID; i++) { | 4 | Num of Multicast Group ID is the number of Multicast Group IDs to be delete | Present when ABS initiates AAI-DSC-REQ Present only if Num of Multi-cast Group ID > 0 |
| Multicast Group ID to be deleted | 12 | Multicast Group ID to be deleted |  |
| } |  |  |  |
| if(Sleep Cycle Setting is included) { | — | — | May be present when Sleep Cycle setting needs to be changed or switched |
| Response_Code | 2 | This indicates response type of AAI-SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI-SLP-REQ 0b10 : Rejection of AAI-SLP-REQ 0b11 : Reserved | This parameter shall be included only when ABS transmit this control message. |
| Operation | 2 | This indicates operation request type 0b00~0b01 : Reserved 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(Operation == 0b10 ¦¦ Operation==0b11) { | — | — |  |
| SCID | 4 | Sleep Cycle ID. 0~15 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Start Frame Number | 6 | 0~63 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept 1: the fast feedback channel is de-allocated at the frame specified by Start_Frame_Number 2: the fast feedback channel is automatically de-allocated at the beginning of the Sleep Window whenever the fast feedback channel is newly assigned to the AMS during the Listening Window 0~3 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| if(Operation =0b10) | — | — |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI-TRF-IND message is not sent for the AMS 1 : AAI-TRF-IND message is sent to the AMS during every Listening Window | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| LWEF | 1 | Listening window Extension Flag 0: the Listening window is of fixed duration 1: the Listening window is extensible | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00~0b11 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(TIMF == 1) { | — | — |  |
| SLPID | 10 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode 0~1023 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| } |  |  |  |
| if(NSCF == 0b10) { |  |  |  |
| New Initial Sleep Cycle | 5 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode Value: 0~31 New_Initial_Sleep_Cyle = Value+1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| if(LWEF == 1) { | — | — |  |
| T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| } | — | //ENDIF (Operation =0b10) |  |
| } | — | //ENDIF (Operation == 0b10 ¦¦ Operation==0b11) |  |
| REQ_duration | 8 | Waiting value for the AAI-SLP-REQ message retransmission, which is the Least Significant 8 bits of Frame Number. If REQ_duration is missing when Response_Code == 0b10 in AAI-DSx-RSP message, the AMS shall consider REQ_duration = 0 0~255 | This parameter can be included only when BS transmits AAI-DSx-RSP message in response to AAI-DSx-REQ message sent by AMS. |
| } | — | // ENDIF (Sleep Cycle Setting is included) |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D9, page 252, line 13, modify Table 738 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 738—AAI-DSC-RSP Message Field Descriptions

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| FID Change Count | 4 | FID Change Count from corresponding the AAI-DSC-REQ message | Shall be always present |
| SFID | 32 | Service flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| Confirmation Code | 1 | 0 = request was successful. 1 = request was failure |  |
| If(received an AAI-DSA-REQ including Group Parameter Create/Change parameter ) { |  |  |  |
| For(i=1; i<=N-FIDs; i++) { |  | N-FIDs is the number of successfully created service flow. The range of N-FID is 1..16. | Shall be included when N-FIDs > 0 |
| FID | 4 | FIDs of service flows created successfully with this Group Parameter Create/Change. |  |
| } |  |  |  |
| } |  |  |  |
| if(Sleep Cycle Setting is included) { | — | — | May be present when Sleep Cycle setting needs to be changed or switched |
| Response_Code | 2 | This indicates response type of AAI-SLP-RSP message. 0b00 : Request by ABS in Unsolicited manner 0b01 : Approval of AAI-SLP-REQ 0b10 : Rejection of AAI-SLP-REQ 0b11 : Reserved | This parameter shall be included only when ABS transmit this control message. |
| Operation | 2 | This indicates operation request type 0b00~0b01 : Reserved 0b10 : Change Sleep Cycle setting 0b11 : Switch Sleep Cycle setting | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(Operation == 0b10 ¦¦ Operation==0b11) { | — | — |  |
| SCID | 4 | Sleep Cycle ID. 0~15 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Start Frame Number | 6 | 0~63 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| FFBCH_Operation | 2 | 0: the fast feedback channel assigned to the AMS is kept 1: the fast feedback channel is de-allocated at the frame specified by Start_Frame_Number 2: the fast feedback channel is automatically de-allocated at the beginning of the Sleep Window whenever the fast feedback channel is newly assigned to the AMS during the Listening Window 0~3 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| if(Operation =0b10) | — | — |  |
| TIMF | 1 | Traffic Indication Message Flag 0 : AAI-TRF-IND message is not sent for the AMS 1 : AAI-TRF-IND message is sent to the AMS during every Listening Window | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| LWEF | 1 | Listening window Extension Flag 0: the Listening window is of fixed duration 1: the Listening window is extensible | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| NSCF | 2 | Next sleep cycle indicator. 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00~0b11 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). Value: 0~15 Initial Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). Value: 0~1023 Final Sleep Cycle = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| Listening Window | 6 | Assigned duration of AMS's default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. Value: 0~63 Listening Window = Value + 1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| if(TIMF == 1) { | — | — |  |
| SLPID | 10 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode 0~1023 | This parameter shall be included only when ABS transmit this control message and (Response Code == 0b00 ¦¦ Response Code == 0b01). |
| } |  |  |  |
| if(NSCF == 0b10) { |  |  |  |
| New Initial Sleep Cycle | 5 | This is an identifier assigned by the ABS when TIMF is set to 1. This ID shall be unique within an ABS. The other AMS shall not be assigned the same ID while the AMS is still in sleep mode Value: 0~31 New_Initial_Sleep_Cyle = Value+1 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| if(LWEF == 1) { | — | — |  |
| T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | In case of ABS’s transmission: This parameter shall be omitted when Response_Code == 10. |
| } | — | — |  |
| } | — | //ENDIF (Operation =0b10) |  |
| } | — | //ENDIF (Operation == 0b10 ¦¦ Operation==0b11) |  |
| REQ_duration | 8 | Waiting value for the AAI-SLP-REQ message retransmission, which is the Least Significant 8 bits of Frame Number. If REQ_duration is missing when Response_Code == 0b10 in AAI-DSx-RSP message, the AMS shall consider REQ_duration = 0 0~255 | This parameter can be included only when BS transmits AAI-DSx-RSP message in response to AAI-DSx-REQ message sent by AMS. |
| } | — | // ENDIF (Sleep Cycle Setting is included) |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D9, page 1016, line 51, modify as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AAI-DSC-REQ ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

absInitDscInfo                 AbsInitDscInfo OPTIONAL,

directionIndicator             DirIndicator,

serviceClassName               ServiceClassName OPTIONAL,

globalServiceClass             GlobalServiceClassName OPTIONAL,

qosParameterSet                QosParameterSetType OPTIONAL,

qoSParameters                  QosParameter OPTIONAL,

sduInterArrival                Interval OPTIONAL, -- 0.5ms

timeBase                       Interval OPTIONAL, -- ms

classifierDSCAction            ClassifierDSCAction OPTIONAL,

classificationRules            ClassificationRule OPTIONAL,

rohcAttributes                 RohcAttributes OPTIONAL,

packetErrorRate                PacketErrorRate OPTIONAL,

emergencyIndication            BOOLEAN OPTIONAL,

embsService                    EMBSService OPTIONAL,

fullEMBSIDArray                SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

newEmbsZoneID                EMBSZoneID,

carrierIndex                 PhyCarrierIndex,

unicastAvailIntervalBitmap     CHOICE {

nmsi2                        BIT STRING (SIZE(4)),

nmsi4                        BIT STRING (SIZE(4)),

nmsi8                        BIT STRING (SIZE(8)),

nmsi16                       BIT STRING (SIZE(16))} OPTIONAL,

serviceFlowUpdateType        CHOICE {

bitmapAndNew               SEQUENCE {

serviceFlowUpdateBitmap  BIT STRING (SIZE(16)),

embsidFIDMappingArray    SEQUENCE (SIZE(1..16)) OF SEQUENCE {

newEMBSID              EMBSID,

newFID                 FID},

currentAndNew              SEQUENCE (SIZE(1..16)) OF SEQUENCE {

currentEMBSID            EMBSID,

currentFID               FID,

newEMBSID                EMBSID,

newFID                   FID}

}

}

},

groupParameterCreateChange     SEQUENCE {

-- ???? this part of the table is obscure

commonQosParameters           QosParameter OPTIONAL,

qtySfid                           INTEGER (0..255) OPTIONAL,

fidArray                          SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                              INTEGER (0..15) OPTIONAL,

nonCommonQosParameters       QosParameter OPTIONAL}

} OPTIONAL,

coupledGroupCreateChange        SEQUENCE {

-- ??? this part of the table is obscure

commonQoSParameters           QosParameter OPTIONAL,

qtyCoupledSfid                   INTEGER (0..255) OPTIONAL,

coupledFidArray                  SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                              INTEGER (0..15) OPTIONAL},

couplednonCommonFidArray      SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid                              INTEGER (0..15) OPTIONAL,

nonCommonQosParameters       QosParameter OPTIONAL}

} OPTIONAL,

multicastGroupAddition                    SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupAddedId                 INTEGER (0..4095)} OPTIONAL, -- multicast group id to be added

multicastGroupDeletion                    SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupDeletedId                 INTEGER (0..4095)} OPTIONAL, -- multicast group id to be deleted

sleepCycleSetting             SleepCycleSetting OPTIONAL,

initialBackoffWindowSize       INTEGER (0..15) OPTIONAL,

maxBackoffWindowSize           INTEGER (0..15) OPTIONAL,

backoffScalingFactor           INTEGER (0..15) OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------