| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-SCD, AAI-MC-ADV ASN.1 code |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-SCD, AAI-MC-ADV ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-SCD, AAI-MC-ADV ASN.1 code

Joey Chou

Intel

Introduction

The “delta Information” attribute in the AAI-MC-ADV message is to indicate the SFH and AAI-SCD attributes that have been changed between current carrier / preceding carrier and the reference carrier. Each SFH or AAI-SCD attribure should only be present when such attribute is changed. Therefore, each SFH or AAI-SCD attribute should be optional. But, the ASN.1 code in AAI-MC-ADV message in delta information does not provide OPTIONAL tag on per SFH or AAI-SCD attribute basis.

This contribution provides text to fix the delta information issue and other issues.

Proposed Text

Remedy 1:

In the P802.16m/D9, page 975, line 6, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- System Configuration Descriptor Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

Alpha ::= INTEGER {one       (0),

oneOver2  (1),

oneOver4  (2),

oneOver8  (3),

oneOver16 (4),

zero      (5)} (0..7)

-- The number of bits in the bit map = the maximum number of sub-bands for a given bandwidth (i.e., 20MHz, 10MHz, 5MHz) - 1.

-- 20 MHz: b0-b19

-- 10 MHz: b0-b8

-- 5 MHz: b0-b2

-- bi = 1 if resource is changed between sub-band i and sub-band i+1.

-- bi = 0 if resource is not changed between sub-band i and sub-band i+1.

-- See section 16.9.3.1.

ZoneAllocationBitmap ::=  CHOICE {

ch20Mhz                        BIT STRING (SIZE(20)),

ch10Mhz                        BIT STRING (SIZE(9)),

ch5Mhz                         BIT STRING (SIZE(3))}

-- Zone Flag bit. Indicates the use of the last zone.

-- 0b0: Unicast

-- 0b1: E-MBS

-- See section 16.9.3.1.

ZoneFlag ::= ENUMERATED {unicast,

embs}

-- The length of an MSI in units of the number of superframes

-- 0b00: 2 superframes, 40 ms (NMSI = 2)

-- 0b01: 4 superframes, 80 ms (NMSI = 4)

-- 0b10: 8 superframes, 160 ms (NMSI = 8)

-- 0b11: 16 superframes, 320 ms (NMSI = 16)

MsiLength ::= INTEGER (2|4|8|16)

-- The location of the AAI frame where the E-MBS data burst ends

-- MSI length == 0b00: 3 bits

-- MSI length == 0b01: 4 bits

-- MSI length == 0b10: 5 bits

-- MSI length == 0b11: 6 bits

EmbsFrameOffset ::= CHOICE {

msiLength2FrameOffset          INTEGER(0..7),

msiLength4FrameOffset          INTEGER(0..15),

msiLength8FrameOffset          INTEGER(0..31),

msiLength16FrameOffset         INTEGER(0..63)

}

DataSinrMax ::= INTEGER (10|12|14|16|18|20|22|24|26|28|30|32|34|

36|38|40)

UlpcDataChannelSet ::= SEQUENCE {

-- gammaIotFp ( IoT) is the fairness and IoT control factor,

-- broadcast by the ABS. It has 4 bits to represent the value among

-- {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2,

-- 1.3, 1.4, 1.5}. It is different for each frequency partition

-- (FP0, FP1, FP2, FP3)

gammaIotArray SEQUENCE (SIZE(4)) OF SEQUENCE {

gammaIot INTEGER (0..15) OPTIONAL

},

-- alpha is the factor according to the number of receive antennas

-- at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0,

-- reserved, reserved}

alpha Alpha

,

beta INTEGER (0..1),

-- dataSinrMin is the SINR requirement for the minimum data rate

-- expected by ABS. SINRmin_Data has 4 bits to represent the value

-- in dB among {-INF, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5,

-- 2, 2.5, 3, 3.5, 4}

dataSinrMin INTEGER  (0..15),

-- dataSinrMax is the maximum SINR threshold defined by ABS.

-- SINRmmax_Data has 4 bits to represent the value in dB among

-- {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40}

dataSinrMax DataSinrMax

}

UlpcControlChannelSet ::= SEQUENCE {

-- It is the HARQ feedback channel target SINR value broadcasted

-- by the ABS. It has 4 bits to represent the value among {-3.5, -3,

-- -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB

targetHarqSinr INTEGER (0..15),

-- It is the synchronized channel target SINR value broadcasted

-- by the ABS. It has 4 bits to represent the value among {-9, -8.5,

-- -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2,

-- -1.5} dB

targetSyncRangingSinr INTEGER (0..15),

-- It is the P-FBCH target SINR value broadcasted by the ABS.

-- It has 4 bits to represent the value among {-4.5, -4, -3.5, -3,

-- -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB

targetPfbchSinr INTEGER (0..15),

-- It is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5,

-- 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB.

targetSfbchBaseSinr INTEGER (0..15),

-- It is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23,

-- 0.24, 0.25, 0.26}

targetSfbchDeltaSinr INTEGER (0..7),

-- It is the bandwidth request channel target SINR value

-- broadcasted by the ABS. It has 4 bits to represent the value

-- among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5,

-- 1, 1.5, 2, 2.5, 3} dB

targetBwRequestSinr INTEGER (0..15),

-- It is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}.

gammaIotSounding INTEGER (0..15),

-- It is the minimum SINR requirement for sounding expected by ABS.

-- It has 4 bits to represent the value in dB among { -4, -3.5, -3,

-- -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5}

soundingSinrMin INTEGER (0..15),

-- It is the maximum SINR requirement for sounding expected by ABS.

-- It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9,

-- 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20}

soundingSinrMax INTEGER (5..20)

}

EMBSConfigParameters ::= SEQUENCE {

zoneAllocationBitmap           ZoneAllocationBitmap,

zoneFlag                       ZoneFlag,

msiLength                      MsiLength,

embsFrameOffset                EmbsFrameOffset

}

PeriodOfPeriodicRngTimer ::= INTEGER (2|4|7|10|15|20|25|35) -- in sec

TReTxInterval ::= CHOICE {

dl-N-Max-ReTxEq4               INTEGER (1..8),

dl-N-Max-ReTxEq8               INTEGER (1..4)}

RingingSyncInfo ::=                SEQUENCE {

-- the periodicity of ranging channel for synchronized AMSs

-- allocation (Table 893)

periodicityOfRngChSync         INTEGER (0..3),

-- the parameter Ks controlling the start root index of ranging

-- preamble codes for synchronized AMSs

cntlStartCodeOfRngChSync       INTEGER (0..15),

-- the number of codes for periodic ranging (Table 921)

rangingPreambleCodeSync        INTEGER (0..3)}

-- Sounding sequence

-- D is decimation value for frequency decimation multiplexing

-- P is number of codes for code division multiplexing

-- Present when Uplink AAI subframes for sounding in S-SFH SP1 is

-- not set to 0b000

MultiplexingType ::= CHOICE {

decimationValueD               INTEGER (4|6|8|9|12|16|18|36),

maxCyclicShiftIndexP           INTEGER (4|6|8|9|12|16|18|36)}

-- The number of HARQ feedback channel per HARQ feedback region.

-- Describes LHFB in 16.3.8.3.3.2.

HarqfdbkChannels ::= CHOICE {

fiveMhzChannel                 INTEGER (6|12|18|24),

tenMhzChannel                  INTEGER (6|12|24|30),

twentyMhzChannel               INTEGER (12|24|48|60)}

-- quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB.

IotValue ::= INTEGER (0..127)

NSPID ::= BIT STRING (SIZE(24))

VerboseName ::= IA5String (SIZE(1..128))

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- AAI-SCD message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SCD ::= SEQUENCE {

configChangeCount INTEGER (0..15),

bsRestartCount INTEGER (0..15),

-- SA Preamble partition per ABS type

-- 1: macro hot-zone,

-- 2: Relay,

-- 3: OSG femto,

-- 4: CSG-open femto

-- 5: CSG-closed femto ABSs

-- Indicates the SA-Preamble partition information.

-- Each 4 bits represent a partition range for each cell type, as defined in 16.3.5.1.2 and Table 822

saPreamblePartitions SEQUENCE (SIZE(5)) OF PreamblePart,

triggers TriggerConditions,

olMimoParameters SEQUENCE {

olRegionType0On BOOLEAN,

olRegionType1NLRUSize INTEGER (0..15),

olRegionType1SLRUSize INTEGER (0..15),

olRegionType2SLRUSize INTEGER (0..15)

} OPTIONAL,

ringingSyncInfo                RingingSyncInfo OPTIONAL,

periodOfPeriodicRngTimer PeriodOfPeriodicRngTimer

,

ulpcDataChannelIe UlpcDataChannelSet,

ulpcControlChannelIe UlpcControlChannelSet,

tReTxInterval TReTxInterval,

-- BR Channel Configuration MIN Access Class for frame i, i+1, i+2,

-- and i+3 frame

brChCfgMINAccessClassForFrame SEQUENCE (SIZE(4)) OF SEQUENCE {

accessClass INTEGER (0..3) OPTIONAL

},

-- Sounding sequence

-- D is decimation value for frequency decimation multiplexing

-- P is number of codes for code division multiplexing

-- Present when Uplink AAI subframes for sounding in S-SFH SP1 is

-- not set to 0b000

multiplexingType MultiplexingType {

,

shiftValueUForSoundingSymbol INTEGER (0..255),

relayZoneAmsAlocIndc INTEGER (0..1) OPTIONAL

,

embsConfigParameters EMBSConfigParameters OPTIONAL,

ulFeedbackInfoArray SEQUENCE (SIZE(1..8)) OF SEQUENCE {

primaryCarrierIndex PhyCarrierIndex OPTIONAL,

-- The start DLRUs index for feedback channel

startDLRUIndex INTEGER (0..127),

-- The number of DLRUs for feedback channel per UL AAI sub-frame (Refer to 16.3.8.3.3.2)

numberOfDLRUs INTEGER (0..15),

-- The number of HARQ feedback channel per HARQ feedback region.

-- Describes LHFB in 16.3.8.3.3.2. Channel numbers represented by the two bits (0, 1, 2, 3) are as follows.

-- For 5 MHz band, 6, 12, 18, 24

-- For 10 MHz band, 6, 12, 24, 30

-- For 20 MHz band, 12, 24, 48, 60

numberOfHARQChannels HarqfdbkChannels

} OPTIONAL,

-- See Table 809 to Table 811.

-- Resource_Metric_FP2

-- Resource Metric of the first power de-boosted frequency partition which is defined in Table 797. This parameter does not affect "Configuration Change Count"

resourceMetricFP2 INTEGER(0..15) OPTIONAL,

-- See Table 806 to Table 808

-- Resource_Metric_FP3

-- Resource Metric of the second power de-boosted frequency partition which is defined in Table 797. This parameter does not affect "Configuration Change Count"

resourceMetricFP3 INTEGER(0..15) OPTIONAL,

-- Indicates whether ABS achives synchronization from backhaul network (0b01) or not (0b00)

networkSynchronization BOOLEAN OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D9, page 1051, line 20, change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

OptUlpcDataChSet ::= SEQUENCE {

gammaIotArray                  SEQUENCE (SIZE(4)) OF SEQUENCE {

gammaIot                     INTEGER (0..15) OPTIONAL},

alpha                          Alpha OPTIONAL,

beta                           INTEGER (0..1) OPTIONAL,

dataSinrMin                    INTEGER  (0..15) OPTIONAL,

dataSinrMax                    DataSinrMax OPTIONAL

}

OptUlpcControlChSet ::= SEQUENCE {

targetHarqSinr                 INTEGER (0..15) OPTIONAL,

targetSyncRangingSinr          INTEGER (0..15) OPTIONAL,

targetPfbchSinr                INTEGER (0..15) OPTIONAL,

targetSfbchBaseSinr            INTEGER (0..15) OPTIONAL,

targetSfbchDeltaSinr           INTEGER (0..7) OPTIONAL,

targetBwRequestSinr            INTEGER (0..15) OPTIONAL,

gammaIotSounding               INTEGER (0..15) OPTIONAL,

soundingSinrMin                INTEGER (0..15) OPTIONAL,

soundingSinrMax                INTEGER (5..20) OPTIONAL

}

SfhScdDeltaInfo ::= SEQUENCE {

sfhSubpacket1 OptSFHSubpacket1 OPTIONAL,

sfhSubpacket2 OptSFHSubpacket2 OPTIONAL,

sfhSubpacket3 OptSFHSubpacket3 OPTIONAL,

bsRestartCount INTEGER (0..15) OPTIONAL,

saPreamblePartitions SEQUENCE (SIZE(5)) OF PreamblePart OPTIONAL,

triggers Triggers OPTIONAL,

periodicityOfRngChSync INTEGER (0..3) OPTIONAL,

periodOfPeriodicRngTimer PeriodOfPeriodicRngTimer

,

olMimoParameters SEQUENCE {

olRegionType0On BOOLEAN OPTIONAL,

olRegionType1NLRUSize INTEGER (0..15) OPTIONAL,

olRegionType1SLRUSize INTEGER (0..15) OPTIONAL,

olRegionType2SLRUSize INTEGER (0..15) OPTIONAL

} OPTIONAL,

cntlStartCodeOfRngChSync INTEGER (0..15) OPTIONAL,

rangingPreambleCodeSync INTEGER (0..3) OPTIONAL,

ulpcDataChannelIe OptUlpcDataChSet OPTIONAL,

ulpcControlChannelIe OptUlpcControlChSet OPTIONAL,

tReTxInterval TReTxInterval OPTIONAL,

brChCfgMINAccessClassForFrame SEQUENCE (SIZE(4)) OF SEQUENCE {

accessClass INTEGER (0..3) OPTIONAL

} OPTIONAL,

multiplexingType MultiplexingType

OPTIONAL,

shiftValueUForSoundingSymbol INTEGER (0..255) OPTIONAL,

relayZoneAmsAlocIndc INTEGER (0..1) OPTIONAL

,

resourceMetricFP2 INTEGER(0..15) OPTIONAL,

resourceMetricFP3 INTEGER(0..15) OPTIONAL,

embsConfigParameters EMBSConfigParameters OPTIONAL,

networkSynchronization 	BOOLEAN OPTIONAL

}

ServingABSCarrierInfo ::= SEQUENCE {

pagingGroupID 	PGID,

carrierIndex 	PhyCarrierIndex,

saPreambleIndex PreambleIndex,

pagingCarrierIndicator ENUMERATED {

noPagingCarrier,

pagingCarrier

},

caSpecificTrigger 	CASpecificTriggers 			OPTIONAL,

pccSpecificTrigger 	Triggers 			OPTIONAL,

sSFHChangeCount 	INTEGER (0..15) 	OPTIONAL,

scdChangeCount 		INTEGER (0..15) 	OPTIONAL,

sfhScdInfo             CHOICE {

sfhSubpack                SEQUENCE {

sfhSubpacket1 	SFHSubpacket1,

sfhSubpacket2 	SFHSubpacket2,

sfhSubpacket3 	SFHSubpacket3,

aaiSCDInfo 		AAI-SCD

},

deltaRefToCurCarrier       SfhScdDeltaInfo,

deltaRefToPrevCarrier      SfhScdDeltaInfo

}

}

AAI-MC-ADV ::= SEQUENCE {

mcChangeCount 		INTEGER (0..15),

macProtocolVersion 	MacProtocolVersion,

nNbrCrxForMCTrigger		ENUMERATED {twoCarriers, threeCarriers}	OPTIONAL,

crxSpecificPHYCtrlMode	BOOLEAN,

servingABSCarrierInfoArray SEQUENCE (SIZE(1..maxBSCarriers-1)) OF ServingABSCarrierInfo,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D9,Table 749 at  page 272, line 26, change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 749—AAI-MC-ADV Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| PCC-specific Trigger definitions | Variable | Primary Carrier Change-specific triggers with encoding defined in Table 768—Trigger description. | Optional Present when the corresponding carrier is required to apply different trigger condition from definitions of AAI-SCD message. |
| SFH and SCD encoding format | 2 | 0b00: full SFH subpacket and AAI-SCD information 0b01: delta encoding of SFH_Info (SFH subpacket and AAI-SCD) with reference to the information of current carrier which transmits this AAI-MC-ADV message 0b10: delta encoding of SFH_Info (SFH subpacket and AAI-SCD) with reference to the information of preceding carrier 0b11: reserved |  |
| S-SFH change count | 4 | Change count of S-SFH SP IE(s) |  |
| AAI-SCD Change Count | 4 | Change configuration count of AAI-SCD |  |
| If (SFH and SCD encodingformat==0b00) { |  |  |  |
| SFHSubpkt 1 | Variable | SFH subpacket 1 is included, length depends on FFT size. |  |
| SFHSubpkt 2 | Variable | SFH subpacket 2 is included, length depends on FFT size. |  |
| SFHSubpkt 3 | Variable | SFH subpacket 3 is included, length depends on FFT size. |  |
| AAI-SCD_Info | Variable | Information from AAI-SCD is included |  |
| } |  |  |  |
| If (SFH and SCD encodingformat==0b01 or 0b10) { |  |  |  |
| Delta information | Variable | Indicates delta encoding between the reference carrier and the current carrier where this message is transmitted if SFH_encoding_format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format = 0b10 Delta information may contain SFH SP1, SP2, and SP3 attributes as defined in Table 840, Table 841, and table 842, respectively. It may contain the following AAI-SCD attributes: •BS_Restart_Count •SA_PreamblePartitionforBStype •Trigger definitions •NormalizedCINR •OL-Region-Type0-ON •OL-Region-Type2-SLRU-Size •periodicityOfRngChSync •cntlStartCodeOfRngChSync •rangingPreambleCodeSync •UlpcDataChannelSet •gammaIotFp0 •gammaIotFp1 •gammaIotFp2 •gammaIotFp3 •Alpha •Beta •dataSinrMin •dataSinrMax •UlpcControlChannelSet •targetHarqSinr •targetSyncRangingSinr •targetPfbchSinr •targetSfbchBaseSinr •targetSfbchDeltaSinr •targetBwRequestSinr •gammaIotSounding •soundingSinrMin •T_ReTx_Interval •BR_Channel Configuration MIN Access Class of the (i+0)-th frame •BR_Channel Configuration MIN Access Class of the (i+1)-th frame •BR_Channel Configuration MIN Access Class of the (i+2)-th frame •BR_Channel Configuration MIN Access Class of the (i+3)-th frame •SoundingMultiplexingType •decimationValueD •maxCyclicShiftIndexP •shiftValueUForSoundingSymbol •AAI_Relay_zone_AMS_allocation_indicator •E-MBS is configured •resourceMetricFP2 •resourceMetricFP3 •Network synchronization indicator Each delta information attribute is optional. |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------